3GPP TSG-RAN WG2 Meeting NR ad-hoc	Tdoc R2-1700433

Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.5

Source: 	Ericsson

Title:  	Lossless PDCP SN reconfiguration at HO

Document for:	Discussion, Decision

# Introduction

This contribution introduces some solutions to achieve lossless PDCP SN reconfigurations.

# Discussion

Normally there should be no reason for changing the SN length because the initial SN length should in most cases be sufficient. However, there may be a need for changing the SN length at handover, because one of the two nodes involved in the handover may support a different SN range set.

If, for example, a radio bearer is configured a high SN value by a node because it carries a high data rate service, and the UE moves to a node not supporting that format, then it is necessary to shorten the SN length at handover. Also, we will have the opposite case, when a UE is configured with a small SN value and moves to a target node that supports a larger SN format, then the target node may need to extend the SN length due to the higher data rates which may be supported by this node.

Hence, we have the following cases for handling the PDCP SN format at handover:

- Both in the source and in the target node support and configure the same SN value.
This case is handled without any data losses.

- The target node supports a different set of SN values or the target node configures a SN value higher than the configured by the source node: the SN length needs to be extended.
This case is handled without any data losses.

- The target node supports a different set of SN values or the target node configures a SN value lower than the configured by the source node: the SN length needs to be shortened.
This case is handled with data losses

We consider that it is sufficient to change PDCP SN length only during handover when MAC layers are reset and PDCP and RLC re-established. Changing the PDCP SN length at any time would be more complex to specify and may require updates of the PDCP SN field of PDCP PDU(s) that are buffered in the RLC and the MAC layers.

PDCP SN may be reconfigured only during a handover

## SN length decrease

This is the only case which the standard does not handle efficiently. Basically, all data which has not been received by the UE will be lost in the radio interface and would have to be retransmitted by higher layers above PDCP.

Two different approaches may be followed to address this problem:

- To study mechanisms reduce the data losses

- To study mechanisms which completely avoid data losses

### Mechanism to reduce data loss

When a handover happens, the source node forwards PDCP SDU(s) with the initial SN length to the target node.

In general, the conversation of the number is done as follows, using an example where the difference in SN size is 2 bits:

- The 2 most significant bits of the SN is moved to the HFN as two new bits and forms the 2 least significant bits of the new HFN.

- The 2 most significant bits of the SN is dropped to form the new format of the SN.

See the following picture shows an example about how this conversion can be done when the source node had configured a SN length of 14 bits and the target node configures a SN length of 12 bits.

HFN

PDCP SN

COUNT

2 MSB

HFN

PDCP SN

2 MSB

12 bit PDCP SN

20 bit HFN

HFN

PDCP SN

COUNT

2 MSB

HFN

PDCP SN

2 MSB

12 bit PDCP SN

20 bit HFN

If a PDCP SDU with a long SN length is sent from the source node and the target node is does not support it or configures a smaller value, then the target node will only read a subset of the least significant bits of the SN length configured by the source node.

After the reconfiguration, the network may request a status report to the UE. According to current LTE specs, the FMS that the UE transmits after the reconfiguration only contains the number of bits configured by the network for the PDCP SN. Since the new PDCP receiving window is smaller than the previous one, the UE can only send a report for a subset of the PDCP PDUs which were in the PDCP receiving window before the reconfiguration. This is exemplified in the figure below.

The receiver should only consider half of PDCP receiving window to avoid potential mismatches with the transmitter. If the receiver would consider the full window and the transmitter had sent more packets, when the transmitter receives the FMS, the transmitter would not be able to resolve the uncertainty as it could have two packets with the same SN.

The PDCP PDUs that have been received but which are waiting in the PDCP receiver window that is outside the shorter window range, can be forwarded to higher layers, but this means that there will be data losses because the PDCP PDUs that are not received within this area of the window will be lost in the radio interface.

While this method is quite simple, the fact that the receiver side only keeps a subset of the original PDCP receiving window, there is still risks of having PDCP losses. That may be the case when the losses are outside of the window that the receiver takes.

### Mechanism to avoid data losses

Another solution which would avoid data losses is to request the PDCP receiver to forward all already received PDCP SDUs to higher layers (even if there are holes in the sequence number range), then reset the SN and HFN to zero values. The PDCP transmitter will then retransmit all PDCP PDUs for which it has not yet received a positive acknowledgement. As a result, this would mean that some PDCP PDUs would be transmitted out-of-order, but there would be no data losses.

In addition, an indication could be sent from the receiver side to the target node using a FMS which length is either the SN length of the configuration from the source node, or a new FMS format could be introduced which length is 32 bits (HFN + SN).

The source target sends all packets pending for acknowledgment to the target node. The information includes both the HFN and the SN value. If the UE sends enough information to identify these packets, then the network (transmitter) could recover any missing PDCP PDUs.

Then, when the UE is reconfigured (handover), the reconfiguration could indicate to the UE to use a special report which could contain the SN of the missing PDCP PDUs using the previous format, or using a new format value (e.g. COUNT value). By doing this, the network (transmitter) could identify all missing packets.

The challenge to solve is that the target (new) node cannot retransmit the data using the previous configuration from the source node. Thus, to be able to retransmit any data, the transmitter and receiver would be to re-initialize the PDCP variables.

From a higher layer point of view (higher than PDCP), there would not be any missing PDUs in such case, and the handover would be fully lossless.

This solution has limited complexity and avoids losing data when a HO is performed and the SN length is reduced; thus this solution is preferred.

# Proposals

We kindly request RAN2 to discuss and agree on:

Proposal 1	PDCP SN may be reconfigured only during a handover

Proposal 2	Capture these solutions in the TR, and discuss possible recommendations.