3GPP TSG-RAN WG2 Meeting NR ad-hoc	Tdoc R2-1700434

# Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.1 Overall user plane architecture

Source:	Ericsson

Title:	Additional aspects of the SCG split bearer

Document for:	Discussion, Decision

# Introduction

The need of supporting SCG split bearer is motivated in [1]. This contribution looks at additional aspects related to the SCG split bearer.

# Discussion

- SCG split use case

Current technical report states that the use case of SCG is met when a set of conditions are met. Among those conditions, we find:

- Ample backhaul provisioning

- NR bit rate is comparable to LTE bit rate

These two conditions may be needed or true for certain scenarios. However, in real networks, there may be other scenarios in which the SCG split bearer bring benefits, and these two conditions do not necessarily have to be fulfilled.

One such use case may be for reliability while maximizing the bit rate. This may happen when a certain UE is within an area covered by the SCG cell at high frequencies.  High frequencies might experience deep sudden fades. Thus, SCG cell may transmit user plane data while the MCG cell may transmit the signalling to ensure a robust connection. When very high bit rates can be achieved by the NR leg (SCG cell) compared to what the LTE leg can provide for a UE (i.e. NR bit rate >> LTE bit rate), it is likely that the user plane data is only transmitted in the NR leg since the aggregated bit rate may still be close to what the NR bit rate is. To have efficient routing of UP data from S-GW, SCG bearer should be considered. However, to achieve the robustness for the concerned bearer, also MCG leg could be used.

With regards the condition “ample backhaul provisioning”, in many cases, this condition might not be needed. SCG cells might not be close to the centre of the MCG cells. Thus, the bit rates that the MCG cell may offer to a UE could be moderate. This, therefore, means that the backhaul needed to route the data from the SCG to the MCG cell should cope with moderate bit rates, and an “ample” backhaul is not needed.

On the other hand, for the MCG split bearer, an ample backhaul provisioning is very likely to be needed. The reason is that, when a UE is in SCG cell area, it is likely that the SCG cell can provide very high bit rates, especially if the SCG cell area is small. Thus, the backhaul capacity needed to route this data from the MCG cell to the SCG cell will be comparable to the maximum bit rates that the NR cell may offer in its area.

Update the use cases conditions for SCG split bearer.

- SCG split bearer benefits

The benefits of the MCG/SCG split bearer have been discussed already: robustness, dynamic and quick control of path on which data is transmitted maximizing throughput, and minimum interruption of the user plane time, among others.

Although MCG split bearer support itself gives many benefits, SCG split bearer may be a better option in some important cases.

MCG split bearer may have a larger impact in the already deployed networks. eNBs may need to be upgraded to cope with the additional processing requirement to cope with the NR bit rates at the PDCP layer, and the backhaul will also need to be upgraded to be able to carry the NR bit rates.

The SCG split bearer, contrary, imposes looser backhaul requirements compared to the MCG split bearer as LTE bit rates are expected to be lower than NR bit rates and, as discussed in the previous section, in many situations, the LTE bit rates in the coverage areas of SCG cells may be considerably lower than the maximum LTE bit rates.

In conclusion, the SCG split bearer may be a more viable solution for early deployments as well as for deployments in which an operator wants to minimize the impact in the already installed infrastructure.

- SCG split bearer: impacts and complexity

The expected impacts of introducing SCG split bearer in the specifications are limited. In the network side, signalling to coordinate the MCG/SCG roles and configuration will be needed. In the UE side, although the architecture can remain transparent from a UE point of view and, minor updates might be needed e.g. IEs and/or procedural text to specify which ciphering keys are used by the UE. Yet, these updates are rather straight forward.

Having said so, it is also expected that the complexity of supporting SCG split bearer is limited from both UE and NW side.

SCG split bearer should be supported

# Conclusion

SCG split bearer is a solution which can minimize the impact in the network when NR is used in combination with LTE. MCG split may require an upgrade in several parts of a network such as the backhaul and the MCG processing capabilities. While, MCG/SCG bearer could be an alternative solution to cope with the MCG issues, the MCG/SCG bearer has other drawbacks, as discussed in this document and in previous documents.

Proposal 1	Update the use cases for SCG split bearer.

Proposal 2	SCG split bearer should be supported

# References

R2-1700402, Support of SCG split bearer, NTT DoCoMo et al, RAN WG2 Meeting NR ad-hoc, Spokane, USA, 17th – 19th January 2017

# Annex Text Proposal:

#### 5.2.1.2	Bearer types for Dual Connectivity between LTE and NR

--- unmodified text is skipped ---

Table 5.2.1.2-1:	Comparison results on the bearer types for LTE-NR Dual Connectivity

| Bearer types | SCG bearer (1A) | Split bearer via MCG (3C) | Split bearer via SCG |
| --- | --- | --- | --- |
| Utilisation of radio resources across MeNB and SeNB | Not possible for the same bearer, requires at least two DRBs for having user plane traffics in MeNB and SeNB | Possible for the same bearer  | Possible for the same bearer  |
| Dynamic offload | Need to involve MME, very static | Controlled by MeNB, can be dynamic as long SCG is setup  | Controlled by SeNB, can be dynamic as long MCG is setup  |
| Additional MeNB processing capacity requirement for SCG path | No additional processing capacity requirement  | Additional processing capacity requirement in MeNB to process NR leg  | No additional processing capacity requirement  |
| Buffering requirements | Full termination of CN bearer at SeNB offloads PDCP buffering from MeNB  | Bearer splitting implies increased reordering-buffering requirement, at UE and MeNB  | Bearer splitting implies increased reordering-buffering requirement, at UE and SeNB  |
| Per-user throughput enhancements | The gain is low if only one bearer exists; The gain depends on the data volume of MCG bearer and SCG bearer if two bearers exist. | The gain is higher than 1A if only one bearer exists; The exact gain depends on the available throughput in MCG and SCG. | The gain is higher than 1A if only one bearer exists; The exact gain depends on the available throughput in MCG and SCG. |
| Interruption upon UE mobility | Interruption visible due to MeNB unable to support SeNB bearer  | Interruption limited thanks to the ability of the MeNB to transmit data for the split bearers  | For UE moving from SeNB coverage to the area without the coverage of any SeNB scenario, interruption limited thanks to the ability of the MeNB to transmit data for the split bearers (e.g., by NW implementation), but for UP termination point change from SeNB to MeNB scenario, interruption visible  |
| Signalling load to CN due to mobility in/out of SeNB coverage | Not hidden to CN  | Hidden to CN  | Not hidden to CN  |
| MeNB – SeNB backhaul requirements | No additional throughput requirement on backhaul of MeNB  | The Xn interface has to offer the latency of 5-30 ms and sufficient capacity.  Increased throughput requirement on backhaul compared to 1A: backhaul needs to cope with NR bitrates  | The Xn interface has to offer the latency of 5-30 ms and sufficient capacity.  Increased throughput requirement on backhaul compared to 1A: backhaul needs to cope with LTE bitrates  |
| U-plane latency | No additional U-plane latency  | Additional U-plane latency for SCG path  | Additional U-plane latency for MCG path  |
| Use case | When ANY of the following holds: - Limited backhaul provisioning - NR bit rate is much higher than LTE bit rate - UE has limited buffering capabilities - MeNB and SeNB have limited buffering capabilities | When ALL of the following hold: - Ample backhaul provisioning - NR bit rate is comparable to LTE bit rate - MeNB has sufficient processing power - MeNB and UE have sufficient buffering capabilities | When ANY of the following hold: - Ample backhaul provisioning - NR bit rate is comparable to or larger than LTE bit rate - MeNB does not have sufficient processing power - SeNB and UE have sufficient buffering capabilities |