3GPP TSG-RAN WG2 Meeting #96-AH	Tdoc R2-1700435

Spokane, Washington, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.3 - NR – User Plane – QoS

Source:	Ericsson

Title:	QoS framework for NR

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss the remaining open issues related to the NR QoS framework.

# Discussion

## Updating QoS-Flow to DRB filters

At RAN2-96 it was discussed how the network can change a mapping of UL flows to DRBs and RAN2 agreed that “The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly”.

The word “continuously” was put in quotation marks since companies wanted to study whether really each and every DL packet needs to be analysed.

We believe that this is the simplest way to allow the eNB to update the mapping by redirecting the packets of a DL flow onto a different DRB. For example, if the UE observes initially a downlink packet with Flow ID X on DRB 1, it creates an AS filter that maps uplink packets with Flow ID X to DRB 1, too. But if the UE later observes a downlink packet with Flow ID X on DRB 2, it should change the filter for Flow X so that also the UL packets are mapped to DRB 2.

We therefore suggest confirming and clarifying the previous agreement as follows:

The UE monitors the Flow ID (if present) in all downlink PDCP packets and updates the corresponding reflective “uplink QoS Flow ID to DRB mapping” accordingly.

For completeness, we also suggest to capture that the NW may change a configured uplink flow to DRB mapping by means of an RRCConnectionReconfiguration:

The RAN may configure, modify and release a configured “Uplink QoS Flow to DRB” mapping (a pair of “QoS Flow ID” and “DRB ID”) by an RRCConnectionReconfiguration.

## Precedence order of reflective and configured mapping

RAN2-96 did not yet agree on “The precedence of the RRC configured mapping and reflective QoS”. There are basically three options:

1) An RRC configured mapping overrides any reflective mapping for that flow.

2) A newly derived reflective mapping overrides a mapping configured previously by RRC.

3) The UE applies always the most recent mapping, i.e., either provided by RRC or derived by reflective QoS

We think that the second option would introduce an undesirable dependency between the RRC configuration and the user plane data plane. For example, the RRC configuration (AS-Config) would not represent the mapping that the UE actually applies. This would be undesirable in case of mobility since the UE would not behave as expected by the target node. It would also remove the possibility to override a previous reflective QoS mapping by a dedicated configuration.

The third option suffers from possible race conditions since it may not be fully predictable whether the UE received a DL data packet or the RRCConnectionReconfiguration first. Also, just as the second option, ambiguities exist upon mobility.

Generally, we think that RRC signalling should always have precedence over L2 and L1 control signalling. It would ensure a clean split and avoid any ambiguity. Also during mobility this principle ensures that the target eNB is aware of all configured UL QoS mappings applied by the UE. Besides that, it would also allow the eNB to map a DL QoS flow onto a different DRB than the UL QoS flow with the same ID.

If the eNB configures the UE with an “uplink QoS Flow to DRB filter”, it overrides any reflective mapping for this QoS flow.

In the context of inter-cell mobility it should be discussed whether the UE maintains the reflective UL QoS filters. As mentioned above, the target eNB does not know the UE’s reflective QoS filters from the AS-Config. One could consider that the source eNB provides the reflective UL QoS mappings to the target eNB (e.g. in AS-Context). But we consider this being unnecessarily complex and it would also introduce risk of state mismatch. It appears simpler that the UE maintains a reflective UL QoS mapping as long as the DRB with which it is associated exists, i.e., also during normal RRC mobility. The UE releases the reflective UL QoS mapping when the eNB releases the DRB with which the mapping is associated.

The UE maintains a reflective UL QoS mapping as long as the DRB with which it is associated exists, i.e., also during normal RRC mobility. The UE releases the reflective UL QoS mapping when the eNB releases the DRB with which the mapping is associated.

## Packets not matching any Flow ID to DRB filter

In the previous meetings RAN2 discussed how to handle initial uplink packets. At RAN2-96 the following working assumption was captured: “If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.”

With this rule, the UE will follow the configured mapping whenever there is one. If there is none, it applies the reflective mapping as soon as those have been determined based on an observed DL packet. Before that happens, the UE maps initial uplink packets to the default bearer.

This may happen if the CN configured via N1 a NAS mapping (IP tuple to QoS Flow) whereas the RAN decided not (yet) to configure an QoS to DRB mapping but rather to rely on reflective QoS. It is our understanding that the RAN has this option in accordance with previous agreements (RAN determines the mapping…).

Confirm the working assumption: If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.

## Omitting Flow ID in PDCP header

To enable reflective QoS, the RAN marks downlink packets over Uu with a QoS flow ID. The UE marks uplink packets over Uu with the QoS flow ID for the purposes of marking forwarded packets to the CN.

RAN2-96 discussed whether these fields should always be present or whether the eNB should configure their presence explicitly. In our view, the reflective QoS concept intends to reduce the signalling overhead but the need to include the flow ID in each PDCP header increases the user plane protocol overhead. The relative overhead is marginal for large IP packets but could be considered significant for services such as VoIP.

One could consider omitting the flow ID in all but the first DL packet of a certain flow. Or one could compress the flow ID on the radio interface. But since such schemes increases the complexity (book-keeping) in UE and eNB they don’t come for free either.

There are however still cases when the Flow ID does not need to be conveyed: Whenever the eNB configures a dedicated DRB for a Flow (1:1 mapping) and if it configures the Flow to be mapped to the DRB (explicitly configured UL “AS filter”), neither the UE nor the eNB need to include the flow ID. Since this may be a typical case for IMS Voice as well as for latency critical services where the relative overhead due to the Flow-ID is significant, the eNB should have means to configure for each DRB whether the Flow-ID is conveyed in the PDCP header.

As discussed during RAN2-96, the NAS level will not provide a QoS Flow ID in deployment scenario where NR connects to EPC (scenario 3). Also in such cases, it should be possible not to include the Flow ID over the Uu interface.

The eNB configures by RRC for each DRB whether or not the UE shall include the Flow-ID in uplink headers.

In the last meeting it was discussed which Access Stratum protocol header should carry the Flow ID and/or other possible QoS related fields. Some companies suggested introducing a new protocol residing above PDCP. One reason for doing this could be that the function mapping an incoming IP packet to a DRB (PDCP entity) should logically not be part of a PDCP entity. However, the PDCP protocol may of course also describe functionality done outside (before) the PDCP entity. Therefore, we consider it a natural choice that QoS fields and the handling thereof is defined as part of PDCP:

The QoS fields (at least QoS Flow ID) and handling thereof are modelled as part of the PDCP header.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	The UE monitors the Flow ID (if present) in all downlink PDCP packets and updates the corresponding reflective “uplink QoS Flow ID to DRB mapping” accordingly.

Proposal 2	The RAN may configure, modify and release a configured “Uplink QoS Flow to DRB” mapping (a pair of “QoS Flow ID” and “DRB ID”) by an RRCConnectionReconfiguration.

Proposal 3	If the eNB configures the UE with an “uplink QoS Flow to DRB filter”, it overrides any reflective mapping for this QoS flow.

Proposal 4	The UE maintains a reflective UL QoS mapping as long as the DRB with which it is associated exists, i.e., also during normal RRC mobility. The UE releases the reflective UL QoS mapping when the eNB releases the DRB with which the mapping is associated.

Proposal 5	Confirm the working assumption: If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.

Proposal 6	The eNB configures by RRC for each DRB whether or not the UE shall include the Flow-ID in uplink headers.

Proposal 7	The QoS fields (at least Flow ID) and handling thereof are modelled as part of the PDCP header.

# Annex: QoS-related agreements in previous meetings

RAN2-95 discussed the basic principles of the NR QoS framework and reached the following agreements:

Agreements

1	For DL for a non-GBR flow, the eNB sees an indication over NG-u and based on the indication the eNB maps the packet to a DRB of an appropriate QoS. RAN2 understanding of SA2 agreements is that eNB has a QoS profile associated with the indication.

FFS whether there is a requirement for every different QoS indication to be mapped to a different radio bearer.

2	Functionality is required to differentiate flows from different PDN-connections over the radio interface (e.g. by using separate DRBs or by an explicit indication in a header)

3	For DL, the eNB establishes DRBs for the UE taking the QoS profiles in to account.

FFS how the DRB is established in the first packet is an UL packet.

At RAN2-95bis some further agreements were achieved and the first of the FFSs above was resolved:

Agreements

1:	RAN determines the mapping relationship between QoS flow (as determine by the UE in UL or marked by the CN in DL) and DRB for UL and DL.

1a	RAN can map multiple QoS flows to a DRB.

2	Specification will not forbid a GBR flow and non-GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

3	Specification will not forbid more than one GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

FFS: Whether traffic from different PDU sessions can be mapped to one DRB or not.

Agreements

1	Default DRB is established by eNB at PDU session establishment (or an existing DRB may be used if mapping of more than one session to a DRB is allowed)

2. 	If the first packet of the flow is UL packet, if no mapping rule is configured in the UE, the packet is sent through default DRB to the network.

FFS How and when the network can remap the flow to more appropriate DRB.

FFS the first packet is handled in the case that pre-authorised QoS is configured

FFS whether the pre-authorised QoS applies to RAN or only to the UE.

FFS whether there is a single level of mapping from UL TFT (5 tuple) to DRB, or whether there is a 2 level mapping from UL TFT to QoS flow and then from QoS flow to DRB.

RAN2#96:

Agreement

1:	Traffic from different PDU sessions are mapped to different DRBs

2:	In DL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping (confirmation of SA2 agreement status).

3:	In UL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping.

4	DL packets over Uu are marked in band with QOS-flow-id for the purposes of reflective QoS

5	UL packets over Uu are marked in band with QOS-flow-id for the purposes of marking forwarded packets to the CN.

FFS for bullets 4 and 5 whether it can be semi-statically configured to not include the QOS flow ID in some cases.

FFS for bullets 4 and 5 whether it might be possible to use a shorter id over the radio compared to that received from the CN. This is a stage 3 issue.

Agreements

1	For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

2	The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly.

3	RRC can configure an uplink mapping

FFS The precedence of the RRC configured mapping and reflective QoS (e.g. can reflective QoS update an RRC configured mapping)

Working assumption:

If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.

=>	FFS whether the QoS field is added by PDCP or a new protocol layer above PDCP.