3GPP TSG-RAN WG2 Meeting #NR AH2                                                             R2-1706403

Qingdao, China, 27th - 29th June 2017		revision of R2-1704218

Source:	CATT

Title:	Inter-RAT Measurement for NR

Agenda Item:	10.4.1.4.6

Document for:	Discussion and Decision

Introduction

In the NR WID [1], it is specified the dual connectivity is aimed to be supported as followings:

This work item is aimed at supporting the following connectivity options (*): For single connectivity option: - NR connected to 5G-CN (Option 2 in TR 38.801 section 7.1). For Dual Connectivity options: - E-UTRA-NR DC via EPC where the E-UTRA is the master (Option 3/3a/3x in TR 38.801 section 10.1.2); - E-UTRA-NR DC via 5G-CN where the E-UTRA is the master (Option 7/7a/7x in TR 38.801 section 10.1.4); - NR-E-UTRA DC via 5G-CN where the NR is the master (Option 4/4A in TR 38.801 section 10.1.3).

This contribution discusses the inter-RAT measurement issues that impact the support of standalone NR and LTE-NR dual connectivity.

Discussion

Considering the SA scenario, for single connectivity (NR connected to 5G-CN), the handover from NR to LTE can be supported when EPC and 5G-CN has interface, or LTE connected to 5G-CN. So to support this scenario, the inter-RAT measurement event of B1 and B2 is needed.

Proposal 1: Introduce the following NR inter-RAT measurement events:

- Event B1:  Neighbour becomes better than absolute threshold;

- Event B2:  PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2.

For dual connectivity of NR-E-UTRA DC via 5G-CN where the NR is the master, the similar inter-RAT measurement events are needed to be introduced like EN-DC as analysis in [2].

For SeNB change, SeNB release, the measurement configuration can be done by SeNB itself or by MgNB depending on the scenario. If SeNB configures the measurement, it is intra-LTE measurement, the event A1~A6 can be used. If MgNB configure the inter-RAT measurement, the following NR inter-RAT measurement events can be considered for NE-DC:

Event D1: All LTE SCG cells become worse than a threshold1 and a LTE neighbour cell becomes better than a threshold2 (used for LTE SeNB change)

Event D2: All LTE SCG cells become worse than a threshold (used for LTE SeNB release)

So it is proposed:

Proposal 2: Introduce the following NR inter-RAT measurement event for LTE SeNB change initiated by NR MgNB:

- Event D1: All LTE SCG cells become worse than a threshold1 and a LTE neighbour cell becomes better than a threshold2.

Proposal 3: Introduce the following NR inter-RAT measurement event for LTE SeNB release initiated by NR MgNB:

- Event D2: All LTE SCG cells become worse than a threshold.

Considering the periodical inter-RAT measurement reporting in NR, it can be used for the ANR/SON feature, with LTE as the candidate measurement object. The purpose of periodical reporting can be reportStrongestCellsForSON and reportCGI.

Proposal 4: Introduce periodical inter-RAT measurement reporting in NR for ANR/SON feature.

Conclusion

In this contribution, we discuss the inter-RAT measurement and propose:

Proposal 1: Introduce the following NR inter-RAT measurement events:

- Event B1:  Neighbour becomes better than absolute threshold;

- Event B2:  PCell becomes worse than absolute threshold1 AND Neighbour becomes better than another absolute threshold2.

Proposal 2: Introduce the following NR inter-RAT measurement event for LTE SeNB change initiated by NR MgNB:

- Event D1: All LTE SCG cells become worse than a threshold1 and a LTE neighbour cell becomes better than a threshold2.

Proposal 3: Introduce the following NR inter-RAT measurement event for LTE SeNB release initiated by NR MgNB:

- Event D2: All LTE SCG cells become worse than a threshold.

Proposal 4: Introduce periodical inter-RAT measurement reporting in NR for ANR/SON feature.

Reference

[1] RP-170855, New WID on New Radio Access Technology

[2] R2-1706397, inter-RAT measurement, CATT