3GPP TSG-RAN2 #98	R2-1706715

Qingdao, China, 27-29 June, 2017

Agenda item:		10.2.2.3

Source: 	Huawei, HiSilicon

Title: 	Packet Duplication for SRBs

Document for:	Discussion and decision

# Introduction

This contribution describes the details for activating/deactivating packet duplication for SRBs. In comparison to DRBs, the SRBs are generally characterized by different traffic characteristics and reliability requirements. In RAN2 AH, the following was agreed:

Agreements:

1:  Packet duplication is supported for user plane and control plane in NR-PDCP (This agreement does not preclude discussion of other mechanisms to improve mobility robustness)

FFS whether packet duplication should also be supported for LTE-NR dual connectivity

2  The PDCP function in the transmitter supports packet duplication and the PDCP function in the receiver supports duplicate packet removal.

In RAN2 #97bis, the following agreements were reached:

Agreements

1: SCG SRB is of higher scheduling priority than all DRBs.

2: UE processes messages received on SCG SRB one message at a time in the order received at the RRC. (i.e. same rules as in LTE).

3: There is no requirement on the UE to perform any reordering of RRC messages between MCG SRB and SCG SRB.

FFS: What terminology will be used to describe the SCG SRB.

Agreement

1:	The following RRC messages can be sent via the SRB in the SCG.

•	RRCConnectionReconfiguration and RRCConnectionReconfigurationComplete

•	MeasurementReport

In RAN2 #98, the relevant agreements were:

Agreements for EN-DC and NGEN-DC:

1:	SCG SRB is modelled as one of the NR SRBs defined in 38.331.

2:	SCG SRB uses NR-DCCH logical channel type.

3:	A UE can be configured with both split MCG SRB and SCG SRB simultaneously.

4:	SCG SRB and the SCG leg of split SRB1/2 will be independently configured

5:	SCG-SRB establishment and release can be done at SCG addition and SN change

6	SCG-SRB reconfiguration can be done at SCG modification procedure.

7	RRC PDUs on SCG-SRB are ciphered using NR PDCP.

8	RRC PDUs on SCG-SRB are integrity protected using NR PDCP.

9	Security keys for SCG-SRB are derived from S-KgNB.

In the evaluations provided in contribution R2-1700172, it was observed that packet duplication enhances resource utilization efficiency when certain conditions related to channel quality, PDU size and reliability requirement are satisfied. Considering the differences between SRBs and DRBs, the benefits of packet duplication can be realized by ensuring that the criteria and the triggering mechanism used for enabling packet duplication for SRBs is different from that of DRBs.

# Discussion

Packet duplication is supported at the PDCP entity for satisfying the high reliability requirement of both DRBs and SRBs. In the case of SRBs, packet duplication can be used for achieving robustness and RRC diversity.  The RRC messages are transmitted over more than one link to the terminating RRC entity in the network (in UL) or UE (in DL). In comparison to DRBs, SRBs are characterized by less frequent transmissions, small PDU sizes and higher scheduling priority. As such, the packet duplication criteria for SRBs may be different than for the DRBs. In certain scenarios (e.g. cell centre with LOS conditions), it may be possible to satisfy the reliability requirement for SRBs by transmitting only over the single best link as in the case of using link selection.  However, in cell edge scenarios, it may be necessary to perform packet duplication on the SRBs and transmit the SRBs using two links.

The network configures the UE for packet duplication of SRBs using RRC signalling. The decision for configuring packet duplication can be made based on the criteria which include reliability requirements of the SRBs, channel quality measurements, loading conditions and expected SRB PDU sizes.

For UL transmission, packet duplication for SRBs can be configured for both CA and DC architectures. While in the CA case the duplicated SRBs can be transmitted by the UE using two component carriers (i.e. either MN or SN), in DC the transmissions are performed using both the MN and SN links, similar to case of using split SRB.

In the EN-DC architecture, the MN and SN use different PDCP (i.e. MN uses LTE PDCP and SN uses NR PDCP). In light of the existing RAN2 agreements, packet duplication can only be configured for the SRBs intended for SN (i.e. SCG SRBs) supporting the NR PDCP. Since the MN supports only LTE PDCP in EN-DC, packet duplication is not configurable for MCG SRBs.

The criteria for packet duplication of SRBs in both DL and UL can be determined by the network based on the type of the SRB. This is because different types of SRBs, which contain different CP messages, have different priority. For example, SRB1 (e.g. RRCConnectionReconfiguration messages, configured measurement reports) has a higher priority than SRB2 (e.g. NAS messages).

For the case of NR-NR DC architecture, both SRB1 and SRB2 can be configured for packet duplication. The CA architecture should also be considered for packet duplication of SRBs. Specifically, the scenarios considered for packet duplication for MCG SRB (i.e. SRB1 and SRB2) are listed as follows:

- CA in NR-NR: RRC packets on SRB1/SRB2 are sent via PCell and SCell links to NR PDCP in MN

- DC in NR-NR: RRC packets on SRB1/SRB2 are sent via MN and SN links to NR PDCP in MN

Packet duplication can also be used for satisfying the reliability requirement for SCG SRBs (i.e. SRB3) terminating at the SN. The SRB3, which carry measurement reports and RRCConnectionReconfiguration messages, are established between the UE and SN in order to minimize the latency associated with forwarding the RRC containers over the Xn between the MN and SN. For SRB3, packets can be transmitted directly by the UE to the SN without coordination from the MN. However, higher robustness and RRC diversity can be achieved with packet duplication in both CA and DC architectures. In this case, the scenarios considered for packet duplication for SRB3 are listed as follows:

- CA in EN-DC: RRC packets on SRB3 are sent via PSCell and SCell links to NR PDCP in SN

- CA in NR-NR: RRC packets on SRB3 are sent via PSCell and SCell links to NR PDCP in SN

- DC in EN-DC: RRC packets on SRB3 are sent via MN and SN links to NR PDCP in SN

- DC in NR-NR: RRC packets on SRB3 are sent via MN and SN links to NR PDCP in SN

Proposal 1:  For NR-NR, packet duplication for SRBs should be supported in both CA and DC architectures. The following cases should be supported:

- In CA, RRC packets on SRB1/SRB2 are sent via PCell and SCell links to NR PDCP in MN

- In CA, RRC packets on SRB3 are sent via PSCell and SCell links to NR PDCP in SN

- In DC, RRC packets on SRB1/SRB2 are sent via MN and SN links to NR PDCP in MN

- In DC, RRC packets on SRB3 are sent via MN and SN links to NR PDCP in SN

Proposal 2:  For EN-DC, packet duplication for SRB3 should be supported in both CA and DC architectures. The following cases should be supported:

- In CA, RRC packets on SRB3 are sent via PCell and SCell links to NR PDCP in SN

- In DC, RRC packets on SRB3 are sent via MN and SN links to NR PDCP in SN

- Since RRC is used to configure packet duplication, it can also be used to activate/deactivate packet duplication. A separate RRC command can be used to activate/deactivate packet duplication for an SRB.

- If an SRB is configured for packet duplication, individual RRC message requests may include an RRC activation/deactivation command to specify whether packet duplication is required for the RRC response. For example, if the gNB sends an RRC request for neighbour cell measurements, the request can include the packet duplication activation/deactivation command, rather than sending a separate RRC command for activation/deactivation. After receiving the command, the UE will respond using packet duplication on the SRB only if requested.

- Proposal 3: The RRC command to activate/deactivate packet duplication for SRBs can be included within an RRC message request.

The decision to activate packet duplication may depend on the destination node for the RRC message. For example, if a measurement report request is sent from the MN, the response from the UE to the MN should be sent on the same SRB as the request message.

However, if the channel condition to the MN node is below a threshold then packet duplication may be necessary to ensure the reliability of the RRC message even if the DL request was only sent on a single link from the MN.

In contrast, if a measurement report is requested by the SN then the response should be sent to the SN on the same SRB as the request. Since the channel condition to the SN may be higher than that of the MN, packet duplication may not be necessary for the response message.

- Proposal 4: For SRBs that have been configured for packet duplication, the activation/deactivation of packet duplication can be different for different RRC messages, depending on the destination node (i.e. MN or SN).

Since the network is aware of the each SRB message requested from the UE and provides the necessary resource configuration for UL transmissions, dynamic activation/deactivation of packet duplication is not required for SRBs. The PDCP entity in the UE performs the duplication as configured and utilizes the resource grants provided by the network to transmit the duplicate SRBs over the configured links.

- Proposal 5: No additional signalling (e.g. L1/L2 signalling) for dynamic activation/deactivation of packet duplication of SRBs is necessary.

# Conclusion

Based on the above discussion, we propose the following:

Proposal 1:  For NR-NR, packet duplication for SRBs should be supported in both CA and DC architectures. The following cases should be supported:

- In CA, RRC packets on SRB1/SRB2 are sent via PCell and SCell links to NR PDCP in MN

- In CA, RRC packets on SRB3 are sent via PSCell and SCell links to NR PDCP in SN

- In DC, RRC packets on SRB1/SRB2 are sent via MN and SN links to NR PDCP in MN

- In DC, RRC packets on SRB3 are sent via MN and SN links to NR PDCP in SN

Proposal 2:  For EN-DC, packet duplication for SRB3 should be supported in both CA and DC architectures. The following cases should be supported:

- In CA, RRC packets on SRB3 are sent via PCell and SCell links to NR PDCP in SN

- In DC, RRC packets on SRB3 are sent via MN and SN links to NR PDCP in SN

Proposal 3:	The RRC command to activate/deactivate packet duplication for SRBs can be included within an RRC message request.

Proposal 4:	For SRBs that have been configured for packet duplication, the activation/deactivation of packet duplication can be different for different RRC messages, depending on the destination node (i.e. MN or SN).

Proposal 5:	No additional signalling (e.g. L1/L2 signalling) for dynamic activation/deactivation of packet duplication of SRBs is necessary.

# References

[1]  R2-1700172 Evaluation of packet duplication in multi-connectivity.