3GPP TSG-RAN WG2 NR Ad Hoc                             	      R2-1706853

Qingdao, China, 27th – 29th June, 2017

Agenda item:	   10.3.4.3 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 		Precedence of the RRC configured mapping and reflective QoS

Document for:	Discussion and Decision

1.	Introduction

RAN2 discussed the issue of reflective QoS at the RAN2 #96 and RAN2#97bis meetings, and made following agreements.

3GPP RAN2 #96 1 For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs. 2 The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly. FFS The precedence of the RRC configured mapping and reflective QoS (e.g. can reflective QoS update an RRC configured mapping) 3GPP RAN2 #97bis 1 DL packets over Uu are not marked with “Flow ID” at least for cases where UL AS reflective mapping and NAS reflective QoS is not configured for DRB.

This document discusses the FFS point highlighted above.

2.	Discussion

As mentioned in [2], there are basically three options on the precedence of the RRC configured mapping and reflective QoS:

- Option #1: An RRC configured mapping overrides any reflective mapping for that QoS flow.

- Option #2: A newly derived reflective mapping overrides a mapping configured previously by RRC.

- Option #3: The UE always applies the most recent mapping, i.e., either provided by RRC or derived by reflective QoS.

We are trying to compare different aspects of these three options.

The first aspect is restriction in applying the new DRB mapping. Whether to override the existing DRB mapping can vary according to the above options, as shown in the Table 1. DRB mapping configured previously by RRC is NOT overridden by reflective QoS in the option #1, but is overridden in the option #2 and #3. DRB mapping derived previously by reflective QoS is NOT overridden by RRC in the option #2, but is overridden in the option #1 and #3.

In other words, DRB mapping configured previously by RRC is only overridden by RRC in case of the option #1, whereas DRB mapping derived previously by reflective QoS is only overridden by reflective QoS in case of the option #2.

Table 1. Whether to override the DRB mapping

|  |  | DRB mapping configured previously by RRC | DRB mapping derived previously by reflective QoS |
| --- | --- | --- | --- |
| Option #1 | By RRC | Overridden | Overridden |
|  | By reflective QoS | Not overridden | Overridden |
| Option #2 | By RRC | Overridden | Not overridden |
|  | By reflective QoS | Overridden | Overridden |
| Option #3 | By RRC | Overridden | Overridden |
|  | By reflective QoS | Overridden | Overridden |

Observation 1: DRB mapping configured previously by RRC is only overridden by RRC in case of the option #1, whereas DRB mapping derived previously by reflective QoS is only overridden by reflective QoS in case of the option #2.

The second aspect is signalling overhead. Based on the observation 1, in case of the option #1, DRB mapping configured previously by RRC is only overridden by RRC. Therefore, RRC procedure is always performed when the existing DRB mapping needs to be changed.

Observation 2: In case of the option #1, RRC procedure is always performed when the existing DRB mapping, which has been configured previously by RRC, needs to be changed.

The last aspect is delay in applying the new DRB mapping. Based on the observation 1, in case of the option #2, DRB mapping derived previously by reflective QoS is only overridden by reflective QoS. Therefore, reflective QoS should be triggered when the existing DRB mapping needs to be changed. But, if there is not any DL packet for the QoS flow, gNB is NOT able to trigger the reflective QoS. The gNB should postpone the reflective QoS until receiving DL packet for the QoS flow from 5G-CN.

Observation 3: In case of the option #2, the gNB postpones the reflective QoS until receiving DL packet from 5G-CN, if the existing DRB mapping, which has been derived previously by reflective QoS, needs to be changed, but there is not any DL packet for the QoS flow.

The following Table 2 is a summary of our observations.

Table 2. Comparison of options on the precedence of the RRC and reflective QoS

|  | Option #1 | Option #2 | Option #3 |
| --- | --- | --- | --- |
| Restriction in applying the new DRB mapping | DRB mapping configured previously by RRC is only overridden by RRC. | DRB mapping derived previously by reflective QoS is only overridden by reflective QoS. | None |
| Signalling overhead | It always occurs in the following case: - RRC procedure is performed when the DRB mapping configured previously by RRC needs to be changed. | It can be minimized, by using reflective QoS. |  |
| Delay in applying the new DRB mapping | gNB can initiate it any time. | gNB postpones it in the following case: - The DRB mapping derived previously by reflective QoS needs to be changed, but there is not any DL packet for the QoS flow. | gNB can initiate it any time. |

Because option #3 has advantages all of the three aspects, we propose the following:

Proposal 1:	The UE always applies the most recent mapping, i.e., either provided by RRC or derived by reflective QoS.

3.	Proposal

In this document, we present our view on the precedence of the RRC configured mapping and reflective QoS. We have following proposal:

Proposal 1: The UE always applies the most recent mapping, i.e., either provided by RRC or derived by reflective QoS.

References

[1] R2-1700029	Impact to RAN User Plane Protocol architecture	Samsung

[2] R2-1700842	QoS framework for NR	Ericsson

[3] R2-1701284	Reducing reflective QoS processing load		Samsung