3GPP TSG-RAN WG2 NR AdHoc                                                         R2-1707000

Qingdao, China, 27– 29 Jun, 2017

Agenda Item:	10.4.1.4.2

Source:	CMCC

Title:	Value of N for Cell Quality Derivation

Document for:	Discussion, Decision

Introduction

In last RAN2#98 meeting, it was agreed that the value of N for beam related cell quality derivation is configured per carrier [1]:

Agreements

-	N (used in cell quality derivation) is configured per carrier.

FFS Whether a different value can be configured for NR-SS and CSI-RS and whether it can be configured per cell.

But it has not been decided yet whether a different value of N could be configured for NR-SS and CSI-RS, as well as for per cell. This contribution will discuss on these issues.

Discussion

NR-SS vs. CSI-RS

In our understanding, NR-SS is used for UEs in RRC_IDLE, and in order to reduce system overhead, it may be transmitted in an omnidirectional way or in wide beams. On the other hand, CSI-RS is defined for RRC_CONNECTED UEs to reduce system overhead and improve the accuracy of RRM measurement, so it is likely that it may be transmitted in narrower beams.

Since the number of beams UE could measure is related to the width of beams, so that the UE could measure more beams for the case of narrower beam width. Therefore, it is likely that UE can measure more CSI-RS beams than NR-SS beams, if both of them are configured for RRM measurement, and then different values of N could be configured.

Observation 1: It is likely that UE can measure more CSI-RS beams than NR-SS beams, if both of them are configured for RRM measurement, and then different values of N could be configured.

Furthermore, it was agreed in RAN2#97bis meeting that “Events A1-A6 can be configured to use CSI-RS” and “CSI-RS configured for RRM purpose can be used to derive a cell level quality” [2]. Therefore, it is feasible to configure different values of “N”, for Events A1-A6 triggered by NR-SS and CSI-RS, to derive cell quality.

Agreements

1: CSI-RS configured for RRM purpose can be used to derive a cell level quality

2: Events A1-A6 can be configured to use CSI-RS. Events are evaluated on the cell level quality.

3	Previous agreements on measurement model and cell quality derivation are also applicable for CSI-RS.

4	When the serving cell quality is above S-Measure, the UE is not required to measure the IDLR RS and CSI-RS for neighbour cells.

Observation 2: For events triggered by NR-SS and CSI-RS, it is feasible to configure different values of “N”.

Therefore, we propose:

Proposal 1: A different value of N can be configured for NR-SS and CSI-RS based event.

Intra-frequency cell

As discussed in our contribution [3], for cells working on the same carrier frequency, the number of beams might also be different, due to different coverage requirement, different number of antennas, etc. So the number of beams to derive cell quality could also be configured differently.

To configure different N for intra-frequency cell measurement, the gNB may need to include the mapping of neighbouring cell ID and corresponding value of N in the measurement configuration, as following table 1 shows:

Table1. Configuration of N per cell

| Neighbouring Cell | Corresponding “N” |
| --- | --- |
| Neighbouring Cell 1 | N1 |
| Neighbouring Cell 2 | N2 |
| … | … |
| Neighbouring Cell K | NK |

Furthermore, to obtain the value of “N” of neighbouring cells, gNBs need to exchange the value of “N” or other related information on Xn interface. Therefore, according to above analysis, to configure cell specific “N”, additional signalling overhead will be introduced in measurement configuration and on Xn interface.

On the other hand, in last RAN2#98 meeting, it was agreed that “best N of the detected beams above absolute threshold” are used to derive cell quality and “Averaging will be based on power values (i.e. not dBm values)” [1]:

Agreements for combining of beam measurements if N > 1:

1	Averaging will be based on power values (i.e. not dBm values)

Working assumption: Average of up to best N of the detected beams above absolute threshold

In following table 2, we assume the three neighbouring cells working on the same carrier frequency but configured with different beams, e.g. 32 beams, 16 beams, and 8 beams respectively. The absolute threshold is assumed to be -110dBm, i.e. 10-11mw, and then the cells quality with different values of N are derived.

It is noted that, the more beams are configured, the narrower each beam, and then the beam’s quality is likely to be better. Hence, compared with cell 2 and 3, UE can detect more good beams from cell1.

Table2. Cells’ quality with different values of N

| Neighbouring cells working on same carrier frequency | Cell1 with 32 beams | Cell2 with 16 beams | Cell3 with 8 beams |
| --- | --- | --- | --- |
| Beams’ quality above the threshold -110dBm (10-11mw) | -90dBm(10-9mw) -95dBm(31.6*10-11mw) -100dBm(10*10-11mw) -108dBm(1.58*10-11mw) | -95dBm(31.6*10-11mw) -100dBm(10*10-11mw) -105dBm(3.16*10-11mw) | -105dBm(3.16*10-11mw) |
| Cell quality with N=5 | 35.795*10-11mw | 14.92*10-11mw | 3.16*10-11mw |
| Cell quality with N=4 | 35.795*10-11mw | 14.92*10-11mw | 3.16*10-11mw |
| Cell quality with N=3 | 47.2*10-11mw | 14.92*10-11mw | 3.16*10-11mw |
| Cell quality with N=2 | 65.8*10-11mw | 20.8*10-11mw | 3.16*10-11mw |
| Cell quality with N=1 | 100*10-11mw | 31.6*10-11mw | 3.16*10-11mw |

As showed in table 2, for all the cases of same value of N configured for three cells, cells’ quality is always in the order of cell1>cell2>cell3. For the case that different N is configured, e.g. N is 4 for cell1, 3 for cell2 and 1 for cell 3, the sequence of cells’ quality is still cell1>cell2>cell3.

This means that, whether “N” is configured same or different, the sequence of cells’ quality doesn’t change. Since all beams used to derive cell quality are good beams above the threshold, and the averaging result is not so dependent on the exact value of “N”.

Therefore, it is not necessary to configure different values of N for intra-frequency cells, since no obvious benefit but additional overhead will be introduced. Therefore:

Proposal 2: The same value of N is configured for cells working on the same carrier frequency.

Conclusion

According to above analysis, we made following observations:

Observation 1: It is likely that UE can measure more CSI-RS beams than NR-SS beams, if both of them are configured for RRM measurement, and then different values of N could be configured.

Observation 2: For events triggered by NR-SS and CSI-RS, it is feasible to configure different values of “N”.

Moreover, based on these observations we propose the following:

Proposal 1: A different value of N can be configured for NR-SS and CSI-RS based event.

Proposal 2: The same value of N is configured for cells working on the same carrier frequency.

Reference

Report of RAN2-98; ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_98/Report/.

Report of RAN2-97bis; ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_97bis/Report/.

R2-1705793, On the Number of Beams to Derive Cell Quality, CMCC