3GPP TSG-RAN WG2 NR Ad Hoc #2		R2-1707061

Qingdao, China, 27th - 29th June 2017		(revised R2-1704574)

Source:	vivo

Title:	Placement of RLC segment in MAC PDU

Agenda Item:	10.3.1.3

Document for:	Discussion and Decision

- Introduction

At the RAN2 NR AH meeting #1, data multiplexing in case of multiple TBs in one TTI was discussed in [1]. In this contribution, we further discuss placement of RLC segment in MAC PDU and provide our proposal.

- Discussion

  - Placement of RLC segment in MAC PDU

In [1], the LCP function was discussed in case that the UE might need to create multiple transport blocks (TBs) within one TTI when carrier aggregation or MIMO is used. The contribution suggested that the UE shall transmit the remainder of segmented RLC PDU in another transport block of the same subframe if such is available for the UE, and Within one transport block the RLC data PDUs of one logical channel shall be in continuously increasing sequence number order (except for RLC retransmissions).

Based on these suggests, it is hard for the UE to pre-process and parallel process multiple TBs because the starting position of the second TB depends on the ending position of the first TB.  The process time will be extended and it seems very challenging for the transmitter side to process the real time service with high throughput.

For the pre-processing and parallel processing, if the segments are put at the end of the TB, the transmitter side can assemble multiple TBs parallel from the beginning of the TB and just update the RLC and MAC header field at the end of TB, like illustrated in the Figure1, total 191 RLC SDUs can be sent in this TTI by 3 TBs. The transmitting side should pre-process and parallel-process these 3 TBs, the end of TB can be filled by the segments of RLC SN=190 and RLC SN=191.

Figure1: Data multiplexing in case of multiple TBs in one TTI

Proposal 1: The segments of RLC should be put at the end of each TB and the RLC segment SDU should be the last RLC SDUs in this TTI.

- Conclusion

In this contribution, we discuss the data multiplexing in case of multiple TBs in one TTI. We give the below proposal:

Proposal 1: The segments of RLC should be put at the end of each TB and the RLC segment SDU should be the last RLC SDUs in this TTI.

References

- R2-1700429	 Further design aspects of NR User Plane stack	Ericsson