3GPP TSG-RAN WG2 #98-AH	Tdoc R2-1707164

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.3.1.6

Source:	Ericsson

Title:	BSR triggering aspects

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in contributions.

In RAN2#98 some agreements related to BSR were reached.

Agreements

- The number of LCGs will be increased up to 8.

- The concept of periodicBSR-timer and retx-BSRtimers are reused and are configured per MAC entity

- As a baseline, the concept of logicalChannelSR-ProhibitTimer is reused in NR. It is allowed to configured infinite value for this timer.

- The logicalChannelSR-Mask is supported

# This contribution discusses when and how to trigger BSR in NR. Discussion

After RAN2#98 there was an e-mail discussion about BSR triggers. The proposed outcome of that e-mail discussion is that no new BSR triggers are needed. We think this is very unfortunate and in this contribution, we explain why, as the room for explaining technical details in an e-mail discussion is very limited. Furthermore, as roughly 9 companies expressed an interest for new BSR triggers there should be more than enough interest to really explore the technical depth of the problem, beyond what can be done in an e-mail discussion.

## Problems with existing BSR triggers

In LTE, BSR is triggered in the below example cases.

- UE has UL data to transmit (Regular BSR)

  - UL data for a logical channel which belongs to a LCG becomes available for transmission.

  - Data becomes available for a logical channel with higher priority than current ones for which data is already available for transmission.

  - no data available for transmission for any of the logical channels which belong to a LCG

- UL Grant results in padding is added to the MAC PDU (Padding BSR).

- retxBSR-Timer is expired and UE has some data to transmit (Regular BSR)

- periodicBSR-Timer is expired (Periodic BSR)

We believe that with the above framework of LTE BSR triggering will not work satisfactory for NR. Some of the above triggers trigger BSR too seldom. For instance, new data with the same or lower priority compared to currently transmitted data will not trigger a BSR. This may result in a status mismatch between UE and eNB. The retxBSR-Timer is designed to compensate the loss of a BSR transmission. The periodicBSR-Timer is used for a UE to periodically update its status to the network. To avoid redundant BSR transmissions, these timers are typically set with relatively long durations. This may lead to an additional latency for some UEs or redundant information transmitted if shorter timer values are configured.

There may be buffer status mismatch between UE and eNB based on current LTE BSR triggers.

These problems are potentially further increased in NR. The number of LCGs that should be supported is agreed to be increased to up to eight for NR. This will require new BSR formats that will increase in size to cope with all LCGs in a BSR. This will increase the redundant information if e.g. periodicity of the periodicBSR-Timer is increased. On top of this we expect the buffer status field to increase from 6 bits to 8 bits to handle the larger span in data volume, further adding to the size of the BSR. It can be noted that all BSR triggers (except periodic BSR) deals with scenarios when data arrives when there previously was no data in one way or another. To report the amount of data available at "steady-state operation", i.e., data arrives to already existing data, the gNB is limited to periodic BSR, which is a very blunt tool.

The increased number of LCGs increase the cost of periodic BSRs.

The prime targets of NR include extreme MBB rates (10 Gbps), but also much lower rates e.g. from less extreme MBB or URLLC, and moreover an increased mix of high and low rate services. This could make the buffer sizes for the different LCGs to change more or less rapidly and would make it difficult to configure a periodicBSR-Timer with a value that avoids mismatch between UE and NW and at the same time avoids redundant transmissions. Hence, a conservative setting would be a short periodicBSR-timer which comes with a high cost.

The data rates of NR will increase the risk of mismatch of buffer sizes in NR with the existing BSR framework of LTE.

In current LTE, the Periodic BSR is used to periodically update the scheduler knowledge on the UE buffer status for durations where no regular BSR is triggered. For instance, it has the advantage to resolve some deadlock situations when new data arrive after a BSR has been transmitted to the eNB.

To e.g. support ultra-reliable low latency communication (URLLC) that has 1ms latency requirement, a very short periodicBSR-Timer should be configured. However, this may lead to many redundant information transmissions when the buffers are unchanged.

Short periodicity of the periodicBSR-Timer may also lead to redundant information transmissions and unnecessary load on the system.

The periodicBSR-Timer is configured per-UE and not per logical channel group. Due to the wide targets of data rates in NR it is our view that it will be merely impossible to have a periodicBSR-Timer configured to meet all traffic rates.

The periodicBSR-Timer of LTE will with high probability result in a mismatch between the UE and NW for the LCGs buffer status or, increase the load of the system and at the same time transmit redundant information.

NR should extend the number of BSR triggers to address inefficiencies, in particular steady-state operation.

## New BSR trigger

In NR, it would be beneficial if another trigger of a BSR could be defined, so that the buffer status update is timelier delivered when needed, while not contributing to the transmission of redundant information when not needed.

In defining a new trigger, it would thus be of value if the trigger is based on events which relate to the amount of (in the BSR) reported data served by the gNB scheduler, and the inflow of new data to the UE buffer.

The triggered BSR will in this case result in that a BSR is not transmitted at a configured interval, but rather continuously depending on the actual change in UE buffer. As such, the events and criteria for the BSR trigger should be configurable by RRC signalling. This trigger would be set on a per LCG basis rather than per UE, resulting in that this trigger would cope with the broad variety traffic and data rates offered by NR.

Similar to the Periodic BSR, a BSR triggered by this event should not trigger an SR.

Below is one example on how this new BSR trigger could be defined:

- The UE maintains a variable of reported buffer status, “reported_volume”, when a BSR has been sent

- The UE also maintains a variable, “unreported_volume”, set to the amount of new data not considered in the reported BSR (reported_volume)

- The UE continuously evaluates the amount of remaining data of the “reported volume”, to a threshold configured by the NW

- When the remaining data of the reported volume is less than the threshold, and the amount of new data (unreported_volume) is not zero, a BSR is triggered.

For logical channel group that carries URLLC, the threshold can be set to the “report_volume” so that BSR is triggered when any new data arrives.

This new trigger would allow to reduce the periodicity of the periodicBSR-Timer to a large extent. The new trigger would be event driven so that it is sent only when needed, removing the redundant information transmission of the periodicBSR-Timer and lower the load of the system.

Define a new event driven BSR trigger which is related to the amount of reported data served by the gNB scheduler, and the inflow of new data to the UE buffer.

# Conclusion

In section 2 we made the following observations:

Observation 1	There may be buffer status mismatch between UE and eNB based on current LTE BSR triggers.

Observation 2	The increased number of LCGs increase the cost of periodic BSRs.

Observation 3	The data rates of NR will increase the risk of mismatch of buffer sizes in NR with the existing BSR framework of LTE.

Observation 4	Short periodicity of the periodicBSR-Timer may also lead to redundant information transmissions and unnecessary load on the system.

Observation 5	The periodicBSR-Timer of LTE will with high probability result in a mismatch between the UE and NW for the LCGs buffer status or, increase the load of the system and at the same time transmit redundant information.

Based on the discussion in Section 2 we propose the following:

Proposal 1	NR should extend the number of BSR triggers to address inefficiencies, in particular steady-state operation.

Proposal 2	Define a new event driven BSR trigger which is related to the amount of reported data served by the gNB scheduler, and the inflow of new data to the UE buffer.