3GPP TSG-RAN WG2 #AH	Tdoc R2-1707175

Qingdao, P.R. of China, 26th – 30th June 2017	update of R2-1704683

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	General HARQ aspects of SPS UL

Document for:	Discussion, Decision

# Introduction

In RAN2#98, the following agreements were made in the chairman’s notes for SPS [1]:

In NR, when the UE is configured with SPS, the UE should always skip SPS grant if there is no data to transmit, i.e., Skipping SPS grant is mandated in NR regardless of SPS periodicity.

In this paper, we discuss general HARQ aspects of SPS UL in NR. An enhancement for HARQ feedback mode for data with high reliability requirement is provided in [2], some further SPS enhancements are in [3] and our views on grant-free are in [4].

# Discussion

## Baseline HARQ feedback operation

In LTE UL SPS, the retransmissions could be performed on dynamic granted resources, in which gNB has the possibility to modify the grant and free up the configured resources for new low-latency data.

In addition, in LTE, for an UL transmission with grant, the UE can interpret whether an UL grant with NDI not-toggled is received or not as HARQ feedback. If an UL grant with NDI not-toggled (i.e., retransmission UL-grant) is received, then this indicates a failure in the gNB and UE can retransmit according to the UL-grant.

Similarly, for SPS UL, a reception of an UL-grant for retransmission should be supported and considered as a negative HARQ feedback. Due to the lack of a PHICH-like channel in NR, the feedback for UL transmissions has to be sent in PDCCH. But, to always send an explicit positive or negative feedback in PDCCH for each UL transmission induces a large signaling load in the downlink. For reduced load of the DL control channel, feedback for UL transmissions can be skipped and replaced with UL-grant for retransmission without substantially degrading the UL transmission performance.

As in LTE SPS UL, retransmission for SPS UL transmission based on an UL grant is supported.

After UL transmission, data has to be kept in the HARQ buffer in case an UL grant for retransmission is received, and consequently no new data can be transmitted on the same HARQ process. To support transmission of any new data that can arrive in the UE buffer shortly after a first transmission, multiple HARQ processes should be supported. This will allow for low latency and increased throughput of SPS UL data. Multiple HARQ processes for SPS UL is also the case for LTE.

As in LTE SPS UL, multiple HARQ processes for SPS UL are supported in NR.

## Impact of non-explicit-HARQ feedback

In LTE, a fixed timing (n+4 or n+3) is configured so that UE knows implicitly when the feedback on PHICH will be sent and proper actions are taken depending on the feedback in PHICH. If ACK is sent on PHICH, then the UE can overwrite the buffer and transmit new data on SPS UL. In NR, unless a PDCCH indicating ACK is sent, UE keeps data in the buffer and no new data can be sent in SPS UL with the same HARQ PID. Note that, this issue is not relevant in dynamic scheduling since the transmission of the new data is triggered by an UL grant with NDI-toggled from the gNB.

In NR SPS, as one straightforward solution, UE can also keep the data in the buffer after the transmission and use other HARQ PIDs for new data transmsion. Once all PIDs are used and we need to reallocate a new PID, data buffer is flushed for new data. However, this puts a limitation that the UL-grant for retransmission needs to be sent before all HARQ PIDs are exhausted. Such a limitation might not be desired as SPS can be configured with short duration by mini-slot and higher numerology and with a small number of HARQ processes.

As a more flexbile and forward-compatible solution to support sending new data in UL SPS, a maximum time T can be explicitly configured for the UE to wait for an UL grant for retransmission. This means that the UE keeps the data in the buffer, and waits for a retransmission UL grant during a maximum time T. If no UL grant is received with the same HARQ process ID when the timer expires, the UE assumes that the data is successfully delivered and it can reuse the HARQ process for new data. If a retransmission UL grant is received and it overlaps with the configured SPS grant, then the same behavior from LTE is adopted, that is, the dynamic UL grant overrides the configured SPS grant [6]. Morever, such a maximum time T can also be configured to  wait for the ACK-feedback as presented in the companion paper [2].

A timer should be reset after a transmission or retransmission, counting down from the time T.

In SPS UL, a time T after an UL transmission is configured to wait for a retransmission UL grant for the same HARQ process.

    - Details of the retransmission UL grant are for further study in RAN1.

If no retransmission UL grant is received within the time T, then UE considers the transmission successful and the HARQ process can be overwritten by new data.

## HARQ process ID (PID)

In LTE SPS and Fast UL framework (Rel-14) with multiple HARQ processes, the HARQ PID is not indicated. Formulas have been specified (TS 36.321) to derive the HARQ PID from the absolute system frame number (SFN) and subframe number (which is known in both eNB and UE). In other words, the HARQ PID is synchronized, and eNB and UE are in-sync on which process ID to use on each TTI.

However, this synchronicity restricts both gNB’s flexibility in scheduling retransmissions and UE’s ability to send new data. Typically, the retransmission is on the next TTI that has the same HARQ PID as the initial transmission according to the formula. The retransmission can also be dynamically scheduled in another TTI, but the TTI that has the HARQ PID as the initial transmission can not be used.

For NR, asynchronous HARQ feedback has been agreed in RAN1#85 such that "NR should support at least asynchronous hybrid ARQ in the DL and UL to avoid fixed timing relationship between initial transmission and re-transmission”. Therefore, a straightforward solution of process ID determination in NR is desirable. The most flexible approach is to let the UE select the HARQ PID to use and transmit this information together with the data. It is feasible to transmit HARQ PID in the UCI sent with the SPS UL data.

In SPS UL, it is beneficial to convey HARQ PID in the initial transmission.

# Conclusion

For general HARQ aspects of SPS UL in NR, made the following proposals in section 2:

- As in LTE SPS UL, retransmission for SPS UL transmission based on an UL grant is supported.

As in LTE SPS UL, multiple HARQ processes for SPS UL are supported in NR.

In SPS UL, a time T after an UL transmission is configured to wait for a retransmission UL grant for the same HARQ process.

    - Details of the retransmission UL grant are for further study in RAN1.

If no retransmission UL grant is received within the time T, then UE considers the transmission successful and the HARQ process can be overwritten by new data.

In SPS UL, it is beneficial to convey HARQ PID in the initial transmission.

# References

RAN2-98, Hangzhou Chairman notes

R2-1707176, Enhanced HARQ feedback mode in SPS

R2-1707158, SPS enhancements in NR

R2-1707174, Grant Free and Semi-Persistent Scheduling in NR

R1-1711514, On Pre-emption indication for uplink