3GPP TSG-RAN WG2 Meeting NR AH#2	R2-1707178

Qingdao, China, 27th – 29th June 2017

Agenda Item	: 10.2.12 (Slicing)

Source	: LG Electronics Inc.

Title	: AMF selection based on assistance information

Document for	: Discussion and Decision

1.	Introduction

In RAN2#98, RAN2 made agreements as follows:

Agreements:

- RAN2 assumption is that MSG3 does not to deliver assistance information for AMF selection due to RRC size constraints as in LTE.

- RAN2 assumption is that MSG5 is the earliest message that can be used to deliver assistance information for AMF selection.

According to specification below, AMF selection is based on Temp ID and assistance information related to network slice.

TS38.300:

NG-RAN selects AMF based on a Temp ID or assistance information provided by the UE over RRC. The mechanisms used in the RRC protocol are described in the next sub clause.

Table 17.3.2.1-1 AMF selection based on Temp ID and assistance information

| Temp ID | Assistance Info | AMF Selection by NG-RAN |
| --- | --- | --- |
| not available or invalid | not available | Default AMF is selected |
| not available or invalid | present | Selects AMF which supports UE requested slices |
| valid | not available, or present | Selects AMF per CN identity information in Temp ID |

This contribution discusses the highlighted case above for AMF selection when the UE provides assistance information.

2.	Discussion

RAN2 previously assumed that Msg.5 is the earliest message to deliver assistance information for AMF selection and TS38.300 specifies that a gNB performs AMF selection based on assistance information, if available, to support UE requested slices. The UE may have the configured S-NSSAI(s) for the serving PLMN or have received the allowed S-NSSAI(s) from the network. When the UE has the S-NSSAI(s), it provide the S-NSSAI(s) to the network during RRC Connection Establishment so that the gNB can select an AMF based on the NSSAI information. According to specification [1], if the UE has no allowed S-NSSAI(s) for the serving PLMN, it provides the configured S-NSSAI(s) while the UE provides the allowed S-NSSAI(s) if it has allowed S-NSSAI(s) previously received from the currently serving PLMN.

Proposal 1. The UE provides the configured S-NSSAI(s) or allowed S-NSSAI(s) as assistance information to the gNB for AMF selection.

Proposal 2. The UE requested S-NSSAI(s) information among configured S-NSSAI(s) or allowed S-NSSAI(s) for AMF selection is provided in Msg. 5.

The UE may provide either a full list of S-NSSAI(s) or a subset of the list. Considering scalability of network slices, the message size for delivering S-NSSAIs(s) will be increased depending on the size of the list when the UE provides the full S-NSSAI list. We think that the gNB will select a default AMF if it cannot select an AMF to support the UE requested S-NSSAI(s). Also, we think the services a UE requests at a particular time are limited. Therefore, a UE may not need to provide the full configured S-NSSAI(s). Similarly, although a UE has received many allowed S-NSSAI(s) from the network, the UE may need to use a few services at the time it requests connection to the gNB. Once a particular AMF is selected by gNB, the AMF itself would select otherAMF based on newly requested S-NSSAI information. Therefore, the UE may not need to provide a full list of configured S-NSSAI(s) or a full list of allowed S-NSSAI(s) to the gNB in Msg. 5 if the stored information is big enough (e.g., more than five S-NSSAIs).

Proposal 3. The UE does not need to provide a full list of S-NSSAI(s)).

Network operators may define priority for network slices so that a UE provides a subset of S-NSSAI(s) from configured S-NSSAI(s) or allowed S-NSSAI(s) received form the network. For example, S-NSSAI#1 is configured to support MMTEL voice call and S-NSSAI#2 is configured to support MMTEL video. In this case, the UE may provide only S-NSSAI#1 as assistance information assuming that MMTEL voice service is prerequisite for MMTEL video service. The network operator will define prioritized slicing configurations and may not provide the detailed mapping information to the UE. The details including visibility of slicing on the UE side need to be further discussed.

Proposal 4. How to select a subset of S-NSSAI(s) from configured S-NSSAIs or allowed S-NSSAIs depends on the operator’s requirement.

3.	Conclusion

In this contribution, we discussed how to select AMF based on assistance information. Our proposals are:

Proposal 1. The UE provides the configured S-NSSAI(s) or allowed S-NSSAI(s) as assistance information to the gNB for AMF selection.

Proposal 2. The UE requested S-NSSAI(s) information among configured S-NSSAI(s) or allowed S-NSSAI(s) for AMF selection is provided in Msg. 5.

Proposal 3. The UE does not need to provide a full list of S-NSSAI(s)).

Proposal 4. How to select a subset of S-NSSAI(s) from configured S-NSSAIs or allowed S-NSSAIs depends on the operator’s requirement.

4.	Reference

[1] TS 23.501 “System Architecture for the 5G System”