3GPP TSG-RAN WG2 Meeting #NR AH2                                                           	R2-1706409

Qingdao, China, 27th - 29th June 2017

Source:	CATT

Title:	Periodic RNA update procedure

Agenda Item:	10.4.1.6.2

Document for:	Discussion and Decision

Introduction

In the RAN2#98, periodic RAN notification area (RNA) update is agreed for the UE in RRC_INACTIVE. The related agreement is as follows.

2	A UE in RRC_INACTIVE notifies the NR RAN of RAN-based location area update (RLAU) via a resume procedure when re-selecting to a cell not belonging to the configured RAN-based notification area (RNA) and periodically.

However, the specific mechanism of periodic RNA update is open. In this contribution, we discuss several possible mechanisms, and analyze the feasibility and characteristics of them.

Discussion

We consider the following four options for periodic RNA update:

- Option 1: context retrieval and anchor relocation;

- Option 2: no anchor relocation. Target gNB sends UE to inactive directly via SRB0;

- Option 3: no anchor relocation but context retrieval.

- Option 4: no context retrieval and anchor relocation. Target gNB inform the anchor periodic RNA update. Then anchor generates the RRC message and send it via the target gNB to UE, which is kept in inactive.

Analysis of option 1: Context retrieval and anchor relocation

Figure 1 depicts the procedure of option 1.

Figure 1. Periodic RAN notification area update procedure with context retrieval and anchor relocation

In Figure 1, legacy RRC connection resume procedure with three-step is used for periodic RNA update. After target gNB receives RRC Connection Resume Complete message, it will send a patch switch request to AMF in order to relocate the anchor. Upon reception of the patch switch request, the AMF shall increase its locally kept NCC value by one and compute a new fresh NH, and send the newly computed {NH, NCC} pair to the target gNB in patch switch request ACK message. The relocation is completed after target gNB notifies source gNB to release UE context by sending the corresponding message. Additionally, the new anchor should send the UE back to inactive state if there is no service arrival for the UE. It can be seen that this option is complete from perspective of both logic and security, so it should be regarded as the baseline for periodic RNA update. However, this procedure results in CN involvement.

Proposal 1: Option 1: context retrieval and anchor relocation could be considered as the baseline for periodic RNA update.

Analysis of option 2: no anchor relocation. Target gNB sends UE to inactive directly via SRB0

In this case, the UE will not enter connected state during periodic RNA update procedure. And there is no CN involvement. The gNB sends the UE to inactive with RRC suspend in response to RRC connection resume triggered by periodic RNA update. According to [1], suspend indication to a lightly connected UE shall be integrity protected with replay attack prevention. So if light connection is taken as the baseline of inactive, the security issue exists in case that target gNB sends UE to inactive directly via SRB0.

Observation 1:  the feasibility of option 2: no anchor relocation and target gNB sends UE to inactive directly via SRB0 should be check with SA3 from security perspectives.

Analysis of option 3: no anchor relocation but context retrieval

The procedure of option 3 is depicted in Figure 2.

Figure 2. Periodic RAN notification area update procedure with context retrieval but anchor relocation

Compared with Figure 1, we can observe from Figure 2 that because of no anchor relocation, there is no interaction between RAN and CN during periodic RNA update procedure. Consequently, unlike the security mechanisms of RRC resume and handover in legacy LTE, it is not feasible to get new {NH, NCC} in anchor gNB from CN. To ensure the security of the UE in mobility, the {NH, NCC} update has to be performed in anchor gNB without notifying CN (i.e. AMF). This results in that the {NH, NCC} in RAN is out of sync with that in CN. When anchor gNB needs to interact with CN later (e.g. when the UE moves out of the configured RNA area), it is hard to perform {NH, NCC} update in CN correctly. Therefore, there is security issue in option 3；

Observation 2:  Option 3: no anchor relocation but context retrieval, may result in out of synchronization of security context (e.g. NH, NCC) at RAN and CN. .The feasibility of Option 3 should be checked with SA3 on the security issue due to no interaction between RAN and CN in case of no anchor relocation but context retrieval during periodic RNA update.

Analysis of option 4: no context retrieval and anchor relocation; anchor gNB generates RRC message.

This option is similar with option 1 in [1],  i.e., old eNB decides the suspension for the UE, send X2 suspend indication to the new eNB, and new eNB rejects the UE with suspend indication (to move the UE to Suspend as per Rel.13 Suspend procedure). SA3 replied that “suspend Indication shall be integrity protected. Therefore, if RAN2/RAN3 decides to go forward with Option 1, SA3 would be happy to review the solution and develop a security solution”.  The related security issue is analyzed as follows:

Due to no context retrieval, target gNB cannot perform integrity protection for the message from anchor. Thus in order to fulfill the above SA3’s reply with light connection as the baseline, anchor is required to perform integrity protection for the transmitted RRC message. In this way, the RRC message from anchor should be PDCP PDU, including MAC-I after integrity protection. The involved problems contain 1) target gNB may not be able to differentiate the MAC-I from the PDCP PDU transmitted by anchor; 2) the UE may not be able to identify the MAC-I  as well as other lower layer configuration of the RRC message forwarded by target gNB due to no context retrieval. The former is expected to be resolved via Xn interface protocol. And the latter is expected to be ensured by making target gNB adopt default configuration and indicate the UE to adopt default configuration.

Observation 3:  Option 4:no context retrieval and anchor relocation, and anchor generates the RRC message and send it via the target gNB to UE which is kept in inactive, issues on the following aspects:

1) Target gNB may not be able to differentiate the MAC-I from the PDCP PDU transmitted by anchor;

2) The UE may not be able to identify the MAC-I  as well as other lower layer configuration of the RRC message forwarded by target gNB due to no context retrieval.

Option 2, option 3 and option 4 have security implications and should be checked with SA3 on feasibility. While Option 1 has the CN involvement, option 2 to 4 can be performed without interaction with the core network. Hence the UE mobility within RNA can be kept transparent to the CN with option 2, 3 and 4.

Proposal 2: Send LS to SA3 on security implications of option 2, 3 and 4.

Conclusion

This contribution discusses several typical possible mechanisms for periodic RNA update, and analyzes the feasibility and characteristics of them. According to the analysis in section 2, we have the following observations:

Observation 1:  the feasibility of option 2: no anchor relocation and target gNB sends UE to inactive directly via SRB0 should be check with SA3 from security perspectives.

Observation 2:  Option 3: no anchor relocation but context retrieval, may result in out of synchronization of security context (e.g. NH, NCC) at RAN and CN. .The feasibility of Option 3 should be checked with SA3 on the security issue due to no interaction between RAN and CN in case of no anchor relocation but context retrieval during periodic RNA update.

Observation 3:  Option 4:no context retrieval and anchor relocation, and anchor generates the RRC message and send it via the target gNB to UE which is kept in inactive, issues on the following aspects:

Besides, we get the following proposals:

Proposal 1: Option 1: context retrieval and anchor relocation could be considered as the baseline for periodic RNA update.

Proposal 2: Send LS to SA3 on security implications of option 2, 3 and 4.

Reference

[1] R2-167436/S3-162087, Reply LS on Light Connected UE, RAN2#96, 2016