3GPP TSG-RAN2 Meeting AH#1	R2-1800159

# Vancouver, Canada, 22nd – 26th January 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | yyyy | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on maximum PDCP SDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2017-12-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN2#99 meeting, it was agreed that (this assumption is confirmed by SA3 later in R2-1712108) - Unless SA3 raises some concerns, the maximum supported size of a PDCP SDU is 9000 bytes. On the other hand, at RAN2#99bis meeting, it was agreed that NR PDCP can be configured with LTE security algorithm, when used for bearers anchored at eNB like in EN-DC scenario. 3 Adopt following signalling solution for algorithms: a) Algorithms for the bearers using KeNB and LTE PDCP: use securityConfigHO, b) Algorithms for the bearers using KeNB and NR PDCP: use new signalling in RadioBearerConfig (however, the algorithm should be same as in securityConfigHO) and c) Algorithms for the bearers using S-KgNB and NR PDCP: use new signalling in RadioBearerConfig. New signalling applies to all bearers in RadioBearerConfig. 3i Case a and b can configure LTE algorithms, and case c can configure NR algorithms (This is for Rel-15 and may be re-discussed in future releases) In this case, i.e., when LTE algorithm is used, the maximum supported size of a PDCP SDU should follow the LTE specification (TS 36.323), i.e., 8188 bytes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the 9000-byte SDU is only supported when NR algorithm is used in section 4.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When LTE algorithm is used for NR PDCP, 9000-byte SDU is still assumed to be supported, which is not true. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

4.3.1	Services provided to upper layers

The PDCP layer provides its services to the RRC or SDAP layers. The following services are provided by PDCP to upper layers:

-	transfer of user plane data;

-	transfer of control plane data;

-	header compression;

-	ciphering;

-	integrity protection.

The maximum supported size of a PDCP SDU is 9000 bytes when NR security algorithm is applied, and is 8188 bytes when LTE security algorithm is applied. The maximum supported size of a PDCP Control PDU is 9000 bytes.

CHANGE END