3GPP TSG-RAN WG2 NR Ad hoc 0118	R2-1800175

Vancouver, Canada, 22nd January – 26th January 2018

Source:	CATT

Title:	RNTIs in MAC specification

Agenda Item:	10.3.1.13

Document for:	Discussion and Decision

Introduction

RAN1 introduced some new RNTIs and informed RAN2 via [1]. This contribution aims to complete the RNTI values and usages in MAC specification.

Discussion

On top of the RNTIs that have been newly included in 38.321 by RAN2, RAN1 introduced below some new RNTIs. They are all UE specific.

| Parameter name in specification | New or existing parameter | Parameter name in text | Description |
| --- | --- | --- | --- |
| SPCSI-RNTI | New | SPCSI-RNTI | SPCSI -RNTI used for activation of Semi-persistent CSI reporting on PUSCH. |
| INT-RNTI | New | INT-RNTI | RNTI used for indication pre-emption in DL. Also connected to monitoring of a Type2-PDCCH common search space. |
| SFI-RNTI | New | SFI-RNTI | RNTI used for SFI (Slot Format Indication) on the given cell. |

Proposal 1: Include three new RNTIs: SPCSI-RNTI, INT-RNTI, and SFI-RNTI in 38.321.

Additionally, LTE MAC specification provides a useful table listing all RNTIs and their usage. We think it should also be present in the NR MAC specification.

Proposal 2: Add a table for RNTI usage as in LTE.

Conclusion

This contribution discusses the RNTIs in 38.321. We provide below proposals and corresponding TP.

Proposal 1: Include three new RNTIs: SPCSI-RNTI, INT-RNTI, and SFI-RNTI in 38.321.

Proposal 2: Add a table for RNTI usage as in LTE.

Reference

- R1-1721616, LS on RRC parameters for NR

- 3GPP 38.321, v15.0.0

- Text proposal to 38.321

- 7.1	RNTI values

RNTI values are presented in Table 7.1-1.

Table 7.1-1: RNTI values.

| Value (hexa-decimal) | RNTI |
| --- | --- |
| 0000 | N/A |
| 0001–FFEF | RA-RNTI, Temporary C-RNTI, C-RNTI, CS-RNTI, TPC-CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, SPCSI-RNTI, INT-RNTI, SFI-RNTI, and TPC-SRS-RNTI |
| FFF0–FFFD | Reserved |
| FFFE | P-RNTI |
| FFFF | SI-RNTI |

# Table 7.1-2: RNTI usage.

| RNTI | Usage | Transport Channel | Logical Channel |
| --- | --- | --- | --- |
| P-RNTI | Paging and System Information change notification | PCH | PCCH |
| SI-RNTI | Broadcast of System Information | DL-SCH | BCCH, BR-BCCH |
| RA-RNTI | Random Access Response | DL-SCH | N/A |
| Temporary C-RNTI | Contention Resolution (when no valid C-RNTI is available) | DL-SCH | CCCH |
| Temporary C-RNTI | Msg3 transmission | UL-SCH | CCCH, DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | UL-SCH | DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | DL-SCH | CCCH, DCCH, DTCH |
| C-RNTI | Triggering of PDCCH ordered random access | N/A | N/A |
| CS-RNTI | Configured scheduled unicast transmission (activation, reactivation and retransmission) | DL-SCH, UL-SCH | DCCH, DTCH |
| CS-RNTI | Configured scheduled unicast transmission (deactivation) | N/A | N/A |
| TPC-CS-RNTI | Configured scheduling uplink power control | N/A | N/A |
| TPC-PUCCH-RNTI | PUCCH power control | N/A | N/A |
| TPC-PUSCH-RNTI | PUSCH power control | N/A | N/A |
| TPC-SRS-RNTI | SRS trigger and power control | N/A | N/A |
| SPCSI-RNTI | Activation of Semi-persistent CSI reporting on PUSCH. | N/A | N/A |
| INT-RNTI | Indication pre-emption in DL | N/A | N/A |
| SFI-RNTI | SFI (Slot Format Indication) on the given cell | N/A | N/A |