3GPP TSG-RAN WG2 Meeting #NR AH 1801 	Tdoc R2-1800284

Vancouver, Canada, Jan 22 - 26, 2017 	(Revision of R2-1712484)

Agenda Item: 	10.4.1.6.6

Source:	Ericsson

Title:	SS Block index dependent system information

Document for:	Discussion, Decision

# Introduction

The SS Block reception will provide the UE with information related to the SS Block position within the SS Burst Set, here denoted SS Block Index (SSBI). In case of e.g. analogue beam-sweeping there will be a simple time relation between the SSBI and the beam index.

In case a cell has several beams, then the system information for the cell will contain information related to all beams within that cell. In some cases, we may want to be able to specify that the RRC_IDLE and RRC_INACTIVE mode UE-behavior shall depend on which beam the UE is currently in. For example, the timing of the PRACH window might be different for different beams (e.g. to enable analogue PRACH reception beam-forming). Or we may want the UE to use different PRACH preambles depending on which SS Block beam it receives (e.g. to enable beam-identification). To allow for this, we need a possibility to specify different beam-dependent versions of the minimum SI.

# Discussion

When a UE reads the minimum SI of a cell the UE will receive information related to all beams in the cell. In some cases, the minimum SI may contain different values for one or more parameters (e.g. PRACH timing window or PRACH root sequence index) corresponding to different SS Block beams in the cell. See [1] for further details and discussion on this topic.

In case the different beams in a cell have different SI, then the SSBI can be used to differentiate which part of the SI that is valid in a beam.

Some SI parameter values can be different between SS Block beams belonging to the same cell.

The SSBI (SS Block Index) is used as an index/identifier that enables differentiation of SI in different SS Block beams of the same cell.

The UE may read the minimum SI and extract the SI valid in the current beam (e.g. corresponding to a SSBI). Information related to other beams may be stored and used later, should the UE move to another beam.

Some of the PRACH configuration parameters (related to PRACH preamble sequence, timing, and frequency offset) should be defined as optional lists to enable configuration of different parameter values for different SSBIs.

Some of the RAR configuration parameters (e.g. RAR timing) should be defined as optional lists to enable configuration of different parameter values for different SSBIs.

# Conclusion

Based on the discussion above we propose the following:

Proposal 1	The SSBI (SS Block Index) is used as an index/identifier that enables differentiation of SI in different SS Block beams of the same cell.

Proposal 2	Some of the PRACH configuration parameters (related to PRACH preamble sequence, timing, and frequency offset) should be defined as optional lists to enable configuration of different parameter values for different SSBIs.

Proposal 3	Some of the RAR configuration parameters (e.g. RAR timing) should be defined as optional lists to enable configuration of different parameter values for different SSBIs.

# References

R1-1714043, 4-step random access procedure, Ericsson, 3GPP TSG RAN WG1 Meeting #90, Prague, Czech Republic, 21st – 25th August 2017.