33GPP TSG-RAN WG2 #AH NR 1801	R2-1800332

Vancouver, Canada, Jan 22nd – 26th, 2018        	Revision of R2-1713468

Agenda Item:	10.3.1.10

Source:	Ericsson

Title:	DRX with short on-duration and Wake-up signaling

Document for:	Discussion, Decision

# Introduction

There is a constant need for improving the battery lifetime of mobile devices. Thus, reducing the time a device needs to monitor the DL will help the UE save battery and extending the battery lifetime.

Active UEs in connected mode must continuously monitor the downlink for Downlink Control Information, DCI, to see if there is a downlink assignment or uplink grant to which they should respond. The power saving feature connected mode DRX, C-DRX, allows the UE to enter a sleep mode after a time of inactivity in which the UE only wakes up to monitor PDCCH according to a configured On-Duration time per C-DRX Cycle, see Figure 1.

C-DRX Cycle

C-DRX Cycle

On-Duration

On-Duration

On-Duration

On-Duration

C-DRX Cycle

On-Duration

C-DRX Cycle

On-Duration

Figure 1: C-DRX cycles with On-Duration periods in LTE

When transmitting a DCI to a UE, a CRC is attached to each DCI message payload. The identity of the device or devices, i.e. the Radio Network Temporary identifier, RNTI, is included in the CRC calculation and not explicitly transmitted. Depending on the purpose of the DCI, different RNTIs are used. When the UE monitors and tries to decode the PDCCH, the UE will check the CRC using a set of assigned RNTIs. If the CRC check passes, the message is declared to be correctly received and intended for the UE. Thus, the identity of the device that message is intended for is implicitly encoded in the CRC and not explicitly transmitted.

When the UE wakes up in each C-DRX cycle, it needs to monitor the PDCCH during the On-Duration period. A typical setting for LTE C-DRX On-Duration is observed to be in the order of tens of milliseconds and thus, the UE must stay awake for the entire On-Duration even if there is no data scheduled for the UE. This could be viewed as a power waste for the UE.

Reducing monitoring time in C-DRX will reduce energy consumption.

The power consumption is further increased by the fact that the UE is required to be prepared to both receive and transmit data during and immediately following reception of a valid DCI during the On-Duration, i.e.,  the UE is expected to decode the data received and provide HARQ feedback shortly after reception of a valid DCI implying that the UE in order to perform the delay sensitive processing, (HARQ RTT etc.), in C-DRX need to have more processing hardware activated which consumes more power. In most cases, no DCI will be received and energy used to prepare for transmitting will be of waste.

The more processing hardware activated, the more power is consumed.

# Wake-up signaling

The use of wake-up signaling for UEs in connected mode DRX has earlier been discussed in 3GPP. The wake-up signaling in this paper proposes an alternative solution that would avoid the use of a specific wake-up receiver and thereby potentially reducing both cost and complexity but at the same time give similar energy savings.

The length of the On-Duration timer affects the reachability of the UE and the scheduling flexibility for the network but the longer On-Duration time, the more energy is used in the UE.

Additionally, we want to allow the UE to not prepare its transmitting chain and thereby save additional power when configured with this very short onDurationTimer, illustrated as "Wake up monitoring" in the figure below. In order to get the UE into continuously monitor PDCCH at its next onDuration, the gNB can send a DCI message during the wake-up monitoring. If this message is not scrambled with the C-RNTI of the UE but with a DRX-RNTI, shared by several UEs, then several UEs can be woken up with a single message. Then, a very short wake-up duration could be allowed for each UE when monitoring the common search space or common CORESET for a valid PDCCH to see if any DCI has a CRC attached with the assigned DRX-RNTI included in the calculation. When the UE monitors and tries to decode the PDCCH, the UE will check the CRC using the assigned DRX-RNTI. If the CRC checks, the message is declared to be correctly received and intended for the UE and the UE would know that it e.g. should start monitoring the PDCCH according to a longer on-Duration time, at the next C-DRX cycle, see Figure 2.

Wake-up
Monitoring with DRX-RNTI received

Wake-up
Monitoring with DRX-RNTI received

On-Duration monitoring

On-Duration monitoring

C-DRX Cycle

C-DRX Cycle

C-DRX Cycle

Wake-up
Monitoring

C-DRX Cycle

Wake-up
Monitoring

Figure 2 Wake-up monitoring

If the CRC check fails, the message is not intended as a wake-up message for the UE and it would be able to go back to sleep until the next wake-up occasion according to the C-DRX cycle, see Figure 2.

This wake-up scheme comes with a cost of an increased delay. However, design choices could be made to limit the delay by having the UEs waking up a predefined time after the wake-up signal has been received and there are also services where power saving is more important than short delay.

Add a new DCI message ("wake up message") on PDCCH using a DRX-RNTI upon reception of which the UE switches to a longer On-Duration at next occasion.

This wake-up scheme should be regarded as an add-on to the existing C-DRX framework and used when energy savings are the highest concern.

UEs in C-DRX may be optionally configured to use wake-up signaling using the DRX-RNTI and the very short onDurationTimer.

# Conclusion

We propose:

Proposal 1	Add a new DCI message ("wake up message") on PDCCH using a DRX-RNTI upon reception of which the UE switches to a longer On-Duration at next occasion.

Proposal 2	UEs in C-DRX may be optionally configured to use wake-up signaling using the DRX-RNTI and the very short onDurationTimer.