3GPP TSG-RAN WG2 AH-1801	Tdoc R2-1800346

Vancouver, Canada, 22nd– 26th Jan 2018 	(Resubmission of R2-1712340)

Agenda Item:	10.4.5.5

Source:	Ericsson

Title:	Cell reselection measurement rules

Document for:	Discussion, Decision

# Introduction

This paper discusses the cell reselection measurement rule for a camped UE in RRC_IDLE and RRC_INACTIVE states. In NR, the following are agreed in the previous meetings regarding the SS Burst Set periodicity and SS Block based RRM measurement timing configuration for IDLE mode.

Agreements - Same set of configuration values for SS periodicity for CONNECTED/IDLE & non-standalone cases. Values for configuration set for CONNECTED/IDLE & non-standalone case - {5, 10, 20, 40, 80, 160} ms - SS burst set periodicity default value for initial cell selection: 20/20 msec. - The transmission of SS blocks within SS burst set is confined to a 5 ms window regardless of SS burst set periodicity. - Regarding the SS block based RRM measurement timing configuration (SMTC) i.e., measurement window periodicity/duration/offset information for UE RRM measurement per frequency carrier, - For IDLE mode measurements, only single SMTC is configured per frequency carrier

This paper is a resubmission of R2-1712340.

# Discussion

Based on the agreements, it is still an open issue whether a camped UE in RRC_IDLE state or RRC_INACTIVE state can be configured by the network to know the SS Block based RRM measurement window for the serving cell.

It is an open issue in NR whether a camped UE in RRC_IDLE or RRC_INACTIVE state can derive the SS Block based RRM measurement window for the serving cell.

In LTE, a camped UE in RRC_IDLE or RRC_INACTIVE state is required to measure the serving cell at least every DRX cycle, while the minimum measurement periodicity of intra-frequency measurements can be larger (1, 2, or 4 DRX cycles depending on the DRX cycle length). Furthermore, a camped UE in RRC_IDLE or RRC_INACTIVE state may choose not to perform intra-frequency measurements if the serving cell fulfils Srxlev > SintraSearchP and Squal > SIntraSearchQ shown in the table below.

Following rules are used by the UE to limit needed measurements (TS 36.304): - If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements. - Otherwise, the UE shall perform intra-frequency measurements.

In our view, NR should have similar schemes as in LTE for UE in RRC_IDLE or RRC_INACTIVE state to conserve the energy. Specifically speaking, the minimum measurement periodicity of the intra-frequency SS Block based measurement for a camped UE in RRC_IDLE or RRC_INACTIVE state in NR can be larger than the minimum measurement periodicity of the serving cell measurement. A camped UE in RRC_IDLE or RRC_INACTIVE state in NR can choose not to perform the intra-frequency measurements if the serving cell fulfils a certain condition.

Given the above discussions, we have the following proposal.

The network should provide information to a camped UE in RRC_IDLE or RRC_INACTIVE state, so that the UE can choose to only perform the measurement during a time window that covers the SS Blocks of an SS Burst Set in the serving cell if the serving cell fulfils a certain condition. For example, the number of SS Blocks transmitted by the serving cell in an SS Burst Set can be included in the system information.

# Conclusion

In this paper, we have revisited some of the energy-conservation measurement concepts in LTE for UE in RRC_IDLE state and discussed an open issue for SS Block based measurement in RRC_IDLE and RRC_INACTIVE states.

In section 2 we made the following observation:

Observation 1	It is an open issue in NR whether a camped UE in RRC_IDLE or RRC_INACTIVE state can derive the SS Block based RRM measurement window for the serving cell.

Based on the discussion in section 2 we propose the following:

Proposal 1	The network should provide information to a camped UE in RRC_IDLE or RRC_INACTIVE state, so that the UE can choose to only perform the measurement during a time window that covers the SS Blocks of an SS Burst Set in the serving cell if the serving cell fulfils a certain condition. For example, the number of SS Blocks transmitted by the serving cell in an SS Burst Set can be included in the system information.