3GPP TSG-RAN WG2 NR AH#1801	Tdoc R2-1800693

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.4.4

Source:	Ericsson

Title:	Notification Control

Document for:	Discussion, Decision

# Introduction

Notification Control feature is part of NR QoS framework. TS 23.501 states following about the notification control:

5.7.2.4	Notification control

The Notification control indicates whether notifications are requested from the RAN when the GFBR can no longer (or again) be fulfilled for a QoS Flow during the lifetime of the QoS Flow. If, for a given GBR QoS Flow, notification control is enabled and the NG-RAN determines that the GFBR cannot be fulfilled, RAN shall send a notification towards SMF. The RAN shall keep the QoS Flow, and should try to fulfil the GFBR. Upon receiving a notification from the RAN that the GFBR cannot be fulfilled, the 5GC may initiate N2 signalling to modify or remove the QoS Flow. When applicable, NG-RAN sends a new notification, informing SMF that the GFBR can be fulfilled again. After a configured time, the NG-RAN may send a subsequent notification that the GFBR cannot be fulfilled.

# Discussion

Notification control may be configured for GBR Flows. When configured, RAN has possibility to send indication to the CN if requirements set for the QoS Flow cannot be fulfilled. The decision is made based on parameters associated with the QoS Flows and it is up to the implementation decide exactly when the notification is send.

In the last RAN meeting, some companies proposed that there is need to associate additional information to the notification control message. For example, information about the scheduler state. This, however, is problematic approach as the scheduling information is dynamic may already be outdated when notification reaches the destination. Therefore, the notification control should be kept simple from RAN2 perspective.

The Notification Control only applies in the case when QoS Flow has already been established and RAN has not made decision on removing a Flow. In QoS Flow establishment, RAN has possibility to reject the QoS Flow if it cannot fulfil the requirements set by the QoS parameters. Additionally, if RAN decides to release QoS Flow, there is no need to send notification, as the QoS Flow release message is sent to the CN. It hardly makes sense to send notification of congestion if QoS Flow is removed. Therefore, Notification Control only applies between QoS Flow establishment and QoS Flow release.

Notification control only applies during the lifetime of QoS Flow, i.e. after RAN has accepted the QoS Flow and before RAN has decided to release QoS Flow.

From RAN2 perspective, the notification control is mostly signalling between gNB and SMF. Additionally, Notification Control includes behaviour that 5GC executes upon receiving the notification from RAN. The discussion of Notification Control should be left for RAN3 and SA2.

Notification Control has minimal impact on RAN2 specification

Discussion of Notification Control is left for RAN3 and SA2.

# Conclusion

In section 2 we made the following observations:

Observation 1	Notification control only applies during the lifetime of QoS Flow, i.e. after RAN has accepted the QoS Flow and before RAN has decided to release QoS Flow.

Observation 2	Notification Control has minimal impact on RAN2 specification

Based on the discussion in section 2 we propose the following:

Proposal 1	Discussion of Notification Control is left for RAN3 and SA2.

# References

3GPP TS 23.501 V1.5.0 (2017-11). System Architecture for the 5G System