3GPP TSG-RAN WG2 #AH NR 1801	Tdoc R2-1800702

Vancouver, Canada, Jan 22nd – 26th, 2018

Agenda Item:	10.3.3.5

Source:	Ericsson

Title:	PDCP duplication and discard

Document for:	Discussion, Decision

# Introduction

In this contribution, PDCP duplication is discussed in relation to discarding of packets. Two open issues are discussed: avoidance of redundant duplicate transmission when delivery of a duplicate has been confirmed via another transmission leg, and the PDCP behaviour when PDCP data duplication is deactivated.

# Discussion

In the current transmit operation, when duplication is configured and activated, when submitting a PDCP Data PDU, the PDU is duplicated and submitted to both associated RLC entities (TS 38.323, section 5.21). The transmission of the PDUs on the lower layers may be subject to different latencies resulting from e.g. RLC queueing, RLC retransmissions, RLC segmentation, HARQ retransmissions, different numerologies on the involved carriers, un-synchronized scheduling, etc. Therefore, when providing PDCP duplicates to the two associated lower layers, reception of the duplicates cannot be assumed on the same time. When the reception time difference becomes too large, reception of the later duplicate becomes redundant, thus transmission of the duplicate should be avoided in this case.

Therefore, the following cases should be considered on PDCP within the discard procedure when duplication is used.

## SDU discard after discardTimer expiry

The PDCP SDU discard timer considers a maximum waiting time of a PDCP SDU in the transmit buffer. Upon expiry, the SDU as well as the corresponding PDU are discarded, and the discard is indicated to RLC, where RLC SDU or PDU, if not yet transmitted, are discarded as well.

The specification covers this case already, and it should be considered independently of whether duplication is used or not.

## SDU discard after successful delivery

When successful delivery of a PDCP SDU is confirmed by a PDCP status report, the SDU along with PDU are discarded as well, and discard is indicated to RLC.

The specification covers this case already, and it should be considered independently of whether duplication is used or not.

## PDU discard after successful delivery (AM)

New in duplication is the case in AM, where successful delivery of a PDU is indicated from RLC (due to RLC status report reception) to PDCP, and where transmission of the duplicate of this PDU may still be ongoing in the other RLC entity.

In this case, continuing (attempting) transmission of the duplicate is redundant. It is useful to discard the duplicate in this case, in order to avoid overhead, as well as reduce the queuing times for subsequent PDUs to be transmitted in the slower RLC entity. As described above, avoiding reception time differences among the duplicate transmission is essential for the usefulness of the PDCP data duplication feature. Therefore, we propose that an indication of a successful delivery of an PDCP PDU indicated by one RLC entity should trigger the discard of the corresponding PDCP PDU duplicate on the other RLC entity. On RLC, the currently specified RLC procedures apply, i.e. the corresponding RLC SDU and RLC PDU is discarded in case it had not been transmitted yet.

For DC and CA PDCP duplication, when the successful delivery of a PDCP PDU is confirmed by one of the associated RLC entities:

  - PDCP shall discard the PDCP PDU along with the corresponding PDCP SDU.

  - PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU.

## PDU discard after time (AM and UM)

Same as for AM operation, also in UM operation, due to submission of PDCP PDU duplicate to the two RLC entities at the same time, but their potentially different transmission times on these RLC entities, reception time differences of the duplicates may increase beyond a point where a duplicate reception is still useful. This case where duplicate transmissions among the RLC entities drift apart, must thus be avoided.

Example: for URLLC support, any queueing delay that a duplicate transmission might undergo, while the original transmission takes place immediately, destroys the purpose of introducing duplication in the first place: if a PDU would be pushed as original and duplicate to RLC1 and RLC2, transmitted via RLC1 but queuing up for some time in RLC2 (since previous duplicate packets were pushed to RLC2 as well) the duplicate PDU would be delayed and transmitted only after the RLC2 queueing time. Then, at the time of the reception, the PDU duplicate would be worthless.

For AM, successful transmission is indicated to PDCP, and from that point onwards, no duplicate of this PDU is required anymore. For UM, another mechanism needs to be established. Therefore, a PDCP PDU should be discarded for duplicate transmission if it could not be transmitted within a certain time after the original had been sent. The time should be configurable according to traffic latency targets. Since successful transmission is indicated in AM towards PDCP based on the RLC status report frequency, which may be configured conservatively for overhead avoidance, it may be useful to use the timer-based discard also in AM mode, leading to a quicker discard.

This can be achieved by a timer which can be implemented on PDCP or RLC. If implemented in RLC, the timer would relate to a maximum queueing time in the respective RLC entity. Otherwise, a duplicateDiscardTimer could be implemented on PDCP.

Both options would be different to the current PDCP SDU discard timer, since the PDCP SDU discard timer relates to a maximum queueing time before the first (original) transmission of the PDU.

As the duplication function is introduced on PDCP layer, we prefer a solution where a PDU duplicateDiscardTimer is introduced on PDCP, as implemented in CR [1].

For DC and CA PDCP duplication, a PDCP PDU discard timer is started when a PDCP PDU is initially submitted to lower layers. At expiry of the timer:

  - PDCP shall discard the PDCP PDU along with the corresponding PDCP SDU.

  - PDCP should indicate to lower layers to discard the corresponding PDCP PDU.

## Discard at duplication deactivation

When the UE receives a MAC CE to deactivate PDCP data duplication, the PDCP entity stops indicating duplicate data available for the MAC entity associated with the secondary RLC entity. The question is whether outstanding duplicate data transmission of the secondary RLC entity should be continued to be transmitted on this secondary RLC entity or not.

We consider that it is important to discard the outstanding not yet transmitted RLC data, since if such data would stay, it could propagate towards the PDCP receiver when duplication is resumed at a later point in time. If the PDCP moved by more than half a sequence number space at that time, the old PDCP PDU would likely cause HFN de-sync and cause massive data loss. Such cases must hence be avoided. This consideration should be done especially in the light of the introduced MAC CE for dynamic activation and deactivation. I.e. in a scenario where frequent deactivation is wanted, the deactivation should also be applied immediately, i.e. duplicate transmission stopped on the secondary RLC as soon as possible. Like in the other cases above, the indicated PDCP PDU discard to RLC, leading to RLC SDU discard will nevertheless only be applied to RLC SDUs and associated RLC PDUs that are not yet transmitted.

For CA and DC, upon deactivation of PDCP data duplication, the UE transmitting PDCP entity:

  - PDCP should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity.

# Conclusions

Based on the discussion in section 2 we propose the following:

Proposal 1	For DC and CA PDCP duplication, when the successful delivery of a PDCP PDU is confirmed by one of the associated RLC entities:

a.	PDCP shall discard the PDCP PDU along with the corresponding PDCP SDU.

b.	PDCP should indicate to the other associated RLC entity to discard the corresponding PDCP PDU.

Proposal 2	For DC and CA PDCP duplication, a PDCP PDU discard timer is started when a PDCP PDU is initially submitted to lower layers. At expiry of the timer:

a.	PDCP shall discard the PDCP PDU along with the corresponding PDCP SDU.

b.	PDCP should indicate to lower layers to discard the corresponding PDCP PDU.

Proposal 3	For CA and DC, upon deactivation of PDCP data duplication, the UE transmitting PDCP entity:

a.	PDCP should indicate to lower layers to discard all PDCP PDUs provided for duplicate transmission to the secondary RLC entity.

# References

R2-1800701, draftCR on PDCP duplication and discard, Ericsson, 3GPP RAN2# NR Adhoc 1801, Vancouver, Canada, 22nd – 26th January 2018