3GPP TSG-RAN WG2 NR Ad hoc 1801		R2-1800898

Vancouver, Canada, 22nd-26th January 2018

Source:	vivo

Title:	Text proposal for repetition of the configured grant

Agenda Item:	10.3.1.8

Document for:	Discussion and Decision

- Introduction

In RAN1 #91, it was agreed that the restriction to the initial transmission of a TB should be applied in case that the repetition is configured for NR.

Agreements: - For UL transmission without UL grant, - The n-th transmission occasion of a K repetitions is associated with the (mod(n-1,4)+1)-th value in the configured RV sequence {RV1, RV2, RV3, RV4}, where n=1, 2, …, K. - For RV sequence {0, 2, 3, 1}, - The initial transmission of a TB shall start at the first transmission occasion of the K repetitions. - For RV sequence {0, 3, 0, 3}, - The initial transmission of a TB can start at any of the transmission occasions of the K repetitions that are associated with RV=0. - (working assumption) For RV sequence {0, 0, 0, 0}, - The initial transmission of a TB can start at any of the transmission occasions of the K repetitions when K=1, 2 or 4; - The initial transmission of a TB can start at any of the transmission occasions of the K repetitions, except the last transmission occasion when K=8. - For any RV sequence, repetition end at the last transmission occasion within the period P. - Note: The transmission occasion (TO) refers to the time domain resource allocation of one repetition in an aggregation with factor K where the aggregated transmission occasions start in resources configured by the offset and the period. - FFS: interaction with SFI

The agreements were also captured in TS 38.314.

Section 6.1.2.3            Resource allocation for uplink transmission without grant

The RRC-configured parameter UL-TWG-repK and UL-TWG-RV-rep define the K repetitions to be applied to the transmitted TB, and the redundancy version pattern to be applied to the repetitions. For the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. The initial transmission of a TB may start at - the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1}, - any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3}, - any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K=8.

In this contribution, we provide the MAC TP for the restriction to the initial transmission of a TB in case that the repetition is configured for NR.

- Discussion

  - Text proposal

- 5.4.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell with configured uplink (including the case when it is configured with supplementaryUplink), which maintains a number of parallel HARQ processes.

The number of parallel UL HARQ processes per HARQ entity is specified in TS 38.214 [7].

Each HARQ process supports one TB.

Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response, HARQ process identifier 0 is used.

When repetition is configured with repK >1, the parameter repK provides the number of repetitions of a TB within a bundle. Repetition operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle HARQ retransmissions are non-adaptive and triggered without waiting for feedback from previous transmissions according to repK.

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process(es) associated with this grant, and for each identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

3>	else:

4>	obtain the MAC PDU to transmit from the "Multiplexing and assembly" entity, if any,  according to the restriction to the initial transmission of a TB in section 6.1.2.3 of [xx, TS38.314] in case that the repetition is configured;

3>	if a MAC PDU to transmit has been obtained:

4>	deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a new transmission.

4>	if the uplink grant is addressed to CS-RNTI or the uplink grant is a configured uplink grant:

5>	start or restart the ConfiguredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

- Conclusion

In this contribution, we provide our consideration on the potential impacts for the content of RAR in NR.

Proposal1: apply the above TP for capturing the restriction to the initial transmission of a TB in case that the repetition is configured for NR.

- References

- TS 38.314, “Medium Access Control (MAC) protocol specification”

- TS 38.321, “Medium Access Control (MAC) protocol specification”