3GPP TSG-RAN WG2 NR Ad hoc 1801		R2-1800902

Vancouver, Canada, 22nd-26th January 2018

Source:	vivo

Title:	Collision between dynamic grant and configured grant

Agenda Item:	10.3.1.8

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting, the issue of dynamic grant and configured grant overlapping in time domain was discussed, and the agreement is following:

Agreements

- The dynamic grant addressed to C-RNTI shall override the configured grant for this transmission in case of overlap in time domain, for type 1 and SPS.

FFS if/how we handle the URLLC case (e.g. we only allow to override the configured grant if the dynamic grant can be used for the higher priority data or if the URLLC can preempt eMBB transmission.

However, whether the agreement is also applied for URLLC case is still FFS. In this contribution, we discuss the remaining FFS in detail and give our preference.

- Discussion

To shorten the UL delay, the dynamic UL grant can be replaced by configuring periodically UL resource before UL data arrival. By doing so, steps of SR and UL grant are skipped, and a UE can directly transmit data in a pre-configured UL resource. It was agreed in the RAN1#87 meeting that at least an UL transmission scheme without grant is supported for URLLC.

In the RAN2 2017 Qingdao adhoc meeting, the following agreement was achieved:

Agreements: 1. At least numerology and TTI length are included/taken into account for restriction for LCP.

The above agreement implies URLLC service can only be transmitted using a UL grant if the numerology/TTI of the UL grant matches the service’s requirement. Short TTI and corresponding numerology is often applied to configured grant for URLLC service to meet the strict latency requirement.

To discuss the collision between dynamic grant and configured grant, we take one UE with both configured grant and dynamic grant as an example. The two types of grants use different numerology/TTI. The URLLC service of the UE is restricted to the numerology/TTI of the configured grant; others are scheduled by dynamic grant.

Figure 1 Transmission collision on dynamic grant and configured grant

The above figure shows the resources allocated to the UE.  The orange blocks are the configured grant for URLLC traffic, with 1ms period and 0.5ms TTI duration. The blue blocks are dynamic grant which are allocated according to the request from the UE for the logical channels which are not allowed to use the configured grant. At one time point, if on both of the overlapped resources, the UE has data to transmit (i.e. the blocks with grid in Figure1), transmission collision occurs. Since the network can’t predict when a UE will transmit data on configured grant, transmission collision can’t be avoided by smart scheduling at the network side. For UE can’t transmit on both grants simultaneously, it has to give up one of the transmission when collision occurs.

Option1: dynamic grant always override the configured grant

This option is very simple and aligns with LTE. However, the latency sensitive URLLC service will be delayed if the dynamic grant cannot be used for the URLLC service.

Option2: UE decides which type of resource is prioritized

When collision occurs, if UE has URLLC data which can’t use dynamic grant, it can ignore the dynamic grant; if the URLLC can use dynamic grant, dynamic grant can override the configured grant.

This option can ensure the RULLC’s QoS and requires little standardization work. But it’s very flexible and the UE’s action may out of the network control.

Option3: network indicates whether UE is allowed to decides which type of resource is prioritized

In this option, gNB can decide whether allow UE to ignore the dynamic grant in PDCCH. For example, for one UE has URLLC traffic which can use only configured grant:

- The network can indicate UE that ignoring dynamic grant is allowed. In this case, if UE has URLLC traffic to transmit in the concerned TTI, the UE uses the configured grant; otherwise, UE uses the dynamic grant.

- The network can indicate UE that ignoring dynamic grant is not allowed if the network is scheduling a dynamic grant for eMBB retransmission which is very urgent. In this case, if UE has URLLC traffic to transmit in the concerned TTI, the UE has to give up the URLLC transmission according to the network indication.

This option is more flexible than option1, and can prevent the network from losing control to UE. However, the drawback is this option is a little complex and need more standardization work than option1 and 2.

Take the above into account, we propose to adopt option2 to ensure the URLLC service’s QoS. And propose:

Proposal1: The dynamic grant addressed to C-RNTI shall override the configured grant for this transmission in case of overlap in time domain, if the dynamic grant can be used for the URLLC transmission.

Proposal2: UE is allowed to make the configured grant override dynamic grant for URLLC transmission, if the dynamic grant cannot be used for the URLLC transmission.

- Conclusion

In this contribution, we discuss the collision between configured grant and dynamic grant, and propose:

Proposal1: The dynamic grant addressed to C-RNTI shall override the configured grant for this transmission in case of overlap in time domain, if the dynamic grant can be used for the URLLC transmission.

Proposal2: UE is allowed to make the configured grant override dynamic grant for URLLC transmission, if the dynamic grant cannot be used for the URLLC transmission.