3GPP TSG-RAN WG2 Meeting AH-1801                                                                    R2-1800971

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.4.1.4.2

Source:	MediaTek Inc.

Title:	Measurement Gap Design for EN-DC

Document for:		Discussion and Decision

# 1 Introduction

In the Reno meetings, RAN2 has discussed the measurement gap design in EN-DC and reached the following agreements

RAN2 #100

Agreements:

1	For case of a single gap pattern that applies to both LTE and NR radios of the UE ('per UE gaps'): LTE RRC provides a single measurement gap configuration.

2	For the independent gap case where UE is able to apply a different gap pattern for LTE/FR1 and FR2:

a/	NR RRC configures a measurement gap configuration for FR2.

b/ 	LTE RRC configures a measurement gap configuration for LTE and NR FR1 frequencies

Agreements:

1	In the case of per UE measurement gap configuration, MN decides the configuration and informs the SN about the configuration.

2	For Dec 17, adopt a solution where:

a/ For case of a single gap case the network always configures per UE gaps if the UE is configured to measure any inter-freq or inter-RAT carrier or intra-freq cases where gaps are required.

b/ For the independent gap case the network always configures for the LTE/FR1 gaps if the UE is configured to measure any carrier within the FR1 range, and network always configures for the FR2 gaps if the UE is configured to measure any carrier within the FR2 range.

3	For the independent gap case once EN-DC is setup:

a/	the MN should inform the measurement gap pattern configuration on FR1 to the SN

b/ 	the MN should inform the SN that it wants to measure in FR2 frequency(ies). Some assistance information to the SN to configure the gaps is provided

c/ 	the SN should inform the MN that it wants to measure in NR carriers in FR1 range, if the SN has not already received a measurement gap pattern.  Some assistance information to the MN to configure the gaps is provided

FFS What assistance information is required

4	For the per UE gap case once EN-DC is setup:

a/	the MN should inform the measurement gap pattern configuration to the SN

b/ 	the SN should inform the MN that it wants to measure any inter-freq carrier or intra-freq cases where gaps are required.  Some assistance information to the MN to configure the gaps is provided

5	Capability is added to indicate support for independent gap configuration for FR1 and FR2

In this paper, we will continue the discussion of measurement gap in EN-DC base on RAN2 agreements and the new LS from RAN4 [1]. We propose the RRC signaling configuration in LTE RRC (36.331) and NR RRC (38.331) to finish the per-UE gap and per-FR gap design in EN-DC.

# 2 Discussion

RAN4 has sent a new LS to RAN2 on measurement gap design [1]. The following Table 1 for gap pattern configuration is copied from RAN4 LS. Another table from RAN4 LS about how to apply the gap patterns is provided in Annex A. Due to the table in Annex A is difficult to read, we rearrange the original table to the following Table 2 and Table 3 for easier understand and discussion.

Table 1. Measurement gap patterns configuration

| Gap Pattern Id | Measurement Gap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| FFS: [4] | [6] | [20] |
| [5] | 6 | 160 |
| FFS: [6] | [4] | [20] |
| [7] | 4 | 40 |
| [8] | 4 | 80 |
| [9] | [4] | [160] |
| [10] | 3 | 20 |
| [11] | 3 | 160 |
| FFS: [12] | [5.5] | [20] |
| [13] | [5.5] | 40 |
| [14] | [5.5] | 80 |
| [15] | [5.5] | 160 |
| [16] | [3.5] | 20 |
| [17] | [3.5] | 40 |
| [18] | [3.5] | 80 |
| [19] | [3.5] | 160 |
| [20] | [1.5] | 20 |
| [21] | [1.5] | 40 |
| [22] | [1.5] | 80 |
| [23] | [1.5] | [160] |

Table 2. Applicability for Gap Pattern Configurations (for per-UE gap)

| Per UE gap |  | target cell |  |  |
| --- | --- | --- | --- | --- |
|  |  | E-UTRA | FR1 & FR2 | E-UTRA & FR1 & FR2 |
| serving cell | E-UTRA | [0,1,2,3] | [0-11] | [0,1,2,3] |
|  | E-UTRA & FR1 | [0,1,2,3] | [0-11] | [0,1,2,3] |
|  | E-UTRA & FR2 | [0,1,2,3] | [0-11] | [0,1,2,3] |
|  | E-UTRA & FR1 & FR2 | [0,1,2,3] | [0-11] | [0,1,2,3] |

Table 3. Applicability for Gap Pattern Configurations (for Per-FR gap or independent gap)

| Per FR gap |  | target cell |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | E-UTRA | FR1 | FR2 | E-UTRA & FR1 | FR1 & FR2 | E-UTRA & FR2 | E-UTRA & FR1 & FR2 |
| serving cell | E-UTRA & FR1 | [0,1,2,3] | [0-11] | No | [0,1,2,3] | [0-11] | [0,1,2,3] | [0,1,2,3] |
|  | FR2 | No | No | [12-23] | No | [12-23] | [12-23] | [12-23] |

In LTE R14, there are 4 gap patterns (Id from 0 to 3). In EN-DC, new gap patterns should be introduced based on the LS from RAN4. It is clear from Table 2 and Table 3 that gap pattern Id [0-11] could be configured for LTE/FR1 serving cells. And RAN2 agreed that LTE RRC provide gap configuration for LTE/FR1 serving cells in EN-DC. Therefore, we propose to introduce gap pattern Id [4-11] in LTE RRC SPEC.

Proposal 1: Introduce new measurement gap patterns Id [4-11] in LTE gap configuration. Gap Id 4 and 6 should be marked as FFS for now.

From table 2 and table 3, we could also see that the new gap pattern Id [12-23] could only be configured for FR2 serving cells while independent gap is supported. For per-UE gap or FR1 serving cells, only gap pattern Id [0-11] is applicable. Therefore, we propose to introduce 2 gap configuration IE in NR RRC Signaling. The first gap configuration IE could configure gap pattern Id [0-11] for FR1 serving cells (or all serving cells in per-UE gap case) while the second IE could configure gap pattern Id [12-23] for FR2 serving cells. Furthermore, the first gap configuration IE is only used in NR SA mode since RAN2 agreed that the gap pattern of LTE/FR1 serving cells is configured by LTE RRC in EN-DC mode.

Proposal 2: Introduce 2 measurement gap configuration IE in NR RRC Signaling.

- The first IE could configure gap pattern Id [0-11] and it is used for FR1 serving cells or per-UE gap configuration in NR SA mode.

- The second IE could configure gap pattern Id [12-23] for FR2 serving cells and it is used in EN-DC or NR SA mode while the UE supports independent gap for FR1 and FR2.

- FFS: The real gap pattern Id range. Gap pattern Id 4, 6, and 12 may be removed depends on the final decision of RAN4.

In LTE, the gap offset value is specified in unit of LTE subframe length (1ms). In NR, RAN4 suggests that gap offset should be configurable with granularity based on the maximum slot length of all serving cells which have configured the gap. Furthermore, RAN4 states in 38.133 [2] that

- When measurement gaps are needed, the UE is not expected to detect SSB on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE, the switching time is 0.25ms Otherwise the switching time is 0.5ms.

Our interpretation of RAN4 SPEC is that

- For Per-UE Gap

  - Gap offset granularity for all serving cell is 0.5ms

- For Per-FR Gap

  - Gap offset granularity for FR1 serving cell is 0.5ms.

  - Gap offset granularity for FR2 serving cell is 0.25ms

Therefore, we think that the gap configuration for FR1 serving cells or per-UE gap should include a gap offset value in unit of 0.5ms. While the gap configuration for FR2 serving cells should include a gap offset value in unit of 0.25ms.

Proposal 3: Introduce a gap offset value IE with granularity of 0.5ms in the first gap configuration IE of NR RRC. Introduce a gap offset value IE with granularity of 0.25ms in the second gap configuration IE of NR RRC.

In summary, we provide the Table 4 to illustrate RRC signaling of measurement gap in different scenario. We use temporary IE name GapConfig1 and GapConfig2 in NR RRC for the proposed 2 gap configuration IEs.

Table 4. Summary of RRC Gap Signaling Configuration

| Operating mode | UE capability | serving cell(s) | RRC signalling for measurement gap pattern of the serving cell(s) | Possible gap pattern Id | Granularity of gap offset |
| --- | --- | --- | --- | --- | --- |
| EN-DC | Per-UE Gap | E-UTRA & FR1 & FR2 | measGapConfig in LTE RRC | [0-11] | 1ms |
|  | Per-FR Gap | E-UTRA &FR1 | measGapConfig in LTE RRC | [0-11] | 1ms |
|  |  | FR2 | GapConfig2 in NR RRC | [12-23] | 0.25ms |
| NR SA | Per-UE Gap | FR1 & FR2 | GapConfig1 in NR RRC | [0-11] | 0.5ms |
|  | Per-FR Gap | FR1 | GapConfig1 in NR RRC | [0-11] | 0.5ms |
|  |  | FR2 | GapConfig2 in NR RRC | [12-23] | 0.25ms |

A TP on 36.331 and a TP on 38.331 based on above proposals are provided in Annexes. We think that RAN2 could take this TP as a baseline for measurement gap procedure text and ASN.1 definition.

Proposal 4: RAN2 adopts the TP in the Annex B for 36.331 and the TP in Annex C for 38.331.

# 3 Conclusions

Base on the discussion in section 2, we propose the following:

Proposal 1: Introduce new measurement gap patterns Id [4-11] in LTE gap configuration. Gap Id 4 and 6 should be marked as FFS for now.

Proposal 2: Introduce 2 measurement gap configuration IE in NR RRC Signaling.

- The first IE could configure gap pattern Id [0-11] and it is used for FR1 serving cells or per-UE gap configuration in NR SA mode.

- The second IE could configure gap pattern Id [12-23] for FR2 serving cells and it is used in EN-DC or NR SA mode while the UE supports independent gap for FR1 and FR2.

- FFS: The real gap pattern Id range. Gap pattern Id 4, 6, and 12 may be removed depends on the final decision of RAN4.

Proposal 3: Introduce a gap offset value IE with granularity of 0.5ms in the first gap configuration IE of NR RRC. Introduce a gap offset value IE with granularity of 0.25ms in the second gap configuration IE of NR RRC.

Proposal 4: RAN2 adopts the TP in the Annex B for 36.331 and the TP in Annex C for 38.331.

# 4 References

[1] R4-1713938, “LS on measurement gap”, RAN4

[2] TS 38.133 v1.0.0

# Annex A: Applicability for Gap Pattern configuration from R4

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE

| Measurement gap pattern configuration | Serving cell | Measurement Purpose | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE measurement gap | E-UTRA + FR1, or E-UTRA + FR2, or E-UTRA + FR1 + FR2 | E-UTRA only | [0,1,2,3] |
|  |  | FR1 and/or FR2 | [0-11] |
|  |  | E-UTRAN and FR1 and/or FR2 | [0,1,2,3] |
| Per FR measurement gap | E-UTRA and, FR1 if configured | E-UTRA only | [0,1,2,3] |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only | [0-11] |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only | No gap |
|  | FR2 if configured |  | [12-23] |
|  | E-UTRA and, FR1 if configured | E-UTRA and FR1 | [0,1,2,3] |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 | [0-11] |
|  | FR2 if configured |  | [12-23] |
|  | E-UTRA and, FR1 if configured | E-UTRA and FR2 | [0,1,2,3] |
|  | FR2 if configured |  | [12-23] |
|  | E-UTRA and, FR1 if configured | E-UTRA and FR1 and FR2 | [0,1,2,3] |
|  | FR2 if configured |  | [12-23] |
| Editor notes: this applicability table might need revise according to any change made on Table 9.1.2-1: Gap Pattern Configurations supported by the UE |  |  |  |

# Annex B: TP for 36.331

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if measGapConfig is set to setup:

2>	if a measurement gap configuration measGapConfig or measGapConfigPerCC-List is already setup, release the measurement gap configuration;

2>	if the gapOffset in measGapConfig indicates a non-uniform gap pattern:

3>	setup the measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of the first gap of each non-uniform gap pattern occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = LMGRP/10 as defined in TS 36.133 [16];

2>	else:

3>	setup the measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 36.133 [16];

NOTE 1:	The UE applies a single gap, which timing is relative to the MCG cells, even when configured with DC.

1>	else if measGapConfig is set to release:

2>	release the measurement gap configuration measGapConfig;

1>	if measGapConfigPerCC-List is set to setup:

2>	if a measurement gap configuration measGapConfig is already setup, release measGapConfig;

2>	if measGapConfigToRemoveList is included:

3>	for each ServCellIndex included in the measGapConfigToRemoveList:

4>	release measGapConfigCC for the serving cell indicated by servCellId;

2>	if measGapConfigToAddModList is included:

3>	for each ServCellIndex included in the measGapConfigToAddModList:

4>	store measGapConfigCC for the serving cell indicated by servCellId;

2>	for each serving cell with stored measGapConfigCC indicating a non-uniform gap pattern, setup the measurement gap configuration indicated by the measGapConfigCC in accordance with the received gapOffset, i.e., the first subframe of the first gap of each non-uniform gap pattern occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = LMGRP/10 as defined in TS 36.133 [16];

2>	for each serving cell with stored measGapConfigCC not indicating a non-uniform gap pattern, setup the measurement gap configuration indicated by the measGapConfigCC in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells):

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 36.133 [16];

NOTE 2:	The UE applies gap timing relative to the MCG cells, even when configured with DC.

1>	else (measGapConfigPerCC-List is set to release):

2>	release the measurement gap configuration measGapConfigPerCC-List;

NOTE 3:	When a SCell is released, the UE is not required to apply a per CC measurement gap configuration associated to the SCell.

NOTE 4: In EN-DC, the UE applies a single gap pattern for all MCG serving cell(s), which timing is relative to the MCG cells. The UE may apply the same gap pattern for some NR serving cell(s) according to TS 38.331 [X2 5.5.2.9].

/******************************** Skip unchanged sections ****************************************/

#### MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration and controls setup/ release of measurement gaps.

MeasGapConfig information element

-- ASN1START

MeasGapConfig ::=					CHOICE {

release								NULL,

setup								SEQUENCE {

gapOffset							CHOICE {

gp0									INTEGER (0..39),

gp1									INTEGER (0..79),

...,

gp2-r14								INTEGER (0..39),

gp3-r14								INTEGER (0..79),

gp-ncsg1-r14						INTEGER (0..39),

gp-ncsg2-r14						INTEGER (0..79),

gp-ncsg3-r14						INTEGER (0..39),

gp-ncsg4-r14						INTEGER (0..79),

gp-nonUniform1-r14					INTEGER (0..1279),

gp-nonUniform2-r14					INTEGER (0..2559),

gp-nonUniform3-r14					INTEGER (0..5119),

gp-nonUniform4-r14					INTEGER (0..10239),

gp4-r15 							INTEGER (0..19),  -- FFS

gp5-r15 							INTEGER (0..159),

gp6-r15 							INTEGER (0..19),  -- FFS

gp7-r15 							INTEGER (0..39),

gp8-r15 							INTEGER (0..79),

gp9-r15 							INTEGER (0..159)

gp10-r15 							INTEGER (0..19),

gp11-r15 							INTEGER (0..159)

}

}

}

Editor Note: Gap pattern Id 4 and 6 are still FFS on R4 SPEC. Some of them may be removed depends on the final decision of R4. In case of some pattern id is removed, there will be a shift in Id numbers of original gap patterns.

-- ASN1STOP

MeasGapConfig field descriptions

gapOffset Value gapOffset of gp0 corresponds to gap offset of Gap Pattern Id “0” with MGRP = 40ms and MGL = 6ms, gapOffset of gp1 corresponds to gap offset of Gap Pattern Id “1” with MGRP = 80ms and MGL = 6ms, gapOffset of gp2 corresponds to gap offset of Gap Pattern Id “2” with MGRP = 40ms and MGL = 3ms, gapOffset of gp3 Gap Pattern Id “3” with MGRP = 80ms and MGL = 3ms, gapOffset of gp4 corresponds to gap offset of Gap Pattern Id “4” with MGRP = 20ms and MGL = 6ms, gapOffset of gp5 corresponds to gap offset of Gap Pattern Id “5” with MGRP = 160ms and MGK = 6ms, gapOffset of gp6 corresponds to gap offset of Gap Pattern Id “6” with MGRP = 20ms and MGL = 4ms, gapOffset of gp7 corresponds to gap offset of Gap Pattern Id “7” with MGRP = 40ms and MGL = 4ms, gapOffset of gp8 corresponds to gap offset of Gap Pattern Id “8” with MGRP = 80ms and MGL = 4ms, gapOffset of gp9 corresponds to gap offset of Gap Pattern Id “9” with MGRP = 160ms and MGL = 4ms, gapOffset of gp10 corresponds to gap offset of Gap Pattern Id “10” with MGRP = 20ms and MGL = 3ms, gapOffset of gp6 corresponds to gap offset of Gap Pattern Id “6” with MGRP = 160ms and MGL = 3ms, gapOffset of gp-ncsg1 corresponds to gap offset of NCSG Pattern Id “1” with VIRP = 40ms and ML = 4ms, gapOffset of gp-ncsg2 corresponds to gap offset of of NCSG Pattern Id “2” with VIRP = 80ms and ML = 4ms, gapOffset of gp-ncsg3 corresponds to gap offset of NCSG Pattern Id “3” with VIRP = 40ms and ML = 3ms, gapOffset of gp-ncsg4 corresponds to gap offset of of NCSG Pattern Id “4” with VIRP = 80ms and ML =3ms. gapOffset of gp-nonUniform1 corresponds to gap offset of non uniform gap pattern Id “1” with LMGRP = 1280ms, gapOffset of gp-nonUniform2 corresponds to gap offset of non uniform gap pattern Id “2” with LMGRP = 2560ms, gapOffset of gp-nonUniform3 corresponds to gap offset of non uniform gap pattern Id “3” with LMGRP = 5120ms, gapOffset of gp-nonUniform4 corresponds to gap offset of non uniform gap pattern Id “4” with LMGRP = 10240ms. Also used to specify the measurement gap pattern to be applied, as defined in TS 36.133 [16]. E-UTRAN configure gap pattern 4 to 11 to a UE only if the UE is configured to measure one or more NR measurement object(s). Editor Note: Gap pattern Id 4 and 6 are still FFS on R4 SPEC. Some of them may be removed depends on the final decision of R4. In case of some pattern id is remove, there will be a shift in Id numbers of original gap patterns. For Gap Patterns (including non-uniform gap patterns, but excluding NCSG patterns), E-UTRAN includes the same gapOffset value (gap pattern id and gap offset) for all serving cells that are configured with a Gap Pattern. For NCSG Patterns, E-UTRAN includes gapOffset value indicating VIRP and gap offset consistent with the Gap Pattern configuration (MGRP and gap offset).

servCellId Identifies the serving cell for which measurement gap configuration is provided (setup) or deleted (release).

# Annex C: TP for 38.331

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BLER	Block Error Rate

CCCH	Common Control Channel

CMAS	Commercial Mobile Alert Service

CP	Control Plane

C-RNTI	Cell RNTI

DCCH	Dedicated Control Channel

DL	Downlink

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

EHPLMN	Equivalent Home Public Land Mobile Network

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FR1	Frequency Range 1

FR2	Frequency Range 2

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

IE	Information element

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

MAC	Medium Access Control

MCG	Master Cell Group

MIB	Master Information Block

N/A	Not Applicable

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PTAG	Primary Timing Advance Group

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RNTI	Radio Network Temporary Identifier

ROHC	RObust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

SCell	Secondary Cell

SCG	Secondary Cell Group

SI	System Information

SIB	System Information Block

SRB	Signalling Radio Bearer

STAG	Secondary Timing Advance Group

S-TMSI	SAE Temporary Mobile Station Identifier

TM	Transparent Mode

UE	User Equipment

UICC	Universal Integrated Circuit Card

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

UTC	Coordinated Universal Time

UTRAN	Universal Terrestrial Radio Access Network

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

/******************************** Skip unchanged sections ****************************************/

#### 5.5.2.9	Measurement gap configuration

The UE shall:

1>	if the UE supports independent gap for FR1 and FR2:

2> if the UE is operating in EN-DC mode:

3>	for the serving cell(s) belong to FR1 according to the definition in 38.104[12], the UE applies the same measurement gap pattern configured for MCG cell(s).

2>	else:

3>	if GapConfig1 is present:

4>	if a measurement gap configuration GapConfig1 is already setup, release the measurement gap configuration;

4>	for the serving cell(s) belong to FR1 according to the definition in 38.104[12], setup the measurement gap configuration indicated by the gapPattern and gapOffset1 in GapConfig1.

Editor Note: FFS how to specify the slot that a gap occurs using gapOffset, gapPattern, and SFN

3>	else:

4>	release the measurement gap configuration GapConfig1;

2>	if GapConfig2 is present:

3>	if a measurement gap configuration GapConfig2 is already setup, release the measurement gap configuration;

3>	for the serving cell(s) belong to FR2 according to the definition in 38.104[12], setup the measurement gap configuration indicated by the gapPattern and gapOffset in GapConfig2.

Editor Note: FFS how to specify the slot that a gap occurs using gapOffset, gapPattern, and SFN

2>	else:

3>	release the measurement gap configuration GapConfig2;

1>	else:

2>	if the UE is operating in EN-DC mode:

3>		for all serving cell(s), the UE applies the same measurement gap pattern configured for MCG cell(s).

2>	else:

3>	if GapConfig1 is present:

4>	if a measurement gap configuration GapConfig1 is already setup, release the measurement gap configuration;

4>	for all serving cell(s), setup the measurement gap configuration indicated by the gapPattern and gapOffset1 in GapConfig1.

Editor Note: FFS how to specify the slot that a gap occurs using gapOffset, gapPattern, and SFN

3>	else:

4>	release the measurement gap configuration GapConfig1;

/******************************** Skip unchanged sections ****************************************/

#### –	MeasGapConfig

The IE MeasGapConfig specifies the measurement gap configuration.

MeasGapConfig information element

-- ASN1START

MeasGapConfig ::=	SEQUENCE	{

GapConfig1			SEQUENCE	{

gapPattern	ENUMERATED {gp0, gp1, gp2, gp3, gp4, gp5, gp6, gp7, gp8, gp9, gp10, gp11}

gapOffset1	INTEGER (0..639)

...

} 		OPTIONAL	-- Need R

GapConfig2			SEQUENCE	{

gapPattern	ENUMERATED {gp12, gp13, gp14, gp15, gp16, gp17, gp18, gp19, gp20, gp21, gp22, gp23}

gapOffset2	INTEGER (0..319)

...

}		OPTIONAL	-- Need R

...

}

Editor Note: Gap pattern Id 4, 6, and 12 are still FFS on R4 SPEC. Some of them may be removed depends on the final decision of R4. In case of some pattern id is removed, there will be a shift in Id numbers of original gap patterns.

-- ASN1STOP

MeasGapConfig field descriptions

GapConfig1 The measurement gap configuration for all serving cell(s) if GapConfig2 is not present. If GapConfig2 is present, UE applies this measurement gap configuration for the NR serving cells that belongs to FR1. Network does not include this IE if UE is operating in EN-DC mode.

GapConfig2 If presents, UE applies this measurement gap configuration (including no measurement gap) for the NR serving cells that belongs to FR2. Network only include this filed if UE supports independent gap for FR1 and FR2.

gapPattern Value gp0 corresponds to the Gap Pattern Id “0” defined in 38.133[X2], Value gp0 corresponds to the Gap Pattern Id “1” defined in 38.133[X2], and so on. The corresponding MGRP and MGL value for a gap pattern Id is also defined in 38.133 [X2].

gapOffset1 The measurement gap offset value in unit of 0.5ms. Network insure that the offset value is not larger than the MRGP value of corresponding gap pattern.

gapOffset2 The measurement gap offset value in unit of 0.25ms. Network insure that the offset value is not larger than MRGP value of corresponding gap pattern.