3GPP TSG-RAN WG2 NR AH1801 	R2-1801024

Vancouver, Canada, 22nd – 26th Jan, 2018

Agenda Item:	11.1

Source:	Ericsson

Title:	Duplexing in IAB

Document for:	Discussion, Decision

# Introduction

The study item on integrated access and backhaul aims at supporting NR cells, which are self-backhauled using the NR radio interface to other NR nodes.

Following requirements are mentioned in the study item description:

•	Efficient and flexible operation for both in-band and out-band relaying in indoor and outdoor scenarios

•	Multi-hop and redundant connectivity

•	Support of legacy NR UEs

The aim of this contribution is to identify some of the key questions in radio resource sharing in multi-hop connections with legacy support for NR UEs for further discussion.

# Terminology

In this contribution the following terminology is used:

- IAB donor node (IAB-DN): The gNB or part of the gNB, that is using NR to backhaul other NR nodes.

- IAB node (IAB-N): The NR node being backhauled using NR radio to another NR node (either an IAB node, or an IAB donor node)

- IAB backhaul link: The NR link between the IAB node and the other IAB nodes or IAB donor node providing backhaul.

- Access link: The link between a UE and a IAB donor node (in case UE is being directly served by it without an intermediary IAB-N).

- IAB access link: The link between the IAB node and the UEs.

- Inband/Outband: Inband means that the same carrier frequency is used both for the IAB-backhaul link and the IAB-access link. Outband means that the IAB-backhaul link and the IAB-access link uses different carrier frequency.

- Data traffic directed towards an IAB-DN is considered uplink (UL) traffic; data traffic directed towards NR UEs is considered downlink (DL) traffic.

# Discussion

It is important to have the transmitter and receiver of a wireless transceiver well separated. If not separated in space, this is usually achieved by synchronized transmission and reception in frequency or time domain (FDD or TDD).

## NR-IAB with Out-Band Relaying

In NR-IAB out-band relaying, the access and the backhaul link operate in different carrier frequencies. Thus, separation of access and backhaul domains can be achieved by the same means as Tx and Rx in FDD systems. If radios for access and backhaul are not integrated, physical separation can also be used.
Access and backhaul links can be operated independently and multi-hop connectivity and support of legacy NR UEs are rather independent issues.

In out-band IAB, transceivers in the backhaul and access can be isolated by spatial separation or frequency filters. Access and backhaul links can be operated independently.

## NR-IAB with In-Band Relaying

In NR-IAB with in-band relaying, access and backhaul links use the same carrier frequencies and thus share the same radio resources. Access operation has effects on backhaul operation and vice versa.

### FDD Operation in IAB Backhaul Domain

Figure 1 shows FDD operation in backhaul and access links without any specific coordination/scheduling in time domain. As indicated by red arrows, there is potentially significant interference from near-far effects between backhaul links and backhaul-access links.

Figure 1: FDD Operation in IAB; transmission on uplink carrier is indicated by blue, transmission on downlink carrier by green, and interference between different links by red arrows.

Separation of access and backhaul links by proper frequency domain resource scheduling is not effective, since frequency separation only exist in the digital domain and near-far effects would still exists in the analogue section of antenna ports.

Separation of access and backhaul links can be achieved by non-overlapping time domain scheduling, as depicted in Figure 2. Timeslots in UL/DL scheduled for transmission on IAB access links must not be used for IAB backhaul. However, for IAB-Ns, DL capacity decreases to min(t1,t2) (%), UL capacity decreases to min(t3,t4) (%) and interference is still present in between backhaul links. Thus, system capacity in IAB is reduced; and UEs directly connecting to the IAB-DN can be unaffected.

Figure 2: FDD Operation in IAB with separation of access and backhaul transmission by non-overlapping time domain scheduling.

In order to further reduce the potential of interference between backhaul links, further slicing in time domain (for example between transmission IAB N2-N1 and N1-DN in Figure. 2) can be introduced, but will also result in further reduction of backhauling capacity. Alternatively, one could assign available carrier frequencies to UL/DL in a relay-node specific manner, as shown in Figure. 3 with the UL/DL frequencies between IAB-N1 and IAB-N2 interchanged.

Figure 3: FDD Operation in IAB with separation of access and backhaul transmission by non-overlapping time domain scheduling and relay-node specific DL/UL frequency bands.

Radio systems operated in FDD have a transmitter operating only on a specific set of carrier frequencies that is different from any receiver carrier frequency. Figure 3 indicates an architecture in which IAB-N1 appears as UE to IAB-DN and UE1 appears as UE to IAB-N1. In in-band relay, it is possible that UE1 must connect to the same transceiver of IAB-N1 that also connects to IAB-DN, as illustrated in Figure. 4. But this results in a transceiver that has apparent UL and DL carriers in the same radio transmitter; the same holds for the receiver part. The reason is that the transmission from the same transmitter is uplink to IAB-DN and downlink to UE1.

Figure 4: FDD Operation in IAB with separation of access and backhaul transmission by non-overlapping time domain scheduling relay-node specific DL/UL frequency bands.

If, in IAB-Ns, uplink transmissions in backhaul and the downlink transmissions in access are scheduled at overlapping time slots (e.g. t3 = t2 in Figure 3), each in-band relay node needs to implement multiple transmitters and receivers (one for each UL and DL carrier) in order to support legacy UEs. Complexity in IAB-Ns will be twice as high as in IAB-DNs.

If transmission in access and backhaul domain is scheduled in mutually different time slots (e.g. t3 ≠ t2 in Figure 3), each in-band relay node still needs to implement multiple transmitters and receivers (one for each UL and DL carrier) in order to support legacy UEs operating in FDD; not all transmitters (and receivers) are used at the same time. Complexity in IAB-Ns will still be significantly higher than in IAB-DNs.

Complexity in IAB nodes operated in FDD is significantly higher than in IAB donor nodes.

UE1 in Figure 4 can be stationary or mobile. UE1 could also stand for another relay-node that is on the same layer in a multi-hop star-architecture. UE1 could also be just another relay that connects to IAB-N1 and IAB-DN at the same time in case of an overall mesh-like architecture for multi-connectivity. In this case, the notion of UL and DL depends on the actual routing that can change during operation (as in case of blocking events) and UL and DL carrier in FDD cannot be uniquely identified anymore.

If t1, t2, t3 and t4 would be mutually non-overlapping timeslots and if all UL and DL carrier frequencies would be identical, the system in Figure 3 would become equivalent to a TDD system with the same system capacity, but only one transmitter/receiver implementation.

IAB nodes operating in FDD and preventing self-interference can become equivalent to TDD operated systems.

### TDD Operation in IAB Backhaul Domain

Figure 5 shows standard TDD operation in backhaul and access links; uplink is transmitted in timeslots t1 and downlink is transmitted in timeslots t2. As indicated by red arrows, there is potentially significant interference from near-far effects between backhaul links and backhaul-access links. Separation of access and backhaul links by proper frequency domain resource scheduling is not effective, since frequency separation only exist in the digital domain and near-far effects would still exists in the analogue section of antenna ports.

Figure 5: TDD Operation in IAB; uplink transmission is indicated by blue, downlink transmission by green, and interference between different links by red arrows.

If the UL/DL timeslots would be defined per IAB-N, interference between backhaul links and backhaul-access links could be completely avoided. However, it also results in different UL/DL timing of UEs connected to different IAB-Ns. UEs would not see this, but UEs that are in close proximity like on cell edges can create severe mutual interference.

Figure 6: TDD Operation in IAB with separation of access and backhaul transmission by adjusting UL/DL timing specific per IAB-N.

A solution to that could be to keep all UEs in synchronized UL and DL mode and use dedicated time slots for the UL/DL transmission over backhaul links. This is illustrated in Figure 7, where UEs receive and transmit on time slots t2 and t1 of reduced extend, and backhaul transmission is scheduled in timeslots t3 and t4. All timeslots t1, t2, t3, t4 must not overlap.

Figure 7: TDD Operation in IAB with separation of access and backhaul transmission by non-overlapping time domain scheduling of UL/DL on access and backhaul links.

Since the radio of IAB-Ns that connects to other IAB-Ns or IAB-DN does not use timeslots t1 and t2 for backhaul transmission, UEs can be served from the same transceiver without being in the paradox situation that UL (in backhaul) and DL (in access) transmission would apparently been scheduled at the same time.

In in-band IAB systems, TDD operation should be prioritized over FDD.

# Conclusion

In section 3 we made the following observations:

Observation 1	In out-band IAB, transceivers in the backhaul and access can be isolated by spatial separation or frequency filters. Access and backhaul links can be operated independently.

Observation 2	Complexity in IAB nodes operated in FDD is significantly higher than in IAB donor nodes.

Observation 3	IAB nodes operating in FDD and preventing self-interference can become equivalent to TDD operated systems.

Based on the discussion in section 3 we propose the following:

Proposal 1	In in-band IAB systems, TDD operation should be prioritized over FDD.