3GPP TSG-RAN2 Meeting AH#1	R2-1801090

# Vancouver, Canada, 22nd – 26th January 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | yyyy | rev | - | Current version: | 2.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on generating BSR MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2017-12-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2 #100 meeting, it has noted: =>for triggering an SR when there is already a grant, use as a baseline what is in the current TS “for immediate transmission” however companies can study until next meeting if there are additional changes needed. In current text for generating BSR MAC CE “1> if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled: 2> if UL-SCH resources are available for a new immediate transmission:” We think the “a new immediate transmission” is not clear for UE behaviour on when to generate the BSR MAC CE. When at a time point the MAC determine that there is at least one BSR has been triggered, the conditons which should be met for generating the BSR MAC are: - There is UL-SCH resources available, the resources could mean: - The resources is already assigned by PDCCH which received before this time point; or - The PDCCH indicating UL grant in the near future - At this time point, the MAC entity is able to generate the BSR MAC CE, meaning there is enough time duration for the UE to do LCP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “if UL-SCH resources are available for a new immediate transmission, and at this time point the MAC entity is able to generate the BSR MAC CE(s):” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test may not pass when testing the BSR MAC CE generating procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.4.5	Buffer Status Reporting

…

The MAC entity shall:

1>	if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new immediate transmission, and at this time point the MAC entity is able to generate the BSR MAC CE(s):

3>	instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s);

3>	start or restart periodicBSR-Timer except when all the generated BSRs are long or short Truncated BSRs;

3>	start or restart retxBSR-Timer.

2>	else if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running:

3>	if an uplink grant is not a configured grant; or

3>	if the Regular BSR was not triggered for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers:

4>	trigger a Scheduling Request.

CHANGE END