3GPP TSG-RAN WG2 AH1801 	R2-1801229

# Vancouver, Canada, 22th Jan – 26th Jan 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | CRNum | rev | - | Current version: | 15.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SCG failure handling for EN-DC in 36.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-01-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To enable the sending of the SCGFailureInformation message to the MN and recover from SCG failure, in the case where SCG is the only configured path for SRB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After the SCGFailureInformation message is prepared, if the SCG is the only configured path for SRB1, the UE changes the configured path to the MCG. Then the SCGFailureInformation message is sent via the MCG leg. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will be stuck in SCG failed state, as the SCGFailureInformation can not be sent to the MN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.6.13.3	Actions related to transmission of SCGFailureInformation message

The UE shall set the contents of the SCGFailureInformation message as follows:

1>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG failure information for an E-UTRA SCG:

2>	if the UE initiates transmission of the SCGFailureInformation message to provide SCG radio link failure information:

3>	include failureType and set it to the trigger for detecting SCG radio link failure;

2>	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG change failure information:

3>	include failureType and set it to scg-ChangeFailure;

2>	else if the UE initiates transmission of the SCGFailureInformation message due to exceeding maximum uplink transmission timing difference:

3>	include failureType and set it to maxUL-TimingDiff;

2>	set the measResultServFreqList to include for each E-UTRA SCG cell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [16];

2>	for each E-UTRA SCG serving frequency included in measResultServFreqList, include within measResultBestNeighCell the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

2>	set the measResultNeighCells to include the best measured cells on non-serving E-UTRA frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	if the UE was configured to perform measurements for one or more non-serving EUTRA frequencies and measurement results are available, include the measResultListEUTRA;

3>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

1>	else if the UE initiates transmission of the SCGFailureInformation message to provide SCG failure information for an NR SCG:

2>	include failureType within failureReportSCG-NR and set it to indicate the SCG failure in accordance with TS 38.331 [82, 5.7.3.3];

2>	include and set measResultSCG in accordance with TS 38.331 [82, 5.7.3.4]:

2>	for each NR frequency the UE is configured to measure by measConfig for which measurement results are available:

3>	set the measResultFreqListNR to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

1>	If SRB1 is configured with pdcp-config:

2 > if the primaryPath is associated with the SCG, ul-DataSplitThreshold is set to release, and ul-duplication is set to FALSE:

3> the UE shall change the primaryPath to the MCG;

NOTE 3:	Field measResultSCG is used to report available results for NR frequencies the UE is configured to measure by NR RRC signalling.

The UE shall submit the SCGFailureInformation message to lower layers for transmission.

END OF CHANGES