TSG-RAN WG2 NR Ad hoc 1801	R2-1801421

Vancouver, Canada, 22nd January – 26th January 2018

Agenda Item: 	10.4.2.3

Source:	Apple

Title:	RRC configuration for Single Tx

Document for:		Discussion

# 1	Introduction

According to the previous RAN2 discussion on Single Tx, Single Tx capability and related configuration (i.e. case 1) are introduced to support UE operating in difficult band combinations. While, in RAN1#91 and RAN#78 meetings, the usage of Single Tx is extended for the scenario where UE does not support dynamic LTE-NR power sharing. Thus, corresponding extension on UE capability and configuration of Single Tx should also be considered.

# 2	Discussion

Following is the agreement on the EN-DC dynamic power sharing functionality in RAN#78 [1].

- Proposal 1 - Agree to introduce Rel-15 capability signaling to indicate whether the UE supports dynamic EN-DC power sharing - Dynamic power sharing means that the UE can operate with P_LTE + P_NR > P_powerclass configuration - Agree that the intent is to make dynamic EN-DC power sharing mandatory at a future time - Check any possible updates on status above in March - Proposal 2 - For UEs without dynamic LTE-NR power sharing capability, the support of single UL operation (Operation A with Case 1 in Slide 5) is mandatory with capability signalling - Single UL operation is optional for dynamic power sharing capable UEs

- Agreement: (RAN1#91) - For LTE/NR NSA operation, - If this UE supports dual UL operation and also supports single UL operation with Case 1 HARQ timing, RRC signaling can configure a UE to operate in one of the following modes: - Dual UL operation - Single UL operation with Case 1 HARQ timing - Single UL operation with Case 2 HARQ timing - For UE supporting single UL operation and with Case 1 HARQ timing if UE does not support power scaling for LTE-NR DC with P_LTE+P_NR>Pcmax, UE shall support the following two operations: - Operation A with Case1: P_LTE + P_NR > Pcmax, in which case the UE assumes that no NR UL transmission takes place in an UL subframe/slot that is designated as LTE UL in the Case 1 reference TDD configuration - Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified) - The operation A vs operation B configuration is implicitly determined based on P_LTE and P_NR - Note that the above agreement does not affect the current status on the optional/mandatory support of power scaling for LTE-NR DC with P_LTE + P_NR > Pcmax - Note that the above agreement can become obsolete if power scaling for LTE-NR DC is mandated to all UEs

Following observations can be derived based on these agreements.

Observation 1: For UE without EN-DC dynamic power sharing capability, Single Tx capability should be always supported, no matter UE is working on difficult or non-difficult band combination.

Observation 2: gNB should be able to configure three UL operations, i.e. dual Tx, single Tx with case 1 and single Tx with case 2 by RRC signaling.

According to observation 1, currently Single Tx capability is only applicable for the difficult band combinations. Due to the extension of single Tx to UE without dynamic power sharing capability, Single Tx capability should be allowed to reflect the UE’s capability for all EN-DC band combinations rather than difficult band combinations only. In order to illustrate the linkage between dynamic power sharing and single Tx capability, there are two methods:

- Method-1: Describe the linkage in the field description of UE EN-DC dynamic power sharing capability. And Single Tx capability remains as previously agreed only applicable for the difficult band combinations due to IMD issues.

- Method-2: Extend the usage of the Single Tx capability to indicate that Single Tx capability is mandatory supported when UE does not support LTE-NR dynamic sharing. In this case, UE may set the bit as 1 to all the band combinations since LTE-NR dynamic sharing is a UE specific rather than a band combination specific capability.

Comparing the two methods, method-1 may cause conflict in two capabilities, while method-2 has no such risk. Therefore, we prefer adopting method-2 to make a clear spec.

Proposal 1: Single Tx capability should be applicable to all EN-DC band combinations when UE does not support dynamic LTE-NR power sharing.

Proposal 1a: For the UE without dynamic power sharing capability, the Single Tx field could be set to 1 in all the EN-DC band combinations.

In addition, upon receiving UE capability indicating that Single Tx is required, current situation of UE uplink mode configuration is as following:

- For Single Tx case 1, NW can configure the TDM pattern to UE, and with this configuration UE can enter the single Tx case 1 operation.

- For Single Tx case 2, currently there is no explicit RRC signaling to configure UE to operate on Single Tx.

To reflect RAN1 agreement that RRC signaling can configure a UE operating in Single UL Operation with Case 2 HARQ timing, it is suggested that RAN2 kindly consider introducing an explicit configuration, e.g. 1-bit indication in RRCConnectionReconfiguration message for Single Tx mode. With this, both UE and NW would have a clear understanding on when to start operating in Single Tx.

Proposal 2: It is proposed to introduce an explicit Single Tx mode configuration for Single Tx case 2, i.e. 1-bit indication in RRCConnectionReconfiguration.

# 3	Conclusion

According to the anlaysis in section 2, it is proposed that

Proposal 1: Single Tx capability should be applicable to all EN-DC band combinations when UE does not support dynamic LTE-NR power sharing.

Proposal 1a: For the UE without dynamic power sharing capability, the Single Tx field could be set to 1 in all the EN-DC band combinations.

Proposal 2: It is proposed to introduce an explicit Single Tx mode configuration for Single Tx case 2, i.e. 1-bit indication in RRCConnectionReconfiguration.

# 4	Reference

- RP-172833, Way Forward on NR EN-DC power sharing capability	 Qualcomm Incorporated