3GPP TSG-RAN WG2 NR AH#3			     R2-1801455

Vancouver, Canada, 22nd – 26th January, 2018        (Resubmission of R2-1713667)

Agenda item:		10.3.4.4 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on SDAP entity establishment in DC

Document for:	Discussion and Decision

1.	Introduction

SDAP specification (TS 37.324) has included the sentence “There is an SDAP entity configured per cell group for each individual PDU session.” which should be confirmed by RAN2. First, RAN2 have to discuss if it is necessary for UE to have several SDAP entities for a PDU session depending on the number of the configured cell groups. Besides, RAN2 may need to discuss whether a default DRB should be established for each SDAP entity of the PDU session.

4.2.2	SDAP entities

The SDAP entities are located in the SDAP sublayer. Several SDAP entities may be defined for a UE. There is an SDAP entity configured per cell group for each individual PDU session.

Editor’s Note: RAN2 should confirm if an SDAP entity on the network side has a corresponding peer entity on the UE side.

This document discuss on SDAP entity establishment on the UE side in DC.

2.	Discussion

As mentioned above, UE can have several SDAP entities for a PDU session. In the case of DC, one SDAP entity is established by MN, and another SDAP entity is established by SN. The following additional operations on the UE side are always needed because there is more than one SDAP entity for the PDU session.

- When the MN decides to move a DRB to the SN, the UE should disassociate the DRB and all QoS flow(s) of the DRB from SDAP entity corresponding to the MN, and then should associate them with SDAP entity corresponding to the SN.

- When the MN decides to move a QoS flow to the SN, the UE should disassociate the QoS flow from SDAP entity corresponding to the MN, and then should associate it with SDAP entity corresponding to the SN.

- For every UL transmission, the UE should select one out of two SDAP entities depending on a UL packet’s QoS flow. The UL packet is submitted to the selected SDAP entity.

We think that it is better any protocol layers above PDCP are not affected by the fact whether or not cell group is configured.

Proposal 1:	Only one SDAP entity for a PDU session should be established on the UE side.

If this proposal 1 is not accepted, RAN2 need to discuss on default DRB associated with each SDAP entity for the PDU session. We wonder if the SN always needs to establish the default DRB. When some QoS flows of the PDU session are moved to the SN, the SN can explicitly map all the offloaded QoS flows to DRB(s) in order to indicate to the UE which QoS flows are offloaded. In this case, the SN may not establish the default DRB. Considering this case, the UE may have just one default DRB in the DC where different QoS flows belonging to the same PDU session are mapped to both MN and SN.

5.1.1	SDAP entity establishment

When RRC [3] requests an SDAP entity establishment for a PDU session, the UE shall:

- establish an SDAP entity for the PDU session;

- associate the SDAP entity with a default DRB.

Proposal 2:	If RAN2 consider there is more than one SDAP entity for a PDU session, when an SDAP entity in a CG for the PDU session is established in DC, default DRB for the SDAP entity may not be established.

We also propose to adopt our text proposal which is shown in the section 4.

3.	Proposal

In this document, we present our view on default DRB establishment in DC, and propose following:

Proposal 1:	Only one SDAP entity for a PDU session should be established on the UE side.

Proposal 2:	If RAN2 consider there is more than one SDAP entity for a PDU session, when an SDAP entity in a CG for the PDU session is established in DC, default DRB for the SDAP entity may not be established.

4.	Text Proposal

TP for Proposal 1

4.2.2	SDAP entities

The SDAP entities are located in the SDAP sublayer. Several SDAP entities may be defined for a UE. There is an SDAP entity for each individual PDU session.

TP for Proposal 2

5.1.1	SDAP entity establishment

When RRC [3] requests an SDAP entity establishment for a PDU session, the UE shall:

- establish an SDAP entity for the PDU session;

- associate the SDAP entity with a default DRB, if the default DRB is configured.