3GPP TSG-RAN WG2 NR AH#3			     R2-1801458

Vancouver, Canada, 22nd – 26th January, 2018              (Revision of R2-1713659)

Agenda item:	   10.3.4.4 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 		Configurability for the presence of SDAP header

Document for:	Discussion and Decision

1.	Introduction

RAN2 discussed configurability for the presence of SDAP header at the RAN2#97bis and RAN2-AH2 meetings, and made following agreements.

RAN2 #97bis 1 AS layer header include the UL “Flow ID” depending on network configuration. RAN2 AH2 1 Whether a SDAP header is present or not is configured by RRC per DRB.

This document introduces some problems of this network configuration.

2.	Discussion

For uplink traffic, UE has to attach an SDAP header including QoS flow ID to UL packet so that gNB can determine the QoS marking which is carried in the N3 encapsulation header of the UL packet. If the mapping relationship between QoS flow and DRB is one to one, it is redundant to attach the SDAP header to the UL packet. In this case, it is possible to configure SDAP parameter sdap-Header-UL not to attach the SDAP header to UL packets to be sent via the DRB. However, if such configuration can be changed at any time, it may cause transmission of incorrect payload data.

- CASE #1: sdap-Header-UL value is changed from absent to present

As shown in Figure 1, gNB can reconfigure the sdap-Header-UL for DRB A so that UE attaches SDAP header to UL packets to be transmitted through the DRB A. At that time (T1), UE PDCP/RLC entities, which the DRB A is associated with, have UL packet which has already been processed by the old configuration. After that, the UE transmits the first UL packet without SDAP header and the second/third UL packets with SDAP header. The gNB receives these UL packets via the DRB A, and removes SDAP header of each received UL packet because the gNB considers that SDAP header is attached to the received UL packet. However, because there is no SDAP header in the first UL packet, a part of SDU in the first UL packet is removed by the gNB. It causes transmission of incorrect payload data.

Figure 1. Reception of UL packet omitting SDAP header in case #1

Observation 1: gNB may receive UL packets omitting SDAP header after reconfiguring the sdap-Header-UL to attach the SDAP header.

Observation 2: gNB removes a part of SDU in the UL packet if gNB has reconfigured the sdap-Header-UL to attach the SDAP header, but receives UL packets omitting SDAP header.

- CASE #2: sdap-Header-UL value is changed from present to absent

As shown in the Figure 2, gNB can reconfigure the sdap-Header-UL of the DRB A so that UE does NOT attach SDAP header to UL packets to be transmitted through the DRB A. At that time (T1), UE PDCP/RLC entities, which the DRB A is associated with, have UL packets which have already been processed by the old configuration. After that, the UE transmits the first/second UL packets with SDAP header and the third UL packet without SDAP header. The gNB receives these UL packets via the DRB A, and does NOT remove SDAP header of each received UL packet because the gNB considers that SDAP header is not attached to the received UL packet. However, because there is SDAP header in the first/second UL packets, the gNB ends up transmitting the SDUs with SDAP header to 5GC. It causes transmission of incorrect payload data.

Figure 2. Reception of UL packets with SDAP header in case #2

Observation 3: gNB may receive UL packets with SDAP header after reconfiguring the sdap-Header-UL not to attach the SDAP header.

Observation 4: gNB does NOT remove SDAP header in the UL packet if gNB has reconfigured the sdap-Header-UL not to attach the SDAP header, but receives UL packets with SDAP header.

Because these problems can have a bad effect on service quality, we propose the following:

Proposal 1:	 A reconfiguration of the sdap-Header-UL, which indicates the presence of UL SDAP header for a DRB, can be done only when the DRB's release/addition option is used.

3.	Proposal

In this document, we present our view on configurability for the presence of SDAP header, and propose following:

Proposal 1:	 A reconfiguration of the sdap-Header-UL, which indicates the presence of UL SDAP header for a DRB, can be done only when the DRB's release/addition option is used.