3GPP TSG-RAN WG2 #AH NR 1801	Tdoc R2-1801547

Vancouver, Canada, Jan 22nd – 26th, 2018

Title:	[draft] LS on Clarifications in MAC

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (to be RAN2)

To:	RAN4, RAN1

Contact Person:

#### Name:	Mats Folke

Tel. Number:	+46761271385

E-mail Address:	mats.folke@ericsson.com

1. Overall Description:

Action 1 – In which specification to capture the delays for activation and deactivation of secondary cells

Upon discussing the delays used for activating and deactivating secondary cells where TS 38.321 refer to TS 38.213 it was discovered that in sub-clause 4.3 TS 38.213 refer back to the RRC specification TS 38.331 for the definition of the delays.

RAN2 believe that this is an erroneous reference and that the correct reference should be TS 38.133. RAN2 does not intend to include any value for the delay in TS 38.331.

RAN2 therefore asks

- RAN1 to investigate and if needed, update the reference to the correct specification.

- RAN4 to confirm that TS 38.133 will contain the delay for activation and deactivation of secondary cells.

Action 2 – Values for DELTA_PREAMBLE

In the Random Access procedure for NR in 38.321, section 5.1.3, the MAC layer determines the preamble transmission power according to

- set PREAMBLE_RECEIVED_TARGET_POWER to ra-PreambleInitialReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep;

In 38.331, values for ra-PreambleInitialReceivedTargetPower and powerRampingStep are specified. The DELTA_PREAMBLE value is deduced from the configured preamble format. In LTE, the DELTA_PREAMBLE is included in a table in 36.321. However, for NR, no values have yet been specified and are not included in 38.321.

RAN2 believe that the DELTA_PREAMBLE values should be specified by RAN1 for the available preamble formats.

RAN2 therefore asks

- RAN1 to specify values for the DELTA_PREAMBLE parameter for the available preamble formats.

Action 3 – HARQ Feedback for SCell Activation and Deactivation

RAN2 has discussed the HARQ feedback when receiving a MAC CE for SCell Activation/Deactivation and captured the following text in TS 38.321 (similar text can be found in TS 36.321):

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

The motivation of this text in LTE was to guarantee that the eNB can confirm whether UE received activation/deactivation MAC CE to determine when to start/stop monitoring the CSI for the SCell. The similar requirement should be needed also for the NR SCell unless RAN4 confirms the activation and deactivation is always completed before the ACK/NACK transmission for the MAC CE. RAN2 thinks the UE behaviour in NR should correspond the UE behaviour in LTE, as shown with the specification text above.

RAN2 therefore asks

- RAN4 to confirm that HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell, and PUCCH SCell interruptions due to SCell activation/deactivation.

2. Actions:

To RAN1 group.

ACTIONS:	RAN2 asks RAN1 to investigate and if needed, update the reference to the correct specification.

RAN2 asks RAN1 to specify values for the DELTA_PREAMBLE parameter for the available preamble formats.

To RAN4 group.

ACTIONS:	RAN2 asks RAN4 to confirm that TS 38.133 will contain the delay for activation and deactivation of secondary cells.

RAN2 asks RAN4 to confirm that HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell, and PUCCH SCell interruptions due to SCell activation/deactivation.

3. Date of Next RAN2 Meetings:

RAN2#101	26th February – 2nd March 2018	Athens, Greece

RAN2#101-Bis	16th – 20th April 2018	China