3GPP TSG-RAN WG2 NR AH#3	Tdoc R2-1801598

Vancouver, Canada, 22nd – 26th January 2018

Title:	[DRAFT] LS on simplification of CSI-MeasConfig

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (to be RAN2)

To:	RAN1

Contact Person:

#### Name:	Henning Wiemann

Tel. Number:	+49 172 2798587

E-mail Address:	Henning [DOT] Wiemann [AT] Ericsson [DOT] com

# 1	Overall description

RAN2 aims to improve the RRC signalling for configuration of L1 CSI-RS measurements and reporting.

In this LS RAN2 provides a brief analysis of the current structure (based on the L1 parameter table (R1-1721581) and the subsequent parameter update received in RAN1's LS to RRC parameters related to NR MIMO (R1-1721734)) and a revised structure.

The new structure intends so simplify and correct the RRC signalling without limiting the CSI measurement reporting functionality designed by RAN1.

## 1.1	Analysis of current structure

In Figure 1 visualizes the current CSI-RS signalling structure in 38.331 v101. The figure does not aim for completeness but is rather supposed to show the dependencies of the high level information elements.

RAN2 understands that the main IEs for CSI-RS measurement reporting are ...

1.	[NZP-CSI-RS-/CSI-IM-/CSI-SSB-] Resource: Determines the actual resource elements to measure on

2.	CSI-ReportConfig: determines how/when/what the UE reports its measurements

As can be seen from the figure, there are however many other IEs:

3.	[NZP-CSI-RS-/CSI-IM-/CSI-SSB-] ResourceSets: A resource set defines (instantiates) and groups several resources and provides some additional information that is common to the resources in the set.

4. 	CSI-ResourceConfig (a.k.a”Resource Setting” in RAN1): It instantiates several resource sets and provides information that is common to the resource set in the resource setting.

5.	CSI-MeasIdToAddMod: Links CSI-ReportConfig to CSI-ResourceConfig (resource setting) similarly to the linking of RRM report configuration to measurement objects.

6.	TCI-State (a.k.a. TCI-RS-SetConfig in RAN1): A state provides quasi-co-location information for one or two reference signals. A list of TCI-State IEs is instantiated in the tci-States field in PDSCH-Config. The CSI Measurement configuration is supposed to reuse (refer to) those TCI-States.

7.	(aperiodic)ReportTrigger: A report trigger defines a DCI codepoint and associates it with

a)	a CSI-ReportConfig (which is linked to a CSI-ResourceConfig (setting) by the CSI-MeasIdToAddMod (see above))

b)	a TCI-State (as defined in PDSCH-Config) using the TCI-StateId.

c)	a NZP-CSI-RS-, CSI-IM- or CSI-SSB-ResourceSet (using the ID of the set)

Figure 1: Visualization of current CSI-RS signalling structure in 38.331 v101

When the UE detects the DCI codepoint on PDCCH, it is supposed to generate and transmit the CSI-Report based on measurements performed on the CSI resources (in the set) using the QCL information provided in the TCI-State. Instead of the aperiodic trigger (the DCI codepoint) a report and the associated resources may also be issued periodically (by RRC configuration) or semi-statically (by RRC configuration + subsequent MAC CE).

The CSI-MeasIdToAddMod (linking of report and resource setting), the 3 levels of resource handling (Resource, ResourceSet, ResourceConfig (setting)) and the separate configuration of trigger states (DCI codepoints) and TCI-States was meant to reduce the overhead. I.e., RAN1 intended to avoid duplicating resource configurations if those are used in several reports or associated with different QCL assumptions.

However, the current structure is not only very difficult to comprehend and extend. It is also questionable whether it really results in low overhead: Since the resources are instantiated inside the resource sets, they cannot be re-used in another resource set. Similarly, each resource set can currently be used only in a single resource setting (CSI-ResourceConfig). This implies also that currently the ID space of resources and resource sets is not unique outside the sets and settings. Hence, referring from an aperdiodicReportTrigger to a resource set by its ID is currently ambiguous. As a first step RAN2 therefore agreed to define pools of NZP-CSI-RS-, CSI-IM- and CSI-SSB-Resources on the top level (CSI-MeasConfig) with unique IDs so that they can be referred to widely and easily (e.g. from within sets, settings, TCI-RS-Sets (TCI-States)...). RAN2 considers to do the same for the resource sets which may make it easier to refer to them (their IDs) in MAC control elements. This is, however, not yet reflected in the remainder of this document.

Furthermore, RAN2 observed that the CSI-MeasId list (CSI-MeasIdToAddMod) is not sufficient to link reports to resources: Depending on the type of report (aperiodic/semi-persistent or aperiodic) a report can be linked to just one or to several resource sets. Furthermore, depending on the measurement quantity configured for a report, the UE must be informed about one or more sets of resources as well as how to use which of those resource (for channel or interference measurement). Since the information in the CSI-MeasIdToAddMod does not provide information about the resource sets, RAN1 placed those relations into the aperiodicTriggerState. In the figure above this was visualized by the purple dashed lines which now seem to bypass the linking through the CSI-MeasIdToAddMod (red line). RAN2 would like to avoid such complex linking and rather aim for a simpler structure e.g. by removing the CSI-Meas-IDs and relying on a direct linking between reports and resource sets.

RAN2 also discussed that the overall structure would become simpler and more easily extensible if one of the three levels in which the resources are currently configured (resource, sets, settings) is removed. In the remainder of this document, we removed the resource settings (CSI-ResourceConfig) and pushed the few remaining fields in the setting down into the resource set or into the report configuration. However, if several resource sets share several (large) IEs, the third level may be beneficial in terms of signalling overhead and could be kept. More important than the number of levels in the resource structure is a clean linking between reports and resource(set)s (see above).

## 1.2	Simplified ASN.1 structure

Figure 2 depicts a simplified ASN.1 structure as discussed and proposed in the previous section.

Figure 2: simplified ASN.1 structure

This structure can be summarized as follows:

-	The actual resources are defined in three lists (resource pools) on the top level of the CSI measurement configuration (nzp-csi-rs-Resources, ssb-Resources, csi-IM-Resources).

-	The list of CSI-ResourceConfig IEs (settings) is removed from the CSI measurement configuration.

-	The reports are configured in a list on the top level (as in the current structure)

-	For periodic and semi-persistent reports the CSI-ReportConfig defines up to three resource sets (one for channel, one for interference and/or one for CSI-IM ).

-	For aperiodic reports the CSI-ReportConfig defines one or more trigger states (”DCI codepoints” (to be down-selected by MAC CE)).

-	Each trigger state (codepoint) defines one channel-, one interference and/or one CSI-IM measurement resource-set. The structure of the resource set is lean: it primarily has a list of the resource IDs.

-	The NZP-CSI-ResourceSet points to a TCI-State (QCI information) by the TCI-StateId.

This structure allows re-using the resource configurations (e.g. NZP-CSI-Resource), which are relatively large and likely reusable, in several resource sets. And it allow an unambiguous and easy to comprehend linking between the report (or one of the DCI trigger states of a report) and the resources.

Unlike the original structure, it allows the NW to configure unambiguously, which resource set the UE shall use for which type of measurement (channel/interference).

In the Annex of this LS we provide the ASN.1 reflecting the signalling structure described above and in Figure 2.

RAN2 would like to point out that the current structure assumes that the CSI-ReportConfig links to one resource set (for each of channel, interference and CSI-IM). RAN2 wants to know whether RAN1 considers this sufficient or whether it is necessary to link them to several sets. The corresponding places are highlighted in red in the ASN.1.

# 2	Actions

RAN2 would like to point out that RAN2 intended not to change or limit the CSI measurement reporting functionality developped by RAN1. It is rather an attempt to simplify and streamline the RRC signalling to ensure consistency, implementability and future-extensibility. Therefore, RAN2 has the following request to RAN1.

ACTION to RAN1:  RAN2 respectfully asks RAN1 to review the revised signalling structure and to verify that the functionality intended by RAN1 is still supported. If RAN1 believes that this is not the case, RAN2 appreciates detailed information on those issues to be able to fix them in the RRC signalling. RAN2 would also like to be informed if RAN1 believes that the new signalling structure would result in more RRC reconfiguration procedures compared to the original structure.

# 3	Date of Next RAN2 Meetings:

RAN2#101	26 February - 2 March 2018	Athens, Greece

# Annex: Text Proposal

#### –	CSI-MeasConfig

The CSI-MeasConfig IE is used to configure the UE for measuring CSI-RS (reference signals) and for reporting those measurements on L1 (PUCCH, PUSCH) as channel state information. See also 38.214, section 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEAS-CONFIG-START

-- Top level parameter for CSI/BM framework. Contains lists of  reporting, resource, and link configurations (see 38.214, section 5.2)

CSI-MeasConfig ::=					SEQUENCE {

-- A pool of non-zero-power (NZP) resources that may be grouped and used in NZP-CSI-RS-ResourceSets and used in CSI-ReportConfig

nzp-csi-rs-Resources 				SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF NZP-CSI-RS-Resource						OPTIONAL,

-- A pool of SSB resources that may be grouped and used in NZP-CSI-RS-ResourceSets

ssb-Resources						SEQUENCE (SIZE (1..maxNrofSSB-Resources-1)) OF	CSI-SSB-Resource						OPTIONAL,

-- A pool of interference measurement (IM) resources that may be grouped and used in CSI-IM-ResourceSets and used in CSI-ReportConfig

csi-IM-Resources					SEQUENCE (SIZE (1..maxNrofCSI-Resources)) OF CSI-IM-Resource							OPTIONAL,

csi-ReportConfigs					SEQUENCE (SIZE (1..maxNrofCSI-Reports)) OF CSI-ReportConfig								OPTIONAL,

-- Size of CSI request field in DCI (bits). Corresponds to L1 parameter 'ReportTriggerSize' (see 38.214, section 5.2)

reportTriggerSize				INTEGER (0..6)																				OPTIONAL,

}

-- ASN1STOP

-- TAG-CSI-MEAS-CONFIG-STOP

#### –	CSI-ResourceConfig

The CSI-ResourceConfig IE is used to configure different types of resources on which the UE is supposed to configure measurements. It also allows configuring sets of those resources which are then referred to from within CSI-Report configurations.

CSI-ResourceConfig information element

-- TAG-CSI-RESOURCE-CONFIG-START

-- ASN1START

-- A set of Non-Zero-Power (NZP) CSI-RS resources (their IDs) or SSB-Resources (their IDs) and set-specific parameters.

-- Corresponds to L1 parameter 'NZP-CSI-RS-ResourceSetConfigList' (see 38.214, section 5.2)

NZP-CSI-RS-ResourceSet ::= 					SEQUENCE {

-- FFS: Remove ID if not needed (e.g. AddMod/release list)

nzp-CSI-RS-ResourceSetId						NZP-CSI-RS-ResourceSetId,

-- NZP-CSI-RS-Resources assocaited with this resource set.

-- Corresponds to L1 parameter 'CSI-RS-ResourceConfigList' (see 38.214, section 5.2)

-- FFS: Better make the csi-rs-Resources a common pool on CSI-MeasConfig level?

nzp-csi-rs-Resources 						SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId		OPTIONAL,

-- Indicates whether repetition is on/off. Repetition on (off), means that The UE can (cannot) assume that

-- the network maintains a fixed TX beam over the resources in the set.

-- Corresponds to L1 parameter 'ResourceRep' (see 38.214, section FFS_Section)

repetition									BOOLEAN,

-- Time domain behavior of resource configuration. Corresponds to L1 parameter 'ResourceConfigType' (see 38.214, section 5.2.2.3.5)

resourceType							CHOICE {

aperiodic								SEQUENCE {

-- Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the

-- CSI-RS resource set is transmitted. When the field is absent the UE applies the value 0.

-- Corresponds to L1 parameter 'Aperiodic-NZP-CSI-RS-TriggeringOffset' (see 38,214, section FFS_Section)

-- FFS_CHECK: Is this field at the correct place? Or should it be in the trigger configuration instead? If e.g. several aperiodic

-- triggers (DCIs) issue different reports but use the same resource, the offset must be different for different reports.

aperiodicTriggeringOffset			 		FFS_Value																		OPTIONAL,

-- A reference to one TCI-State and thereby a QCL source and a QCL type.

-- Corresponds to L1 parameter 'QCL-Info-aPeriodicReportingTrigger' (see 38.214, section FFS_Section)

qcl-Info									TCI-StateId																OPTIONAL

},

semiPersistent 							NULL,

periodic								SEQUENCE {

-- A reference to one TCI-State and thereby a QCL source and a QCL type.

-- Corresponds to L1 parameter 'QCL-Info-PeriodicCSI-RS' (see 38.214, section FFS_Section)

qcl-Info									TCI-StateId																OPTIONAL

}

},

-- Indicates whether or not the antenna ports of NZP CSI-RS resources in the CSI-RS resource set is same

-- FFS: Correct description: Should it say ”are the same”? Clarify same as ”what”.

-- Corresponds to L1 parameter 'TRS-Info' (see 38.214, section 5.2.2.3.1)

trs-Info								ENUMERATED {true}																OPTIONAL

}

NZP-CSI-RS-ResourceSetId ::= 			INTEGER (0..maxNrofNZP-CSI-RS-ResourceSets-1)

-- A CSI-RS (reference signal) resource which the UE may be configured to measure on (see 38.214, section 5.2.1.3.1)

NZP-CSI-RS-Resource ::=					SEQUENCE {

nzp-csi-rs-ResourceId					NZP-CSI-RS-ResourceId,

-- Number of ports (see 38.214, section 5.2.2.3.1)

nrofPorts								ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

-- OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource

-- Corresponds to L1 parameter 'CSI-RS-ResourceMapping'  (see 38.214, section 5.2.2.3.1)

resourceMapping							SEQUENCE {

-- Frequency domain allocation within a physical resource block in accordance with 38.211, table 7.4.1.5.2-1.

-- The number of bits that may be set to one depend on the chosen row in that table.

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12)),

row4								BIT STRING (SIZE (3)),

other								BIT STRING (SIZE (6))

},

-- Time domain allocation within a physical resource block. The field indicates the first OFDM symbol in the PRB used for CSI-RS.

-- Value 2 is supported only when DL-DMRS-typeA-pos equals 3.

firstOFDMSymbolInTimeDomain			INTEGER (0..13)

},

-- CDM type (see 38.214, section 5.2.2.3.1)

cdm-Type								ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

-- Density of CSI-RS resource measured in RE/port/PRB. Corresponds to L1 parameter 'CSI-RS-Density'  (see 38.214, section 5.2.2.3.1)

-- Values 0.5 (dot5), 1 (one) and 3 (three) are allowed for X=1,

-- values 0.5 (dot5) and 1 (one) are allowed for X=2, 16, 24 and 32,

-- value 1 (one) is allowed for X=4, 8, 12.

-- For density = 1/2, includes 1 bit indication for RB level comb offset indicating  whether odd or even RBs are occupied by CSI-RS

density									CHOICE {

dot5									ENUMERATED {evenPRBs, oddPRBs},

one										NULL,

three									NULL,

spare									NULL

},

-- Wideband or partial band CSI-RS. Corresponds to L1 parameter 'CSI-RS-FreqBand' (see 38.214, section 5.2.2.3.1)

freqBand								SEQUENCE {

-- PRB where this NZP-CSI-RS-Resource starts in relation to PRB 0 of the associated BWP. Only multiples of 4 are allowed (0, 4, ...)

startingRB								INTEGER (0..maxNrofPhysicalResourceBlocks-1),

-- Number of PRBs across which this NZP-CSI-RS-Resource spans. Only multiples of 4 are allowed. The smallest configurable

-- number is the minimum of 24 and the width of the associated BWP.

nrofRBs									INTEGER (24..maxNrofPhysicalResourceBlocks)

},

-- Power offset of NZP CSI-RS RE to PDSCH RE. Value in dB. Corresponds to L1 parameter Pc (see 38.214, section 5.2.2.3.1)

powerControlOffset						INTEGER(-8..15),

-- Power offset of NZP CSI-RS RE to SS RE. Value in dB. Corresponds to L1 parameter 'Pc_SS' (see 38.214, section FFS_Section)

powerControlOffsetSS					ENUMERATED{db-3, db0, db3, db6}													OPTIONAL,

-- Scrambling ID (see 38.214, section 5.2.2.3.1)

scramblingID							ScramblingId,

-- Periodicity and slot offset in number of slots. Corresponds to L1 parameter 'CSI-RS-timeConfig' (see 38.214, section 5.2.2.3.1)

periodicityAndOffset					CHOICE {

sl5										INTEGER (0..4),

sl10									INTEGER (0..9),

sl20									INTEGER (0..19),

sl40									INTEGER (0..39),

sl80									INTEGER (0..79),

sl160									INTEGER (0..159),

sl320									INTEGER (0..319),

sl640									INTEGER (0..639)

}																							OPTIONAL, -- Cond PeriodicOrSemiPersistentOnly

}

NZP-CSI-RS-ResourceId ::= 					INTEGER (0..maxNrofNZP-CSI-RS-Resources-1)

CSI-SSB-ResourceSet ::=					SEQUENCE {

-- FFS: Remove ID if not needed (e.g. AddMod/release list)

csi-SSB-ResourceSetId						CSI-SSB-ResourceSetId

ssb-Resources								SEQUENCE (SIZE (1..maxNrofSSB-Resources-1)) OF	CSI-SSB-ResourceId

}

CSI-SSB-ResourceSetId ::=				INTEGER (0..maxNrofCSI-SSB-ResourceSets-1)

CSI-SSB-Resource ::=					SEQUENCE {

csi-SSB-ResourceId						CSI-SSB-ResourceId

-- FFS: Undefined what the IE CSI-SSB-Resource contains.

}

CSI-SSB-ResourceId ::=					INTEGER (0..maxNrofCSI-SSB-Resources-1)

-- A set of CSI Interference Management (IM) resources (their IDs) and set-specific parameters

CSI-IM-ResourceSet ::=					SEQUENCE {

-- FFS: Where is the csi-im-ResourceSetId used?

csi-IM-ResourceSetId						CSI-IM-ResourceSetId,

-- CSI-IM-Resources associated with this CSI-IM-ResourceSet

-- Corresponds to L1 parameter 'CSI-IM-ResourceConfigList' (see 38.214, section 5.2)

csi-IM-Resources						SEQUENCE (SIZE(1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId

}

CSI-IM-ResourceSetId ::=				INTEGER (0..maxNrofCSI-IM-ResourceSets-1)

CSI-IM-Resource ::= 					SEQUENCE {

csi-IM-ResourceId						CSI-IM-ResourceId,

-- The resource element pattern for the CSI-IM resource

-- Corresponds to L1 parameter 'CSI-IM-RE-pattern' (see 38.214, section 5.2.2.3.4)

csi-IM-ResourceElementPattern				ENUMERATED {pattern2-2, pattern4-1},

-- OFDM symbol and subcarrier occupancy of the CSI-IM resource within a slot

-- Corresponds to L1 parameter 'CSI-IM-ResourceMapping' (see 38.214, section 5.2.2.3.4)

-- FFS_Values: RAN1 indicated “symbol locations: [0..13] and subcarrier locations: [0..9]”  Should this be a bitmap of 9x13? Or two separate?

resourceMapping								ENUMERATED {ffsTypeAndValue}		OPTIONAL,

-- Frequency-occupancy of CSI-IM. Corresponds to L1 parameter 'CSI-IM-FreqBand' (see 38.214, section 5.2.2.3.2)

freqBand									FFS_Value		OPTIONAL

}

CSI-IM-ResourceId ::= 					INTEGER (0..maxNrofCSI-IM-Resources-1)

-- TAG-CSI-RESOURCE-CONFIG-STOP

-- ASN1STOP

#### –	CSI-ReportConfig

The CSI-ReportConfig IE is used to configure a CSI-Report to be sent on L1 (e.g. PUCCH) (see 38.214, section 5.2.1)

CSI-ReportConfig information element

-- TAG-CSI-REPORT-CONFIG-START

-- ASN1START

CSI-ReportConfig ::=					SEQUENCE {

reportConfigId							CSI-ReportConfigId,

-- Time domain behavior of reporting configuration

reportConfigType						CHOICE {

periodic								SEQUENCE {

channelResourceSet						CHOICE {

channel-NZP-CSI-RS-ResourceSet			NZP-CSI-RS-ResourceSet,

channel-SSB-CSI-ResourceSet				CSI-SSB-ResourceSet

},

interference-NZP-CSI-RS-ResourceSet		NZP-CSI-RS-ResourceSet												OPTIONAL,

csi-IM-ResourceSet						CSI-IM-ResourceSet													OPTIONAL,

-- Periodicity and slot offset . Corresponds to L1 parameter 'ReportPeriodicity'and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CHOICE {

sl5										INTEGER(0..4),

sl10									INTEGER(0..9),

sl20									INTEGER(0..19),

sl40									INTEGER(0..39),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319)

},

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-Resources					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) {

uplinkBandwidthPartId				UplinkBandwidthPartId,

pucch-Resource						PUCCH-Resource

},

semiPersistentPUCCH						SEQUENCE {

channelResourceSet						CHOICE {

channel-NZP-CSI-RS-ResourceSet			NZP-CSI-RS-ResourceSet,

channel-SSB-CSI-ResourceSet				CSI-SSB-ResourceSet

},

interference-NZP-CSI-RS-ResourceSet		NZP-CSI-RS-ResourceSet												OPTIONAL,

csi-IM-ResourceSet						CSI-IM-ResourceSet													OPTIONAL,

-- Periodicity and slot offset. Corresponds to L1 parameter 'ReportPeriodicity' and 'ReportSlotOffset'

-- (see 38.214, section section 5.2.1.4).

reportSlotConfig						CHOICE {

sl5										INTEGER(0..4),

sl10									INTEGER(0..9),

sl20									INTEGER(0..19),

sl40									INTEGER(0..39),

sl80									INTEGER(0..79),

sl160									INTEGER(0..159),

sl320									INTEGER(0..319)

},

-- Indicates which PUCCH resource to use for reporting on PUCCH.

pucch-CSI-Resources					SEQUENCE (SIZE (1..maxNrofUplinkBandwidthParts)) {

uplinkBandwidthPartId				UplinkBandwidthPartId,

pucch-Resource						PUCCH-Resource

}

},

semiPersistentPUSCH						SEQUENCE {

dci-CodePoint							INTEGER(0..63),

channelResourceSet						CHOICE {

channel-NZP-CSI-RS-ResourceSet			NZP-CSI-RS-ResourceSet,

channel-SSB-CSI-ResourceSet				CSI-SSB-ResourceSet

},

interference-NZP-CSI-RS-ResourceSet		NZP-CSI-RS-ResourceSet												OPTIONAL,

csi-IM-ResourceSet						CSI-IM-ResourceSet													OPTIONAL,

-- Periodicity. Corresponds to L1 parameter 'Reportperiodicity-spCSI'. (see 38.214, section FFS_Section)

reportSlotConfig						ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320},

-- RNTI for SP CSI-RNTI, Corresponds to L1 parameter 'SPCSI-RNTI' (see 38.214, section FFS_Section)

-- FFS: RAN1 models different RNTIs as different Search Spaces with independent configurations. Align the configuration

-- of this one (e.g. group with monitoring periodicity, PDCCH candidate configuration, DCI-Payload size...)?

csi-RNTI								RNTI-Value,

-- Index of the p0-alpha set determining the power control for this CSI report transmission.

-- Corresponds to L1 parameter 'SPCSI-p0alpha' (see 38.214, section FFS_Section)

p0alpha									P0-PUSCH-AlphaSetId

},

aperiodic								SEQUENCE {

-- Timing offset Y for aperiodic reporting. This field lists the allowed offset values. A particular value is indicated in DCI.

-- (see 38.214, section 5.2.1.1)

-- FFS_Value: Range wasn’t final in RAN1 table.

-- FFS_FIXME: How are the DCI codepoints mapped to the allowed offsets? Is this the DCI field which also triggers the report?

reportSlotOffsets					SEQUENCE (SIZE (1..4)) OF INTEGER (0..8),

triggers							SEQUENCE (SIZE (1..maxNrofCSI-ReportCodePoints)) {

-- If the UE detects this codepoint on DCI, it triggers this CSI-Report using the associated channel-, interference-,

-- and CSI-IM- resource sets. If the NW intends to configure more report triggers than the DCI field length

-- (configured in the field reportTriggerSize) supports, the mapping of the configured dci-CodePoint to the actual value

-- in the DCI payload is provided via a MAC CE (FFS: Add exact MAC CE name and reference to 38.321).

dci-CodePoint						INTEGER(0..63),

-- A set of non-zero-power (NZP) resources which the UE uses to perform channel measurements

channelResourceSet					CHOICE {

channel-NZP-CSI-RS-ResourceSet		NZP-CSI-RS-ResourceSet,

channel-SSB-CSI-ResourceSet			CSI-SSB-ResourceSet

},

-- A set of non-zero-power (NZP) resources which the UE uses to perform interference measurements

interference-NZP-CSI-RS-ResourceSet	NZP-CSI-RS-ResourceSet												OPTIONAL,

-- A set of interference measurement resources

csi-IM-ResourceSet					CSI-IM-ResourceSet													OPTIONAL

}

}

},

-- The CSI related quanities to report (see 38.214, section REF)

reportQuantity							CHOICE {

none									NULL,

cri-RI-PMI-CQI							NULL,

cri-RI-i1								NULL,

cri-RI-i1-CQI							SEQUENCE {

-- PRB bundling size to assume for CQI calcuation when reportQuantity is CRI/RI/i1/CQI

-- Corresponds to L1 parameter 'PDSCH-bundle-size-for-CSI' (see 38.214, section FFS_Section)

pdsch-BundleSizeForCSI					ENUMERATED {n2, n4}		OPTIONAL

},

cri-RI-CQI								NULL,

cri										NULL,

cri-RSRP								NULL,

cri-RI-LI-PMI-CQI						NULL

},

-- Reporting configuration in the frequency domain. (see 38.214, section 5.2.1)

reportFreqConfiguration					 SEQUENCE {

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) CQI. (see 38.214, section 5.2.1.4)

cqi-FormatIndicator						ENUMERATED { widebandCQI, subbandCQI },

-- Indicates whether the UE shall report a single (wideband) or multiple (subband) PMI. (see 38.214, section 5.2.1.4)

pmi-FormatIndicator						ENUMERATED { widebandPMI, subbandPMI },

-- Indicates a contiguous or non-contigous subset of subbands in the bandwidth part which CSI shall be reported

-- for. FFS: Each bit in the bit-string represents one subband. The right-most bit in the bit string represents the

-- lowest subband in the BWP. (see 38.214, section 5.2.1.4)

-- FFS: Size of the bitmap. Introduce a CHOICE with different bitmap lengths depening on number of subbands in carrier/BWP?

csi-ReportingBand						BIT STRING (SIZE (ffsValue))

},

-- Time domain measurement restriction for the channel (signal) measurements. (see 38.214, section 5.2.1.1)

measRestrictionTimeForChannel				ENUMERATED {ffsTypeAndValue},

-- Time domain measurement restriction for interference measurements. (see 38.214, section 5.2.1.1)

measRestrictionTimeForInterference			ENUMERATED {ffsTypeAndValue},

-- Codebook configuration for Type-1 or Type-II including codebook subset restriction

codebookConfig								CodebookConfig,

-- Maximum number of CQIs per CSI report (cf. 1 for 1-CW, 2 for 2-CW)

nrofCQIsPerReport							ENUMERATED {n1, n2},

-- Turning on/off group beam based reporting (see 38.214, section FFS_Section)

groupBasedBeamReporting						CHOICE {

enabled										SEQUENCE {

-- Number of beams to report for group based beam reporting (see 38.214, section REF)

nrofBeamsToReport						ENUMERATED {ffsTypeAndValue}

},

disabled 								SEQUENCE {

-- The number (N) of measured RS resources to be reported per report setting in a non-group-based report.

-- N <= N_max, where N_max is either 2 or 4 depending on UE capability.

-- FFS: The signaling mechanism for the gNB to select a subset of N beams for the UE to measure and report.

-- FFS: Note: this parameter may not be needed for certain resource and/or report settings

-- FFS_ASN1: Change groupBasedBeamReporting into a CHOICE and include this field into the “no” option?

-- (see 38.214, section FFS_Section)

-- When the field is absent the UE applies the value 1

nrofReportedRS							ENUMERATED {n1, n2, n3, n4}												OPTIONAL

}

},

-- Which CQI table to use for CQI calculation. Corresponds to L1 parameter 'CQI-Table' (see 38.214, section FFS_Section)

-- FFS: Whether URLLC2 should be added as one option

cqi-Table								ENUMERATED {qam64, qam256, urllc1, urllc2}										OPTIONAL,

-- Indicates one out of two possible BWP-dependent values for the subband size

-- Corresponds to L1 parameter 'SubbandSize' (see 38.214, section FFS_Section)

-- FFS_Value: Clarify what value1 and value2 mean.

subbandSize								ENUMERATED {value1, value2},

-- BLER target that the UE shall be assume in its CQI calculation.

-- Corresponds to L1 parameter 'BLER-Target' (see 38.214, section FFS_Section)

-- FFS_Values (now filled with spares)

bler-Target								ENUMERATED {zerodot1, spare3, space2, spare1}										OPTIONAL,

-- Port indication for RI/CQI calculation. For each  CSI-RS resource in the linked ResourceConfig for channel measurement,

-- a port indication for each rank R, indicating which R ports to use. Applicable only for non-PMI feedback.

-- Corresponds to L1 parameter 'Non-PMI-PortIndication' (see 38.214, section FFS_Section)

non-PMI-PortIndication					FFS_Value																		OPTIONAL,

-- Which DL BWP the CSI-ReportConfig is associated with. (see 38.214, section FFS_Section)

-- FFS_CHECK: Should it be possible to link a report to several BWPs? If not, shouldn’t the report configuration be in the BWP?

-- FFS_CHECK: Should it be possible to link a report to the initial BWP? If so, which ID does that have?

bandwidthPartId							BandwidthPartId																	OPTIONAL

}

CSI-ReportConfigId ::= 					INTEGER (0..maxNrofCSI-ReportConfig-1)

-- Codebook configuration for Type-I and Type-II (see 38.214, section 5.2.2.2)

CodebookConfig ::= 						SEQUENCE {

-- Number of antenna ports in first dimension

codebookConfig-N1						ENUMERATED {n1,n2,n3,n4,n6,n8,n12,n16},

-- Number of antenna ports in second dimension

codebookConfig-N2						ENUMERATED {n1,n2,n3,n4},

-- Codebook subset restriction for the different codebooks

-- CodebookType including possibly sub-types and the corresponding parameters for each. Corresponds to L1 parameter 'CodebookType'

-- (see 38.214, section 5.2.2.2)

codebookType 							CHOICE {

type1 									SEQUENCE {

subType									ENUMERATED {typeI-SinglePanel, typeI-MultiPanel},

-- Switch between Config 1 and Config 2

codebookMode							ENUMERATED {config1, config2},

-- Number of panels, Ng, used in multi-panel codebook

numberOfPanels							ENUMERATED {twopanels, fourpanels}					OPTIONAL,		-- Cond TypeI-MultiPanel

codebookSubsetRestrictionType1			CHOICE {

-- Codebook subset restriction for Type I Single-panel codebook

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

-- FFS_Value: RAN1 indicated Bitmap of size N1*N2*O1*O2

singlePanel								BIT STRING (SIZE (ffsValue)),

-- Codebook subset restriction for 2TX codebook

-- Corresponds to L1 parameter 'TypeI-SinglePanel-2Tx-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

singlePanel2TX							BIT STRING (SIZE (6)),

-- Codebook subset restriction for Type I Multi-panel codebook

-- Corresponds to L1 parameter 'TypeI-MultiPanel-CodebookSubsetRestriction' (see 38.214, section FFS_Section)

multiPanel								BIT STRING (SIZE (ffsValue)),

-- i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI

-- Corresponds to L1 parameter 'TypeI-SinglePanel-CodebookSubsetRestriction-i2' (see 38.214, section FFS_Section)

singlePanelCodebookSubsetRestriction-i2	BIT STRING (SIZE (16))

},

ri-Restriction							CHOICE {

-- Restriction for RI for TypeI-SinglePanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-SinglePanel-RI-Restriction' (see 38.214, section FFS_Section)

typeI-SinglePanelRI-Restriction			BIT STRING (SIZE (8)),

-- Restriction for RI for TypeI-MultiPanel-RI-Restriction

-- Corresponds to L1 parameter 'TypeI-MultiPanel-RI-Restriction' (see 38.214, section FFS_Section)

typeI-MultiPanelRI-Restriction			BIT STRING (SIZE (4))

}

},

type2									SEQUENCE {

subType									ENUMERATED {typeII, typeII-PortSelection},

-- The size of the PSK alphabet, QPSK or 8-PSK

phaseAlphabetSize						ENUMERATED {n4, n8},

-- If subband amplitude reporting is activated (true)

subbandAmplitude						BOOLEAN,

-- Number of beams, L,  used for linear combination

numberOfBeams							ENUMERATED {two, three, four},

-- The size of the port selection codebook (parameter d)

portSelectionSamplingSize				ENUMERATED {n1, n2, n3, n4}					OPTIONAL,		-- Cond TypeII-PortSelection

-- Codebook subset restriction for Type II codebook.

-- FFS: Clarify the meaning of the bitmap

-- FFS: The size of the bitmap is ceil(log2(nchoosek(O1*O2,4)))+8*N1*N2 ==> Clarify size. Present different bitmap sizes by CHOICE?

codebookSubsetRestrictionType2			BIT STRING (SIZE (ffsValue)),

ri-Restriction							CHOICE {

-- Restriction for RI for TypeII-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-RI-Restriction' (see 38.214, section FFS_Section)

typeII-RI-Restriction					BIT STRING (SIZE (2)),

-- Restriction for RI for TypeII-PortSelection-RI-Restriction

-- Corresponds to L1 parameter 'TypeII-PortSelection-RI-Restriction' (see 38.214, section FFS_Section)

typeII-PortSelectionRI-Restriction		BIT STRING (SIZE (2))

}

}

}

}

-- TAG-CSI-REPORT-CONFIG-STOP

-- ASN1STOP