3GPP TSG-RAN WG2 NR Ad hoc 0118                                                                R2-1800149

Vancouver, Canada, 22nd January – 26th January 2018              Revision of R2-1712429

Source:	CATT

Title:	Considerations on idle mode measurement

Agenda Item:	10.4.5.3

Document for:	Discussion and Decision

Introduction

In RAN2#97 meeting, the following agreements were made for cell reselection.

Agreements

1	For cell reselection, cell quality can be derived from N best beams where value of N can be configured to 1 or more than 1.

FFS: Details of filtering to be applied (e.g. for the case N=1, the best beam is filtered by a single filter as the best beam changes)

FFS: Whether to only consider beams above a threshold ('good' beams)

And there is no further agreement on cell quality for idle mode measurement. In this contribution, some open issues on idle mode measurement were discussed.

Discussion

- 2.1    Cell selection

We had agreed that LTE cell selection mechanism is the baseline for NR.

In [1], two cell selection procedures are captured.

Cell selection is performed by one of the following two procedures: a) Initial cell selection (no prior knowledge of which RF channels are NR carriers); 1. The UE shall scan all RF channels in the NR bands according to its capabilities to find a suitable cell. 2. On each carrier frequency, the UE need only search for the strongest cell. 3. Once a suitable cell is found this cell shall be selected. b) Cell selection by leveraging stored information. 1. This procedure requires stored information of carrier frequencies and optionally also information on cell parameters, from previously received measurement control information elements or from previously detected cells. 2. Once the UE has found a suitable cell the UE shall select it. 3. If no suitable cell is found the Initial Cell Selection procedure shall be started.

Cell quality is applied when a cell is evaluated for cell selection using cell selection criterion S.

There are four options for cell quality derivation of cell selection:

Option 1: The best beam is used.

Option 2: Quality of one beam can be used as cell quality for cell selection. If there is one beam of a cell that fulfils S criterion, the cell is considered as a suitable cell and can be selected.

Option 3: Same as connected mode, N best beams is used.

Option 4: Up to UE implementation.

There is no requirement for cell selection in RAN4 for LTE. Cell selection procedure aims to select a suitable cell as soon as possible. It is not necessary to derive a stable cell quality and compare with other cells during cell selection procedure. Hence, option 1, option 2, and option 3 are feasible for cell quality derivation of cell selection. There is no obvious disadvantage and advantage among the three options. Considering the flexibility, option 4 is preferred.

Proposal 1: Cell quality for Cell Selection is up to UE implementation.

- 2.2    Cell reselection

1)  Cell quality derivation

Agreements of cell quality derivation in RRC connected mode are listed as follows.

RAN2#Ad Hoc 2 in Qindao: Agreement - Cell quality should be derived by averaging the best beam with the up to N-1 best beams above absolute configured threshold. RAN2#99 Agreements 1: Independent N and independent threshold should be configured per carrier frequency in the MeasObject for NR-SS based and CSI-RS based L3 mobility. (This agreement does not have any implication on the number of CSI-RS resources that can be configured per cell)

Both idle mode measurement and connected mode measurement are based on NR-SS. Cell reselection is used for idle mode mobility while measurement reporting in connected mode is used for connected mode mobility. There is no difference for cell quality derivation of cell reselection and measurement reporting in connected mode. The same principle can be reused.

Proposal 2: Agreements on cell quality derivation of NR-SS measurement in connected mode can be reused for cell quality derivation of cell reselection, i.e.

- Cell quality of cell reselection should be derived by averaging the best beam with the up to N-1 best beams above absolute configured threshold

- Independent N and independent threshold should be configured per carrier frequency in SI for cell reselection

2) Cell quantities

In LTE, both RSRP and RSRQ are supported as cell quantities in idle mode measurement. And both RSRP and RSRQ have already been agreed as measurement quantities in connected mode measurement report. It is nature that both RSRP and RSRQ are supported as cell quantities NR measurement in idle mode.

Proposal 3: both RSRP and RSRQ are supported as cell quantities NR measurement in idle mode.

3)L3 cell filtering

Based on existing agreements on RRM measurement in connected mode, cell quality derivation is performed after L1 filtering, and L3 cell filtering is performed based on cell quality. In RAN2#100, we agreed:

Different filter coefficients can be configured for different measurement quantities, for different RS types, and for cell and beam measurements.

We suggest that principles of cell quality derivation in connected mode are reused for that in cell reselection, i.e. the network broadcasts N and threshold for cell quality derivation in cell reselection. But for L3 cell filtering, we need to discuss whether L3 cell filtering would be defined in idle mode and the network would broadcast a parameter for L3 cell filtering for cell reselection. In LTE, L3 filtering for a cell in idle mode is up to UE implementation. The same principle can be reused for NR.

Proposal 4: How to realize L3 cell filtering for idle mode measurement is up to UE implementation.

4)Beam selection

For connected mode measurement, we agreed to introduce beam selection for beam reporting. Beam section can be used for RACH selection. During cell reselection, RACH selection is not needed. Hence, beam selection is not supported during cell reselection.

Proposal 5: Beam selection is not needed for cell reselection.

Conclusion

In this contribution, we discuss idle mode measurement for cell selection and cell reselection. And we propose that：

Proposal 1: Cell quality for Cell Selection is up to UE implementation.

Proposal 2: Agreements on cell quality derivation of NR-SS measurement in connected mode can be reused for cell quality derivation of cell reselection , i.e.

- Cell quality of cell reselection should be derived by averaging the best beam with the up to N-1 best beams above absolute configured threshold

- Independent N and independent threshold should be configured per carrier frequency in SI for cell reselection

Proposal 3: both RSRP and RSRQ are supported as cell quantities NR measurement in idle mode.

Proposal 4: How to realize L3 filter for idle mode measurement is up to UE implementation.

Proposal 5: Beam selection is not needed for cell reselection.

Reference

[1]  TS 38.304 v0.6.0