3GPP TSG-RAN WG2 NR AH#1801	Tdoc R2-1800699

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.4.4

Source:	Ericsson

Title:	SDAP Configuration Aspects

Document for:	Discussion, Decision

# Introduction

The 38.331 specifies following SDAP configuration:

Table 1	RadioBearerConfig information element

RadioBearerConfig

DRB-ToAddModList ::=					SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

DRB-ToAddMod ::=						SEQUENCE {

cnAssociation							CHOICE {

-- The EPS bearer ID determines the EPS bearer when NR connects to EPC using EN-DC

eps-BearerIdentity						INTEGER (0..15)	OPTIONAL, -- Cond EPS-DRB-Setup

--	The SDAP configuration determines how to map QoS flows to DRBs when NR connects to the 5GC

sdap-Config								SDAP-Config		OPTIONAL		-- Cond NGC

},

drb-Identity							DRB-Identity,

-- may only be set if the cell groups of all linked logical channels are reset or released

reestablishPDCP							ENUMERATED{true}	OPTIONAL, 		-- Cond HO

recoverPDCP								ENUMERATED{true}	OPTIONAL, 		-- Need N

pdcp-Config								PDCP-Config			OPTIONAL,		-- Cond PDCP

...

}

Table 2  SDAP-Config information element

-- ASN1START

-- TAG-SDAP-CONFIG-START

SDAP-Config ::=						SEQUENCE {

-- FFS / TODO: Definition of PDUsessionID to be added

pduSession							PDUsessionID,

-- FFS: separate configuration for UL and DL

sdap-Header-DL 						ENUMERATED {present, absent},

sdap-Header-UL 						ENUMERATED {present, absent}

defaultDRB							BOOLEAN,

reflectiveQoS						BOOLEAN,	-- It is FFS whether this field is needed

-- FFS: Is the simple list sufficient? Replace by add/mod/release list? Or bitmap?

mappedQoSflows						SEQUENCE (SIZE (0..maxNrofQFIs)) OF QFI OPTIONAL, -- Need N

...

}

QFI :: = 							INTEGER (0..maxQFI)

-- TAG-SDAP-CONFIG-STOP

-- ASN1STOP

This contribution discusses configurations that are not yet agreed.

# Discussion

RAN2-AH in January (2017) agreed that there is one SDAP entity per PDU session. RAN2-97bis made clarification to the agreement that two SDAP entities may be established in DC scenarios. When PDU session is not split between MN and SN, there is no problem to identify SDAP entity. The SDAP entity may be established in PDU session establishment and the SDAP can be identified with PDU session ID.

Having PDU session split between MN and SN results that there are two SDAP entities on the network side, one in SN and another in MN. Having same PDU session between SDAP entities should not be a problem as it is expected that the corresponding cell group configures its SDAP entity, resulting that SDAP entity configurations are received separately. From the configuration source, UE may identify and assign ID for SDAP entity if needed. Having PDU session ID in the RRC configuration is useful because it binds the SDAP entity to the PDU session ID in UE.

When PDU session is split between MgNB and SgNB, same PDU session ID is used

PDU session ID is send to UE in SDAP-config

In LTE, when PDN connection is split between MN and SN, the DRB IDs are unique between MN and SN. Having both, DRB and QFI unique inside same PDU session allows UE to have one mapping table, from which it can perform QFI to DRB mapping. Alternatively, not having DRB ID unique inside SDAP entity would require two level mapping between QFI->SDAP and QFI -> DRB. This adds complexity and it is preferable to keep LTE behaviour with unique DRB inside same PDU session. The QFI, by its SA2 definition, is already unique within one PDU session.

DRB IDs are unique inside one UE

## SDAP configuration and N3 marking with multiple Flows per PDU session

When multiple flows are configured per one PDU session, there is a need to differentiate traffic between flows. For this purpose, SA2 agreed all UL flows are marked with QFI in N3 header. For a gNB, this results that all packets destined to CN need to be marked. The marking from the gNB perspective is trivial when there is only one flow in PDU session or all flows in PDU session are mapped into separate DRBs. However, when multiple flows exist on one PDU session and more than one flow is mapped to a same DRB, gNB needs to be able to distinguish traffic between flows. This is only possible if UL packets are marked from UE with the QFI information. This case may occur even when NAS-reflective QoS is not active. For this reason, the SDAP header configuration should be separate for UL and DL.

Even when there are multiple QFIs per PDU session, it is not necessary for gNB to mark QFI for DL packets if no AS reflective QoS is configured. gNB may use the QFI information provided in N3 header to make decisions on scheduling. This way gNB is able to guarantee per-flow treatment, which is enough to full fill QoS requirements over the air. Detailed RRC configuration is discussed in [2].

Existence SDAP header is configured separately for UL and for DL

## SDAP header coupling with AS-reflective QoS

There are two ways for RAN to configure QoS flow to DRB mapping, one is to configure it with AS reflective QoS and another is to configure it via explicit RRC signaling.

When the 5CN uses NAS-level QoS rules, N1 signaling is used to carry QoS information to the UE. This results that the UE is aware of QoS profile associated with a specific flow in PDU session. As the 5CN informs UE, it may correspondingly inform RAN about the QoS flow information. Based on the QoS information associated with the flow, RAN is able to determine QoS flow to DRB mapping, which is carried via RRC signaling. The gNB may inform UE to map the QFI to new DRB or existing DRB. While carrying this information gNB may or may not modify the DRB to have better match for the QoS profile of the flow.  SDAP entity of the UE uses the flow to DRB mapping to send UL traffic in DRB.

Configuration of the SDAP entity is not coupled with the enabling and disabling AS reflective QoS. In the case where many QoS flows are associated with the same DRB, UE attaches QFI to the UL direction. Naturally, the SDAP entity is configured in this case to include the header. Presence of the SDAP header depends on whether there is more than one flow configured per DRB. However, SDAP header may also be present for one flow per DRB. Presence of SDAP header is indicated within RRCConnectionReconfiguration applied with the full configuration option (fullConfig). SDAP header details are discussed in contribution [1].

Presence of the SDAP header is not coupled with the enabling and disabling AS reflective QoS.

## Dynamic presence of QoS flow ID

RAN2-98 discussed the topic again and concluded the following:

1. The QoS flow ID is presence once the AS reflective QoS is active.  FFS whether it is always present.

2. gNB should be informed when NAS layer reflective QoS is activated (e.g. can be used).  It is FFS how we handle NAS reflective QoS and dependent on how/when it will be provided.

3. RAN2 will support a mode in which SDAP header is not present and is configured per DRB.  If configured, FFS how the different fields are handled.

Bullet 3 in above implies that the eNB configures by RRC for each DRB whether or not the UE shall include the SDAP header in uplink PDCP SDUs and whether the SDAP header is present in DL PDCP SDUs. According to bullet 1, it should be discussed further whether the “QoS flow ID”, once SDAP is configured, “is always present” or whether it may be present only dynamically. To achieve the latter the SDAP header would need to indicate with one bit the presence of the “QoS flow ID”. Since such an indication would consume one bit itself it would not allow reducing the size of the SDAP header below one octet. Hence, we consider it more efficient to aim for an SDAP header that has a fixed size of one byte (if configured to be present by RRC).

SDAP header is configured for a DRB by means of RRC. Depending on UL and DL configuration, the QFI is present in all UL and/or DL packets on that DRB (not dynamically enabled/disabled).

## Re-configuring presence of SDAP header

Since UE and network must be aware at any point in time which PDCP SDUs contain an SDAP header, the presence of this header should only be changed by a synchronized reconfiguration, i.e., RRCConnectionReconfiguration including mobilityControlInfo. One should note that this still requires that the PDCP receiver entity informs the SDAP entity for each delivered PDCP PDU whether the SDAP header is present. If one would like to avoid that, RAN2 should restrict the configuration of SDAP header to full configuration (fullConfig). However, we consider it acceptable to allow enabling/disabling the SDAP header during a handover.

The eNB may change the presence of the SDAP header only by means of a handover, i.e., a synchronized reconfiguration.

## Configuration with AS-reflective QoS

RAN2-AH in June agreed that the UE follows the latest flow to DRB mapping, whether it is configured by RRC or via AS reflective QoS mechanism. In order to provide possibility for gNB to have unambiguous behaviour, it is important that gNB may enable/disable AS reflective QoS with RRC configuration. Having possibility to disable AS-reflective QoS is useful when rapid changes to the flow to DRB mapping are not expected. This is, traditionally, for example when UE is configured with eMBB bearer and IMS voice bearer.

AS reflective QoS may be disabled in the case where, for instance gNB performs HO and it is preferred that the flow to DRB mapping of UE corresponds with the RRC configuration. In the email discussion on L2 parameters it was discussed whether the use of AS reflective QoS could be derived from the presence of the (DL) SDAP header. However, the SDAP header (QFI) is also necessary if only NAS reflective QoS is to be used but AS reflective QoS disabled. Hence, the configuration of AS reflective QoS cannot just depend on the configuration of the SDAP header. Configurations of the SDAP header with NAS-reflective QoS are discussed in [2].

When AS reflective QoS is enabled, SDAP header is always present.

In the case where AS reflective QFI to DRB mapping is not removed, it may be enabled/disabled explicitly via RRCConnectionReconfiguration, i.e., it may be disabled even if the DL SDAP header is present.

# Conclusion

In section 2 we made the following observations:

Observation 1	When PDU session is split between MgNB and SgNB, same PDU session ID is used

Observation 2	PDU session ID is send to UE in SDAP-config

Observation 3	When AS reflective QoS is enabled, SDAP header is always present.

Based on the discussion in section 2 we propose the following:

Proposal 1	DRB IDs are unique inside one UE

Proposal 2	Existence SDAP header is configured separately for UL and for DL

Proposal 3	Presence of the SDAP header is not coupled with the enabling and disabling AS reflective QoS.

Proposal 4	SDAP header is configured for a DRB by means of RRC. Depending on UL and DL configuration, the QFI is present in all UL and/or DL packets on that DRB (not dynamically enabled/disabled).

Proposal 5	The eNB may change the presence of the SDAP header only by means of a handover, i.e., a synchronized reconfiguration.

Proposal 6	In the case where AS reflective QFI to DRB mapping is not removed, it may be enabled/disabled explicitly via RRCConnectionReconfiguration, i.e., it may be disabled even if the DL SDAP header is present.

# References

R2-1800698, SDAP Header Format, Ericsson, Vancouver, Canada, 22nd – 26th January 2018

R2-1707652, LS reply on NAS Reflective QoS, SA2, Berlin, Germany, 21st – 25th August 2017.

# Appendix

RAN2#100 Agreed:

Agreements

=>	SDAP header remains fixed to 8 bits.   The details are FFS.

RAN2#99 agreed:

Agreements

1.	RAN2 aims at designing a 1 byte SDAP header.  Whether the QFI is 6 bit or 7 bits is FFS.

2.	If configured, SDAP header size for a DRB is static (assuming 1 byte header).  The QFI will always be present.

3. 	No SN will be introduced in SDAP

RAN2-97bis Spokane agreed:

Agreements 1 NR/NR DC should support that different QoS flows of the same PDU session can be mapped to MgNB and SgNB. 2 In the case of NR/NR DC where different QoS flows of the same PDU session are mapped to MgNB and SgNB then there is one SDAP entity in the MgNB and one in SgNB for that PDU session. RAN2 understand that support of this behaviour is still under discussion on SA2