3GPP TSG-RAN WG2 Meeting NR Adhoc #1801	R2-1800710

# Vancouver, Canada, 22nd – 26th January 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | XXXX | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of termology for Configured Scheduling and SPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2017-12-19 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Alignment of terminology for configured scheduling (CS), SPS, transmission and reception without dynamic scheduling, configured grant Type 1 and Type 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Configured Scheduling (CS) as new umbrella term - DL CS as term for downlink operation, using configured downlink assignments - UL CS Type 1 and UL CS Type 2, both using configured uplink grants - Type 1 and Type2 as currently defined - UL CS confirmation MAC CE - CS-RNTI for both DL CS and UL CS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned terminology remains in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2; 5.3.1; 5.4.1; 5.4.3.1; 5.8; 5.9. 5.15; 6.1.3.7; 6.2.1; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSR			Buffer Status Report

BWP			Bandwidth Part

CE			Control Element

CS			Configured Scheduling

LCG			Logical Channel Group

MCG			Master Cell Group

PHR			Power Headroom Report

PTAG			Primary Timing Advance Group

SCG			Secondary Cell Group

SI			System Information

SpCell			Special Cell

SR			Scheduling Request

SS			Synchronization Signals

SSB			Synchronization Signal Block

STAG			Secondary Timing Advance Group

SUL			Supplementary Uplink

TAG			Timing Advance Group

TPC-CS-RNTI			Transmit Power Control-Configured Scheduling-RNTI

TPC-PUCCH-RNTI	Transmit Power Control-Physical Uplink Control Channel-RNTI

TPC-PUSCH-RNTI	Transmit Power Control-Physical Uplink Shared Channel-RNTI

TPC-SRS-RNTI			Transmit Power Control-Sounding Reference Symbols-RNTI

Next Change

## 5.3	DL-SCH data transfer

### 5.3.1	DL Assignment reception

Downlink assignments transmitted on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity’s C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity’s C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity’s CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this serving cell on the PDCCH for the MAC entity’s CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this serving cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate DL CS deactivation:

4>	clear the configured downlink assignment for this serving cell (if any);

4>	if the timeAlignmentTimer associated with the PTAG is running:

5>	indicate a positive acknowledgement for the DL CS deactivation to the physical layer.

3>	else if PDCCH content indicates DL CS activation:

4>	store the downlink assignment for this serving cell and the associated HARQ information as configured downlink assignment;

4>	initialise (if not active) or re-initialise (if already active) the configured downlink assignment for this serving cell to start in the associated PDSCH duration and to recur according to rules in subclause 5.8.1;

4>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	indicate the presence of a configured downlink assignment for this serving cell and deliver the stored HARQ information to the HARQ entity.

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

1>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

1>	consider the NDI bit to have been toggled;

1>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments, the HARQ Process ID associated with this slot is derived from the following equation:

HARQ Process ID = [floor(CURRENT_slot * 10 / (numberOfSlotsPerFrame * semiPersistSchedIntervalDL))] modulo numberOfConfCS-Processes

where CURRENT_slot=[(SFN * numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

Next Change

## 5.4	UL-SCH data transfer

### 5.4.1	UL Grant reception

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, or configured semi-persistently by RRC. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity’s C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity’s C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity’s CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this serving cell on the PDCCH for the MAC entity’s CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	stop the ConfiguredGrantTimer for the corresponding HARQ process, if running;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate UL CS  deactivation:

4>	trigger UL CS confirmation.

3>	else if PDCCH contents indicate UL CS activation:

4>	trigger UL CS confirmation;

4>	store the uplink grant for this serving cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this serving cell to start in the associated PUSCH duration and to recur according to rules in subclause 5.8.2;

4>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	stop the ConfiguredGrantTimer for the corresponding HARQ process, if running;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

1>	if the ConfiguredGrantTimer for the corresponding HARQ process is not running:

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

NOTE 1:	For the same serving cell, an uplink grant addressed to C-RNTI shall override a configured uplink grant in case of overlap in time domain.

For configured uplink grants, the HARQ Process ID associated with this symbol is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo numberOfConfGrant-Processes

where CURRENT_symbol=(SFN * numberOfSlotsPerFrame * numberOfSymbolsPerSlot + slot number in the frame * numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

NOTE 2:	CURRENT_symbol refers to the symbol index of the first transmission of a repetition bundle that takes place.

Next Change

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical channel prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	lcp-allowedSCS which sets the allowed Subcarrier Spacing(s) for transmission;

-	lcp-maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	lcp-configuredGrantType1Allowed which sets whether a UL CS Type 1 can be used for transmission;

-	lcp-allowedServingCells which sets the allowed cell(s) for transmission.

The MAC entity shall maintain a variable Bj for each logical channel j. Bj shall be initialized to zero when the related logical channel is established, and incremented before every instance of the LCP procedure by the product PBR × T, where PBR is Prioritized Bit Rate of logical channel j and T is the time elapsed since Bj was last updated. The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP. However, the value of Bj can never exceed the bucket size and if the value of Bj is larger than the bucket size of logical channel j, it shall be set to the bucket size. The bucket size of a logical channel is equal to PBR × BSD.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different serving cells), it is up to UE implementation in which order the grants are processed.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in lcp-allowedSCS, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	lcp-maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	lcp-configuredGrantType1Allowed, if configured, is set to TRUE in case the UL grant is a UL CS Type 1; and

2>	lcp-allowedServingCells, if configured, includes the Cell information associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration and Cell information are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in subclause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to "infinity", the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

NOTE:	The value of Bj can be negative.

2>	if any resources remain, all the logical channels selected in subclause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

The UE shall also follow the rules below during the scheduling procedures above:

- 	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given an UL grant size that is equal to or larger than 8 bytes while having data available for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied:

-	the MAC entity is configured with skipUplinkTxDynamic and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and

-	the MAC PDU includes zero MAC SDUs; and

-	the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI or data from UL-CCCH;

-	MAC CE for CS confirmation;

-	MAC CE for BSR, with exception of BSR included for padding;

-	MAC CE for single entry PHR or multiple entry PHR;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for BSR included for padding.

#### 5.4.3.2	Multiplexing of MAC Control Elements and MAC SDUs

The MAC entity shall multiplex MAC CEs and MAC SDUs in a MAC PDU according to subclauses 5.4.3.1 and 6.1.2.

Next Change

## 5.8	Configured Scheduling (CS)

### 5.8.1	Downlink

Configured Scheduling (CS) is configured by RRC per serving cell and per BWP. Multiple configurations can be active simultaneously only on different serving cells. Activation and deactivation of the DL CS are independent among the Serving Cells.

For the DL CS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating CS activation or deactivation.

RRC configures the following parameters when CS is configured:

-	cs-RNTI: CS-RNTI for both activation, deactivation, and retransmission;

-	semiPersistSchedIntervalDL: Interval of CS.

When CS is released by upper layers, all the corresponding configurations shall be released.

After a downlink assignment is configured for CS, the MAC entity shall consider sequentially that the Nth assignment occurs in the slot for which:

(numberOfSlotsPerFrame * SFN + slot number in the frame) =
[(numberOfSlotsPerFrame * SFNstart time + slotstart time) + N * semiPersistSchedIntervalDL * numberOfSlotsPerFrame / 10] modulo 1024

where SFNstart time and slotstart time are the SFN and slot, respectively, at the time the configured downlink assignment were (re-)initialised.

### 5.8.2	Uplink

There are two types of UL CS:

-	UL CS Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	UL CS Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating UL CS activation or deactivation.

Type 1 and Type 2 are configured by RRC per serving cell and per BWP. Multiple configurations can be active simultaneously only on different serving cells. For Type 2, activation and deactivation are independent among the serving cells. For the same serving cell, the MAC entity is configured with either Type 1 or Type 2.

RRC configures the following parameters when the UL CS Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	periodicity: periodicity of UL CS Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN=0 in time domain;

-	numberOfConfGrant-Processes: the number of HARQ processes.

RRC configures the following parameters when UL CS Type 2 is configured:

-	cs-RNTI: CS-RNTI for both activation, deactivation, and retransmission;

-	periodicity: periodicity of UL CS Type 2;

-	numberOfConfGrant-Processes: the number of HARQ processes.

Upon configuration of a UL CS Type 1 for a serving cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated serving cell;

1>	initialise (if not active) or re-initialise (if already active) the configured uplink grant to start in the symbol according to timeDomainOffset and to reoccur with periodicity.

After an uplink grant is configured for UL CS Type 1, the MAC entity shall consider sequentially that the Nth uplink grant occurs associated with the symbol for which:

[(SFN * numberOfSlotsPerFrame * numberOfSymbolsPerSlot) + (slot number in the frame * numberOfSymbolsPerSlot) + symbol number in the slot] = (timeDomainOffset + N * periodicity) modulo 1024

After an uplink grant is configured for a UL CS Type 2, the MAC entity shall consider sequentially that the Nth uplink grant occurs associated with the symbol for which:

[(SFN * numberOfSlotsPerFrame * numberOfSymbolsPerSlot) + (slot number in the frame * numberOfSymbolsPerSlot) + symbol number in the slot] = [(SFNstart time * numberOfSlotsPerFrame * numberOfSymbolsPerSlot + slotstart time * numberOfSymbolsPerSlot + symbolstart time) + N * periodicity] modulo 1024

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, at the time the configured uplink grant was (re-)initialised.

When UL CS is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared immediately.

The MAC entity shall:

1>	if the UL CS confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	instruct the Multiplexing and Assembly procedure to generate an UL CS Confirmation MAC CE as defined in subclause 6.1.3.7;

2>	cancel the triggered UL CS confirmation.

For UL CS Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of CS Confirmation MAC CE triggered by the UL CS deactivation.

Retransmissions except for repetition of configured grants use uplink grants addressed to CS-RNTI.

Next Change

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in subclause 6.1.3.9;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry.

The MAC entity shall for each configured SCell:

1>	if an SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	activate the SCell; i.e. apply normal SCell operation including:

3>	SRS transmissions on the SCell;

3>	CQI/PMI/RI/CRI reporting for the SCell;

3>	PDCCH monitoring on the SCell;

3>	PDCCH monitoring for the SCell;

3>	PUCCH transmissions on the SCell, if configured.

2>	start or restart the sCellDeactivationTimer associated with the SCell in the slot when the SCell Activation/Deactivation MAC CE was received;

2>	(re-)initialize any suspended configured uplink grants of UL CS Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2;

2>	trigger PHR according to subclause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires:

2>	deactivate the SCell;

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CQI/PMI/RI/CRI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

Next Change

## 5.15	Bandwidth Part (BWP) operation

A Serving Cell may be configured with at most four BWPs, and for an activated Serving Cell, there is always one active BWP at any point in time.

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time, and is controlled by the PDCCH indicating a downlink assignment or an uplink grant. Upon addition of SpCell or activation of an SCell, one BWP is initially active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

On the active BWP for each activated Serving Cell configured with a BWP, the MAC entity shall apply normal operations including:

1>	transmit on UL-SCH;

1>	transmit on RACH;

1>	monitor the PDCCH;

1>	transmit PUCCH;

1>	receive DL-SCH;

1>	(re-)initialize any suspended configured uplink grants of UL CS Type 1 according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

On the inactive BWP for each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	not transmit on UL-SCH;

1>	not transmit on RACH;

1>	not monitor the PDCCH;

1>	not transmit PUCCH;

1>	not receive DL-SCH;

1>	clear any DL CS assignment and configured uplink grant of UL CS Type 2;

1>	suspend any configured uplink grant of UL CS Type 1.

Upon initiation of the Random Access procedure, the MAC entity shall:

1>	if PRACH resources are configured for the active UL BWP:

2>	perform the Random Access procedure on the active DL BWP and UL BWP.

1>	else (i.e. PRACH resources are not configured for the active UL BWP):

2>	switch to initial DL BWP and UL BWP;

2>	perform the Random Access procedure on the initial DL BWP and UL BWP.

If the MAC entity receives a PDCCH for BWP switching while a Random Access procedure is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching. If the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure on the new activated BWP. If the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the active BWP.

If BWP-InactivityTimer is configured, the MAC entity shall for each activated Serving Cell:

1>	if the Default-DL-BWP is configured, and the active DL BWP is not the BWP indicated by the Default-DL-BWP; or

1>	if the Default-DL-BWP is not configured, and the active DL BWP is not the initial BWP:

2>	if a PDCCH indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH for BWP switching is received on the active DL BWP, and the MAC entity switches the active BWP:

3>	start or restart the BWP-InactivityTimer associated with the active DL BWP.

2>	if Random Access procedure is initiated:

3>	stop the BWP-InactivityTimer.

2>	if BWP-InactivityTimer associated with the active DL BWP expires:

3>	if the Default-DL-BWP is configured:

4>	perform BWP switching to a BWP indicated by the Default-DL-BWP.

3>	else:

4>	perform BWP switching to the initial DL BWP.

Next Change

### 6.1.3	MAC Control Elements (CEs)

[…]

#### 6.1.3.7	UL CS Confirmation MAC CE

The UL CS Confirmation MAC CE is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size of zero bits.

[…]

Next Change

## 6.2	Formats and parameters

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to zero.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH |
| 000001–100000 | Identity of the logical channel |
| 100001–110111 | Reserved |
| 111000 | Duplication Activation/Deactivation |
| 111001 | SCell Activation/Deactivation (4 octet) |
| 111010 | SCell Activation/Deactivation (1 octet) |
| 111011 | Long DRX Command |
| 111100 | DRX Command |
| 111101 | Timing Advance Command |
| 111110 | UE Contention Resolution Identity |
| 111111 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH |
| 000001–100000 | Identity of the logical channel |
| 100001–110110 | Reserved |
| 110111 | UL CS Confirmation |
| 111000 | Multiple Entry PHR |
| 111001 | Single Entry PHR |
| 111010 | C-RNTI |
| 111011 | Short Truncated BSR |
| 111100 | Long Truncated BSR |
| 111101 | Short BSR |
| 111110 | Long BSR |
| 111111 | Padding |