3GPP TSG-RAN WG2 NR Ad hoc 1801	R2-1800965

Vancouver, Canada, 22 – 26 January 2018	Resubmission of R2-1713381

Agenda item:	10.3.1.4.3

Source:	Nokia, NTT DOCOMO, Nokia Shanghai Bell

Title:	Clarification on the Preamble group B selection

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In the previous RAN2#99Bis meeting the following was agreed wrt. RA preamble group selection:

Agreements:

- As in LTE, preamble group selection can be based on Msg3 size and pathloss in NR.

- The parameters numberOfRA-Preambles, sizeOfRA-PreamblesGroupA, messageSizeGroupA in LTE should be reused in NR

- The parameters numberOfRA-Preambles, sizeOfRA-PreamblesGroupA, messageSizeGroupA are conveyed via NR SIB1.

This contribution seeks to clarify that the first agreement highlighted allows Preamble group B selection also based on only the size of Msg3, as in LTE.

# 2	Discussion on LTE baseline

LTE procedure for the Random Access Preamble group selection reads like follows [TS 36.321]:

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	For BL UEs or UEs in enhanced coverage, if Random Access Preamble group B does not exist, select the Random Access Preambles group corresponding to the selected enhanced coverage level.

-	For NB-IoT, randomly select one of the PRACH resources corresponding to the selected enhanced coverage level according to the configured probability distribution, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	Except for BL UEs or UEs in enhanced coverage in case preamble group B does not exist, or except for NB-IoT UEs, if Msg3 has not yet been transmitted, the MAC entity shall:

-	if Random Access Preambles group B exists and any of the following events occur:

-	the potential message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is greater than messageSizeGroupA and the pathloss is less than PCMAX,c (of the Serving Cell performing the Random Access Procedure) – preambleInitialReceivedTargetPower – deltaPreambleMsg3 – messagePowerOffsetGroupB;

-	the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC header is greater than messageSizeGroupA;

-	select the Random Access Preambles group B;

-	else:

-	select the Random Access Preambles group A.

Since the CCCH SDU size cannot be changed upon reception of UL grant via RAR (due to TM mode RLC entity not supporting segmentation), LTE does not specify UE behaviour where the CCCH SDU could not fit to the given UL grant. After the addition of the Resume procedure in Rel-13, the RRCConnectionResumeRequest had the option of being 64 bits long unlike the RRCConnectionRequest which is 48 bits. To support still a case of providing 56 bits UL grants for UEs establishing RRC connection through RRCConnectionRequest procedure, the above highlighted Preamble group B selection based on solely on the CCCH SDU size was added.

Observation: LTE baseline supports Preamble group B selection based on solely on the CCCH SDU / Msg3 size.

# 3	Conclusions

Since NR will also support RRC_INACTIVE state and hence the connection resume procedure, like LTE, on top of the connection establishment procedure from RRC_IDLE, similar mechanism for Preamble group B selection should be specified from the beginning.

Proposal 1: Confirm the LTE baseline includes the Preamble group B selection also based on solely on the CCCH SDU / Msg3 size and it is supported in NR as well.

Related CR into TS 38.321 is provided in [1] and in an Annex below.

# References

[1]	R2-1800966, Draft CR to 38.321 on Correction on Preamble group B selection, Nokia, NTT DOCOMO, Nokia Shanghai Bell

# Annex: TP into TS 38.321 [1]

*** Start of change ***

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if the Random Access procedure was initiated by a beam failure indication from lower layer; and

1>	if the contention free PRACH resources for beam failure recovery request associated with any of the SS blocks and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SS blocks with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS blocks or the CSI-RSs with CSI-RSRP above csirs-dedicatedRACH-Threshold amongst the associated CSI-RSs is available:

2>	select an SS block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS blocks or a CSI-RS with CSI-RSRP above csirs-dedicatedRACH-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by either PDCCH or RRC; and

1>	if the ra-PreambleIndex is not 0b000000; and

1>	if contention free PRACH resource associated with SS blocks or CSI-RS have not been explicitly provided by RRC:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex.

1>	else if the contention free PRACH resources associated with SS blocks have been explicitly provided by RRC and at least one SS block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS blocks is available:

2>	select an SS block with SS-RSRP above rsrp-ThresholdSSB amongst the associated SS blocks;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block.

1>	else if the contention free PRACH resources associated with CSI-RSs have been explicitly provided by RRC and at least one CSI-RS with CSI-RSRP above csirs-dedicatedRACH-Threshold amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above csirs-dedicatedRACH-Threshold amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else:

2>	select a SS block with SS-RSRP above rsrp-ThresholdSSB;

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B exists and any of the following events occur:

4>	the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – ra-PreambleInitialReceivedTargetPower;

4>	the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:

5>	select the Random Access Preambles group B.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SS blocks is configured:

3>	select a ra-PreambleIndex randomly with equal probability from the random access preambles associated with the selected SS block and the selected group.

2>	else:

3>	select a ra-PreambleIndex randomly with equal probability from the random access preambles within the selected group.

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex.

1>	if an SS block is selected above and an association between PRACH occasions and SS blocks is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SS block.

1>	else if a CSI-RS is selected above and an association between PRACH occasions and CSI-RSs is configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected CSI-RS.

1>	else:

2>	determine the next available PRACH occasion.

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

*** End of change ***