3GPP TSG-RAN2 NR Ad hoc 0118	R2-1801222

Vancouver, Canada, 22nd January – 26th January 2018

Agenda Item:	10.2.14

Source: 	Huawei, HiSilicon

Title: 	Consideration on self evaluation of IMT-2020 for UP latency

Document for:	Discussion

# Introduction

At RAN#77 meeting, the preliminary work plan of self evaluation was agreed in [1], where a three-step plan is made. At RAN#78 meeting, it was agreed in [2] that 3GPP will start Step 2 activity for self evaluation study, which includes the performance evaluation against the eMBB, mMTC and URLLC technical performance requirements as defined in Report ITU-R M.2410 (see [3]).

The evaluation of these requirements would require technical expertise from RAN WGs including RAN2. To well progress the study, a work split on the evaluation of the technical performance requirements is proposed in [2], and it was agreed that RAN WG2 needs to take care of the technical performance requirements including user plane latency. Therefore, this document is submitted to RAN2 for consideration.

In this document, the evaluation approaches and results of NR user plane latency are provided.

# Discussion

## Metric definition and requirements

According to Report ITU-R M.2410, user plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the eMBB and URLLC usage scenarios.

The minimum requirements for user plane latency are

–	4 ms for eMBB

–	1 ms for URLLC

assuming unloaded conditions (i.e., a single user) for small IP packets (e.g., 0 byte payload + IP header), for both downlink and uplink.

Table 1. The requirements of user plane latency

|  | ITU-R requirements |
| --- | --- |
| DL user plane latency for eMBB (ms) | 4 |
| UL user plane latency for eMBB (ms) | 4 |
| DL user plane latency for URLLC (ms) | 1 |
| UL user plane latency for URLLC (ms) | 1 |

## User plane latency calculation

From the definition we can see that user plane latency might be different due to different frame structure and numerology, which are expected to imapct the scheduling delay and the possible retrnasmission opportunities. Therefore, in the following we consider the user plane latnecy for FDD and TDD, respectively, based on the evaluation method defined in Report ITU-R M.2412 (see [4]).

User plane latency of FDD

The NR User plane latency (one way) for a scheduled UE consists of the fixed node processing delays (which includes radio frame alignment) and TTI duration for FDD. The UE processing time has been agreed in RAN1#90bis for different configurations, and in our analysis we assume that BS processing time is the same as that of UE’s, e.g., K OFDM symbols. This assumption is for the sake of initial evaluation. Other BS processing time assumptions can be evaluated.

If the slot alignment is S (in unit of non-slot), the number of OFDM symbols per non-slot is M, and the number of HARQ round trip time is H (in unit of non-slot), then the average one-way latency can be calculated as below.

where p is the error probability of the first HARQ retransmission.

Then we can give the following user plane latency results for NR FDD, by assuming that

- The UE processing time is from front-loaded DMRS only configuration in capability#2, i.e., K=4 OFDM symbols for 15kHz SCS and K=6 OFDM symbols for 30kHz SCS;

- and S is 0.5 for the average alignment delay.

- H is derived according to the length of slot and processing time, which might be different for SCS=15 kHz or 30 kHz.

Table 7. User plane latency results for NR FDD

| User plan latency (ms) |  |  | 15 kHz SCS | 30 kHz SCS |
| --- | --- | --- | --- | --- |
| DL | M=2 (2OS non-slot) | p=0 | 0.79 | 0.54 |
|  |  | p=0.1 | 0.93 | 0.64 |
|  | M =4 (4OS non-slot) | p=0 | 1.00 | 0.64 |
|  |  | p=0.1 | 1.17 | 0.76 |
|  | M =14 (14OS slot) | p=0 | 2.07 | 1.18 |
|  |  | p=0.1 | 2.47 | 1.38 |
| UL | M=2 (2OS non-slot) | p=0 | 0.79 | 0.54 |
|  |  | p=0.1 | 0.93 | 0.64 |
|  | M =4 (4OS non-slot) | p=0 | 1.00 | 0.64 |
|  |  | p=0.1 | 1.17 | 0.76 |
|  | M =14 (14OS slot) | p=0 | 2.07 | 1.18 |
|  |  | p=0.1 | 2.47 | 1.38 |

Observation 1: NR FDD can fulfil the user plane latency requirement of IMT-2020. Specifically,

- The configurations of M=2OS non-slot with 15kHz/30kHz SCS, and M=4OS non-slot with 30kHz SCS can meet URLLC target of 1ms.

- The configurations of M=14 OS slot with 15kHz/30kHz SCS and M=4OS non-slot with 15kHz SCS can meet eMBB target of 4ms.

User plane latency of TDD

The NR U-plane one way latency for a scheduled UE consists of the fixed node processing delays, radio frame alignment and TTI duration for TDD. In the following, we give the user plane latency results for NR TDD assuming a DL-UL pattern through , by assuming that

- The UE processing time is from front-loaded DMRS only configuration in capability#2, i.e., K=4 OFDM symbols for 15kHz SCS and K=6 OFDM symbols for 30kHz SCS;

- and S is 1 for slot alignment.

- H is derived according to the length of slot and processing time, as well as the DL/UL configuration.

Table 8. User plane latency results for NR TDD with DL-UL pattern

| User plan latency (ms) |  |  | 15kHz SCS | 30kHz SCS |
| --- | --- | --- | --- | --- |
| DL | M=2 (2OS non-slot) | p=0 | 0.86 | 0.57 |
|  |  | p=0.1 | 1.00 | 0.67 |
|  | M =4 (4OS non-slot) | p=0 | 1.14 | 0.71 |
|  |  | p=0.1 | 1.31 | 0.86 |
|  | M =14 (14OS slot) | p=0 | 2.57 | 1.43 |
|  |  | p=0.1 | 3.17 | 1.73 |
| UL | M=2 (2OS non-slot) | p=0 | 0.86 | 0.57 |
|  |  | p=0.1 | 1.00 | 0.67 |
|  | M =4 (4OS non-slot) | p=0 | 1.14 | 0.71 |
|  |  | p=0.1 | 1.31 | 0.86 |
|  | M =14 (14OS slot) | p=0 | 2.57 | 1.43 |
|  |  | p=0.1 | 3.17 | 1.73 |

Observation 2: NR TDD can fulfil the user plane latency requirement of IMT-2020. Specifically, (with M=2OS non-slot with 15kHz/30kHz SCS, or M=4OS non-slot with 30kHz SCS).

- The configurations of M=2OS non-slot with 15kHz/30kHz SCS, and M=4OS non-slot with 30kHz SCS can meet URLLC target of 1ms.

- The configurations of M=14 OS slot with 15kHz/30kHz SCS and M=4OS slot with 15kHz SCS can meet eMBB target of 4ms.

# Conclusion

In this document, we discussed NR performance in terms of user plane latency. The following observations are made.

Observation 1: NR FDD can fulfil the user plane latency requirement of IMT-2020. Specifically,

- The configurations of M=2OS non-slot with 15kHz/30kHz SCS, and M=4OS non-slot with 30kHz SCS can meet URLLC target of 1ms.

- The configurations of M=14 OS slot with 15kHz/30kHz SCS and M=4OS non-slot with 15kHz SCS can meet eMBB target of 4ms.

Observation 2: NR TDD can fulfil the user plane latency requirement of IMT-2020. Specifically, (with M=2OS non-slot with 15kHz/30kHz SCS, or M=4OS non-slot with 30kHz SCS).

- The configurations of M=2OS non-slot with 15kHz/30kHz SCS, and M=4OS non-slot with 30kHz SCS can meet URLLC target of 1ms.

- The configurations of M=14 OS slot with 15kHz/30kHz SCS and M=4OS slot with 15kHz SCS can meet eMBB target of 4ms.

# Reference

- RP-172101, “WF on Work plan of Self Evaluation SI”, Huawei, Ericsson, Telecom Italia, September 2017.

- RT-172532, “Work plan for self evaluation towards 3GPP Update Submission to ITU-R”, Huawei, Ericsson, Telecom Italia, December 2017.

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.