TSG-RAN WG2 NR Ad hoc 1801	R2-1801422

Vancouver, Canada, 22nd January – 26th January 2018

Agenda Item: 	10.4.5.2

Source:	Apple

Title:	Cell Reselection in INACTIVE State

Document for:		Discussion

# 1	Introduction

Though UE in INACTIVE state would perform similar cell reselection procedure as in IDLE state, still some difference remains. This contribution gives our thoughts on the special handling/rule for cell reselection procedure in INACTIVE state.

# 2	Discussion

Inactive state was introduced in NR in order to quickly resume the UE connection in Uu interface for data transmission, which reduces both signaling overhead and data transmission delay.

Figure-1 gives one deployment example: F1 and F2 are deployed in the same area, and some cells in F1 belong to one RNA. In this example, one INACTIVE UE moves from cell#5 to cell#3 via the overlapped area of cell#7 and cell#2. With the current cell reselection rule in IDLE, there are two possible cases for the UE mobility trace in INACTIVE state:

- Case 1: cell#5 -> cell#7 -> cell#3

- Case 2: cell#5 -> cell#2-> cell#3

Figure-1. Example for the network RNA deployment

Since cell#7 is not in the same RNA for the UE, the following problems would occur in case 1 but not in case 2:

- Twice RNA update procedures would be triggered if UE first camps on cell#7 and then cell#3. This would lead to unnecessary RRC state transitions and signaling overhead.

- If UE context is unavailable in cell#7, UE connection has to be established from IDLE. This would lead to unnecessary RRC state transitions and signaling overhead.

- If data transmission occurs when UE camps on cell#7, data transmission delay would be increased if cell#7 cannot fetch UE context.

All the problems above are against the motivation of INACTIVE state. To address these problems, case 2 should better be avoided by making INACTIVE UE camp on cell#2 which is within the same RNA as long as cell#2 is suitable.

Observation 1: To avoid unnecessary RRC state transitions and corresponding signaling overhead, inactive UE should better reselect a target cell within the same RNA to camp as long as the cell is suitable.

Proposal 1: During cell reselection in INACTIVE state, it is suggested that target cells and/or frequencies within the same RNA should be prioritized.

In current LTE cell reselection rule, there are two priority related parameters: 1/ priority of frequency and 2/ frequeny-offset and cell-offset. Table 1 presents the cell reselection based on frequency priority. Furtherly, frequency-offset and cell-offset are applied in cell ranking criterion R.

Table-1. Cell reselection based on frequency priority

To reflect the RNA level priority in cell reselection, both RNA level frequency priority and RNA level frequency-offset/cell-offset can be considered.

Proposal 2: If current parameters of cell reselection cannot reflect the RNA level priority appropriately, RNA specific priority could be considered, e.g. RNA level frequency priority, RNA level frequency-offset/cell-offset for cell ranking R-criterion.

# 3	Conclusion

According to the anlaysis in section 2, we proposal that:

Proposal 1: During cell reselection in INACTIVE state, it is suggested that target cells and/or frequencies within the same RNA should be prioritized.

Proposal 2: If current parameters of cell reselection cannot reflect the RNA level priority appropriately, RNA specific priority could be considered, e.g. RNA level frequency priority, RNA level frequency-offset/cell-offset for cell ranking R-criterion.