3GPP TSG-RAN WG2 NR Ad hoc 1801	R2-1801589

Vancouver, Canada, 22 – 26 January 2018

Agenda item:	10.4.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Release and setup of EN-DC (offline discussion #04)

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

The offline discussion #03 was created during RAN2 NR-AH#3 with the following scope:

R2-1800837	Setup, modification and release of EN-DC [N.054, N.056, N.057, N.214, N.218]	Nokia, Nokia Shanghai Bell	discussion	Rel-15	38.331	NR_newRAT-Core	Late

=>	Make clear in the spec that the DRB configuration is not dependent on the container that it is received in.

=>	ServCellIndex index needs to be added to the serving cell configuration in 38.331. Check the release of the PSCell is covered.

=>	Further offline discussion to conclude whether to add in 36.331 a release/setup structure to endc-CellGroupConfig-r15 (as shown in the document). If added then the scg-ConfigReleaseNR-r15 field would have to renamed to be something more specific to the release and add in a single message use case. How to release the tdm-PatternSingle-Tx-r15 needs to be considered as well. (Offline discussion #04, Nokia). Outcome in R2-1801589

In this document we present the result of the offline discussion and resulting ASN.1 structure proposed to be included in LTE RRC.

# 2	Current structure of EN-DC configuration within LTE RRCConnectionReconfiguration

The current structure of EN-DC configuration is shown below for reference:

RRCConnectionReconfiguration-v15x0-IEs ::= SEQUENCE {

endc-Config-r15				SEQUENCE {

scg-ConfigReleaseNR-r15				BOOLEAN,

sk-Counter-r15						INTEGER (0.. 65535)				OPTIONAL,	-- Need ON

nr-SecondaryCellGroupConfig-r15		OCTET STRING					OPTIONAL,	-- Need ON

nr-RadioBearerConfig-r15			OCTET STRING					OPTIONAL,	-- Need ON

nr-RadioBearerConfigS-r15			OCTET STRING					OPTIONAL,	-- Need ON

tdm-PatternSingle-Tx-r15			SEQUENCE {

subframeAssignment-r15				SubframeAssignment-r15,

harq-Offset-r15						INTEGER (0.. 9)				OPTIONAL	-- Need ON

}																	OPTIONAL	-- Need ON

}																		OPTIONAL	-- Need ON	nonCriticalExtension			SEQUENCE {}								OPTIONAL

}

# 3	Proposed structure of EN-DC configuration within LTE RRCConnectionReconfiguration

The discussion was quite varied, and the following attempts to capture the main point expressed by various companies:

- The name of the configuration field should be generic, so it is proposed to rename endc-Config-r15 to nr-Config-r15.

- The fields nr-RadioBearerConfig1 and nr-RadioBearerConfig2 were moved out of the nr-Config since they can be configured also separately.

- The sk-Counter was clarified to be a one-shot field and moved out of nr-Config.

- Since the TDM pattern is independent of the SCG configuration and may be released separately, it was moved out of the main nr-Config and release-setup was added to allow release.

- The name of the field indicating the release and add of SCG was changed to scg-ReleaseAndAdd.

- Missing field description was added for the field subframeAssignment.

Based on these, the proposed ASN.1 structure and modified field descriptions are shown in Annex A (with track changes):

Proposal 1: Adopt the TP as shown in Annex A.

# Annex A: Proposed ASN.1 structure and field descriptions

RRCConnectionReconfiguration-v15x0-IEs ::= SEQUENCE {

nr-Config-r15				CHOICE {

release								NULL,

setup								SEQUENCE {

scg-ReleaseAndAdd-r15				ENUMERATED {true}			OPTIONAL,	-- Need ON

sk-Counter-r15						INTEGER (0.. 65535)				OPTIONAL,	-- Need ON

nr-SecondaryCellGroupConfig-r15			OCTET STRING					OPTIONAL,	-- Need ON

}

} 																		OPTIONAL,	-- Need ON

sk-Counter-r15						INTEGER (0..65535)					OPTIONAL,	-- Need ON

nr-RadioBearerConfig1-r15			OCTET STRING						OPTIONAL,	-- Need ON

nr-RadioBearerConfig2-r15			OCTET STRING						OPTIONAL,	-- Need ON

tdm-PatternSingleTx-r15				SEQUENCE {

release								NULL,

setup								SEQUENCE {

subframeAssignment-r15				SubframeAssignment-r15,

harq-Offset-r15						INTEGER (0..9)

}

}																		OPTIONAL,	-- Need ON

nonCriticalExtension			SEQUENCE {}								OPTIONAL

}

harq-Offset Indicates a HARQ subframe offset that is applied to the subframes designated as UL in the associated subrame assignment.

nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The UE field includes the DRB configuration of RBs configured with NR PDCP.

nr-SecondaryCellGroupConfig Includes the NR RRCReconfiguration message as specified in TS 38.331 [82]. In this version of the specification, the NR RRC message only includes fields secondaryCellGroupToAddModList and measConfig.

scg-ReleaseAndAdd A one-shot field indicating whether the UE simultaneously releases and adds all the SCG related configuration within nr-Config. (NOTE). I.e. the configuration set by the NR RRCReconfiguration message (e.g. secondaryCellGroupToAddModList and measConfig).

sk-Counter A one-shot counter used upon initial configuration of security for EN-DC as well as upon refresh of S-KgNB. Network provides this field upon establishment of first bearer utilizing SN-terminated bearer using S-KgNB.

tdm-PatternSingleTx Indicates the subframes during which a single transmission UE configured with EN-DC is allowed to transmit.

subframeAssignment Indicates DL/UL subframe configuration where sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [21, table 4.2-2].

NOTE: 	The NR SCG configuration does not include the RB configuration (as configured by nr-RadioBearerConfig1 and nr-RadioBearerConfig2).