3GPP TSG-RAN WG2 NR Ad hoc 1801	R2-1801648

Vancouver, Canada, 22 – 26 January 2018

Agenda item:	10.4.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Cell Group Configuration (offline discussion #03)

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

The offline discussion #03 was created during RAN2 NR-AH#3 with the following scope:

R2-1800406	Consideration on the configuration of cell group (RILNo Z081)	ZTE, Sanechips	discussion	Rel-15	NR_newRAT-Core

=>	The ASN.1 will include configuration of 1 single SCG.

=>	Progress the structure offline . Can use the proposal from section 5 of R2-1800837 as a starting point. (Offline discussion #03, Nokia)

In this document we present the result of the offline discussion for agreement to LTE RRC.

# 2	Proposed ASN.1 structure for EN-DC cell group config

The following points were raised during the discussion:

- Minimum change would be to remove only MCG from the structure (so it can be added later on)

- The name of the single cell group IE should be generic

- Alternative would be to have an entirely separate message for the EN-DC reconfigurations and NR RRCReconfigurations (used with stand-alone NR operation). However, this requires more changes since the procedures and the text have to corrected.

The proposed structure resulting from the discussion is shown below:

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

rat-Config							CHOICE {

secondaryNodeConfig					SecondaryNodeConfig,

...

},

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE{}						OPTIONAL

}

SecondaryNodeConfig ::= SEQUENCE {

-- Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP.

-- In In EN-DC this field may only be present if the RRCReconfiguration

-- is transmitted over SRB3.

radioBearerConfig						RadioBearerConfig 				OPTIONAL, -- Need M

-- Configuration of SCG for MR-DC:

secondaryCellGroup						CellGroupConfig					OPTIONAL, -- Need M

-- Measurement configuration MR-DC:

measConfig								MeasConfig						OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

nonCriticalExtension					SEQUENCE{}						OPTIONAL

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

Proposal 1: Adopt the ASN.1 structure for EN-DC CG as shown in this document

In addition to this, the procedures need to be updated to reflect the above structure. Since the contained fields haven't really changed, the only changes needed are for the cases referring to the content of the RRCReconfiguration. However, it seems no real changes are needed since apart from secondaryCellGroupList, the field names haven't changed at all, so the changes can be easily incorporated.