3GPP TSG-RAN WG2 NR AH#3	R2-1801650

Vancouver, Canada, 22nd – 26th January 2018

Title:	DRAFT LS on supporting power sharing

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN1

Cc:                              RAN4

Contact Person:

#### Name:	Yang Zhao

E-mail Address:	zhaoyang[AT] huawei [DOT] com

1	Overall description

RAN2 would like to inform that the signaling support of power sharing has been agreed in RAN2 as follows:

- P_LTE (max power within LTE) and P_NR (max power within NR) can be configured to the UE (via LTE and NR respectively).

2:	UE capability for support of dynamic power sharing is contained within the MR-DC container

3:	UE capability for support of LTE TDM pattern is added to LTE UE capabilities. The support of this UE capability is conditionally mandatory for UEs that do not support dynamic power sharing, and for UEs that indicate single UL for any BC, and optional otherwise.

In addition RAN2 would like to indicate that RAN2 does not see the need to explicitly signal the SUO mode or dual mode to the UE side. It is assumed that:

- by default the UE would operate in dual mode

- if LTE TDM pattern (SUO case 1) is configured to the UE, the UE would justify whether it enters to SUO mode based on the condition that P_LTE + P_NR > Pcmax

- there is no need to differentiate SUO case 2 with dual mode

2	Actions

ACTION to RAN1:

RAN2 respectfully requests RAN1 to take the above into account and in addition to update the PHY specification accordingly.

3	Date of Next RAN2 Meetings:

RAN2#101	26 February - 2 March 2018	Athens, Greece