3GPP TSG-RAN WG2 AH-1807 	R2-1809519

# Montreal, Canada, July 2nd - July 6th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0202 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for X value (CCCH size for LCID allocation) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN1 agreed that both 56bits and 72bits Msg3 size (48bits and 64bits CCCH size) can be supported in NR from a coverage perspective. Therefore, CCCH size of 64 bits should fit a 72-bit UL grant with new assigned LCID 100001 without L field in MAC subheader. And no specific optimization should be foreseen for 48-bit CCCH size. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set x = 64 bits instead of 48 bits in TS38.321 Sections 6.1.2. and 6.2.1. Impact analysis Impacted functionality: MAC subheader for CCCH. Inter-operability: - If the UE implements this CR and the network does not, the network may not parse the Msg3 content. - If the network implements this CR and the UE does not, there will be an interoperability problem that the network may not parse the Msg3 content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 64-bit CCCH MAC SDU must carry the L field, thus resulting in a total Msg3 size of 80 bits exceeding the RAN1-guaranteed coverage for Msg3 payload size. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2. and 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

6.1.2	MAC PDU (DL-SCH and UL-SCH except transparent MAC and Random Access Response)

A MAC PDU consists of one or more MAC subPDUs. Each MAC subPDU consists of one of the following:

-	A MAC subheader only (including padding);

-	A MAC subheader and a MAC SDU;

-	A MAC subheader and a MAC CE;

-	A MAC subheader and padding.

The MAC SDUs are of variable sizes.

Each MAC subheader corresponds to either a MAC SDU, a MAC CE, or padding.

A MAC subheader except for fixed sized MAC CE, padding, and a MAC SDU of CCCH of size 64 bits consists of the four header fields R/F/LCID/L. A MAC subheader for fixed sized MAC CE, padding, and a MAC SDU of CCCH of size 64 bits consists of the two header fields R/LCID.

Figure 6.1.2-1: R/F/LCID/L MAC subheader with 8-bit L field

Figure 6.1.2-2: R/F/LCID/L MAC subheader with 16-bit L field

Figure 6.1.2-3: R/LCID MAC subheader

MAC CEs are placed together. DL MAC subPDU(s) with MAC CE(s) is placed before any MAC subPDU with MAC SDU and MAC subPDU with padding as depicted in Figure 6.1.2-4. UL MAC subPDU(s) with MAC CE(s) is placed after all the MAC subPDU(s) with MAC SDU and before the MAC subPDU with padding in the MAC PDU as depicted in Figure 6.1.2-5. The size of padding can be zero.

Figure 6.1.2-4: Example of a DL MAC PDU

Figure 6.1.2-5: Example of a UL MAC PDU

A maximum of one MAC PDU can be transmitted per TB per MAC entity.

End of the first change

Start of the second change

6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs and padding. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to zero.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH |
| 000001–100000 | Identity of the logical channel |
| 100001–101110 | Reserved |
| 101111 | Recommended bit rate |
| 110000 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 110001 | PUCCH spatial relation Activation/Deactivation |
| 110010 | SP SRS Activation/Deactivation |
| 110011 | SP CSI reporting on PUCCH Activation/Deactivation |
| 110100 | TCI State Indication for UE-specific PDCCH |
| 110101 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 110110 | Aperiodic CSI Trigger State Subselection |
| 110111 | SP CSI-RS / CSI-IM Resource Set Activation/Deactivation |
| 111000 | Duplication Activation/Deactivation |
| 111001 | SCell Activation/Deactivation (four octet) |
| 111010 | SCell Activation/Deactivation (one octet) |
| 111011 | Long DRX Command |
| 111100 | DRX Command |
| 111101 | Timing Advance Command |
| 111110 | UE Contention Resolution Identity |
| 111111 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 000000 | CCCH of size other than 64 bits |
| 000001–100000 | Identity of the logical channel |
| 100001 | CCCH of size 64 bits |
| 100010–110100 | Reserved |
| 110101 | Recommended bit rate query |
| 110110 | Multiple Entry PHR (four octet Ci) |
| 110111 | Configured Grant Confirmation |
| 111000 | Multiple Entry PHR (one octet Ci) |
| 111001 | Single Entry PHR |
| 111010 | C-RNTI |
| 111011 | Short Truncated BSR |
| 111100 | Long Truncated BSR |
| 111101 | Short BSR |
| 111110 | Long BSR |
| 111111 | Padding |

End of the second change