3GPP TSG-RAN2 NR AH#0218	R2-1809549

Montreal, Canada, July 2-6, 2018

Agenda item:         10.4.1.3.6

Source:		Mediatek Inc.

Title:  	   Further Clarification on RRCReject in Connection Resume

Document for:	Discussion

Introduction

In this contribution, we clarify the UE action on T380, SRB1 suspension and security handling upon receiving RRCReject message when performing RRC Connection resume from INACTIVE mode.

Discussion

T380 is used to control the periodic RNAU. RRC connection resume procedure is initiated with the following cases:

- Upper layer request

- Respond to RAN-paging

- RNA update: periodic RNA update or move to a cell not belonging to the RNA

If the RRC connection resume procedure is successful, UE will either transferred to CONNECTED or sent to INACTIVE by RRCRelease message with SuspendConfig.  T380 is started when UE is sent to INACTIVE either from INACTIVE (RNAU) or from CONNECTED. It is kept running when UE is in INACTIVE mode. When UE initiates the RRC connection resume procedure, it’s possible that T380 is still running if the procedure is initiated by upper layer or responds to RAN-paging. When T380 expires, periodic RNAU is initiated. So the timer T380 is stopped if it is running upon initiation of RRC resume procedure.

If the RRC connection resume procedure is unsuccessful and rejected by the network, T380 is not running, because the timer has been stopped upon initiation of RRC connection resume. The consequence is that UE will not perform periodic RNAU in INACTIVE then once the resume request is rejected.  However, UE should continue to perform periodic RNAU even if it is rejected to INACTIVE. Therefore, T380 should be started upon reception of RRCReject message in RRC resume procedure.

One problem is which value can be used for T380 when it is started. The original timer value can be reused.  Or a default value can be specified if T380 is started due to reception of RRCReject message.

Proposal 1: T380 is started.

When UE initiates RRC connection resume procedure and transmits RRCResume message, UE restores the PDCP state and re-establish PDCP entities for SRB1. SRB1 is resumed. Since the RRCReject message is intended to send UE to INACTIVE, the resumed SRB1 should be suspended, similar as UE actions upon reception of RRCRelease message with SuspendConfig.

Proposal 2: SRB1 is suspended.

When UE initiates RRC connection resume procedure and transmits RRCResume message, UE restores the RRC configuration and security context from the stored UE AS context. When RRCReject message is received, RAN2 confirms that there is no update of the UE context, UE starts the wait timer and remains in RRC_INACTIVE. UE AS context should be kept, so that RRC connection can be resumed later when T302 expires. Since UE security context has been updated, UE need to temporarily store the previous security context, so that it can revert back upon reception of RRCReject message. For other UE AS context, the restored UE AS context can be considered as the current UE AS context, which includes the current RRC configuration, the PDCP state including ROHC state, SDAP configuration, C-RNTI, cellIdentity and PCI of the source PCell. Since there is no update for those configuration, UE can store current RRC configuration directly.

Proposal 3: UE reverts back to the previous security context.

Proposal 4: UE stores the current UE AS context including current RRC configuration, the PDCP state including ROHC state, SDAP configuration, C-RNTI, cellIdentity and PCI of the source PCell.

Conclusion

In this contribution, we clarify the UE action on T380, SRB1 suspension and security handling upon receiving RRCReject message when performing RRC connection resume from INACTIVE mode.

We propose upon reception of RRCReject message:

Proposal 1: T380 is started.

Proposal 2: SRB1 is suspended.

Proposal 3: UE reverts back to the previous security context.

Proposal 4: UE stores the current UE AS context including current RRC configuration, the PDCP state including ROHC state, SDAP configuration, C-RNTI, cellIdentity and PCI of the source PCell.

The CR is provided in the appendix.

Appendix

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further Clarification on RRCReject in Connection Resume |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to current RRC draft, T380 is not running if the RRC connection resume procedure is unsuccessful and rejected by the network, because the timer has been stopped upon initiation of RRC connection resume. The consequence is that UE will not perform periodic RNAU in INACTIVE. However, UE should continue to perform periodic RNAU even it is rejected to INACTIVE. So T380 should be started upon reception of RRCReject message. In current RRC draft, how to handle UE AS context upon reception of RRCReject message is not clear. When UE initiates RRC connection resume procedure and transmits RRCResume message, UE restores the RRC configuration and security context from the stored UE AS context. When RRCReject message is received, UE AS context should be kept, so that RRC connection can be resumed later. Since security context has been updated when transmitting RRCResumeRequest, UE need to temporarily store the previous security context, so that it can revert back upon reception of RRCReject message. In current RRC draft, how to handle SRB1 upon reception of RRCReject message is not clear. Since the RRCReject message is intended to send UE to INACTIVE, the SRB1 which is resumed when transmitting the RRCResumeRequest message should be suspended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon reception of RRCReject message: - Start T380; - Revert back to the pervious security context; - Store the current UE AS context except the security context; - Suspend SRB1; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE actions upon reception of RRCReject message is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.13.8	Reception of the RRCReject by the UE

The UE shall:

1>	stop timer T300, if runing;

1>	stop timer T319, if running;

1>	start timer T380;

1>	reset MAC and release the MAC configuration;

1> revert back to the pervious security context;

1> store the UE AS Context including the current RRC configuration, the PDCP state including ROHC state, SDAP configuration, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

1> suspend SRB1;

1>	start timer T302, with the timer value set to the waitTime;

1>	inform the upper layer that access barring is applicable;

1>	if RRCReject is sent in response to an RRCResumeResquest triggered by upper layers:

2>	inform upper layers about the failure to resume the RRC connection, upon which the procedure ends;

1>	if RRCReject is sent in response to an RRCResumeRequest triggered by RRC:

2>	consider the RRCResumeRequest pending i.e. initiate the Resume procedure again when barring is aleviated (when condition specified in 5.3.18a is fulfilled), upon which the procedure ends;

The UE shall continue to monitor RAN and CN paging while the timer T302 is running.

Editor’s Note: FFS Which access control related information is informed to higher layers.