3GPP TSG-RAN WG2#AH-1807	R2-1809687

# Montreal, Canada, 2nd – 6th July, 2018

Agenda Item:	10.4.1.4.5

Source:	Ericsson

Title:	[E233]: Autonomous gaps for CGI reporting

Document for:	Discussion, Decision

# Introduction

NR Automatic Neighbor Relations (ANR) is one key feature in NR Rel. 15 to support an efficient network rollout by automatically identifying neighboring cells via neighbor cell CGI reporting by UEs. Most RAN2 NR ANR discussions have converged, and in RAN2#102 in Busan, the following baseline was agreed for CGI reporting:

Agreements for ANR (reportCGI functionality in RRC)

1: For ANR, including Intra and Inter RAT cases, the following ANR configuration are supported:

-	Inter-RAT ANR towards NR configured by eNB

-	Intra-RAT ANR towards NR configured by gNB

-	Inter-RAT ANR towards LTE configured by gNB

2	 In case of EN-DC UE, ANR function towards NR cell can be configured by SN.

2i	The UE can only be configured with a single reportCGI configuration, from either MN or SN.

2ii	Configuration of ANR towards NR cell requires coordination between MN and SN

3: For ANR reporting, the CGI content includes:

-	a: PLMN list, TAC, frequency band list and CGI as baseline

-	b: RANAC is also reported, if included in SIB1

4:	In the case SIB1/RMSI is not broadcast, UE should report a notification to network. UE report includes:

-	a: “no SIB1 provided” indication

FFS whether UE should also report where CD-SSB of the measured SSB can be found

5:	In the case SIB1/RMSI is not broadcast, UE should report UE should report “no SIB1 provided indication” without waiting for T321 timer expiry and stop timer

6	In case of EN-DC, if reportCGI for NR cell is configured by eNB, then UE behavior follows inter-RAT ANR T321 value; if reportCGI for NR cell is configured by gNB, then UE behavior follows intra-RAT ANR T321 value. RAN2 sends an Ls to RAN4 to confirm RAN2 understanding

7	RAN2 to send an Ls to RAN4 on guidance for T321 values in the following ANR measurement cases:

-	UE served by LTE cell towards NR cell

-	UE served by NR cell towards NR cell

-	UE served by NR cell towards LTE cell

8	For UE capability for ANR towards NR cell:

-	a: DRX based reading of ANR towards NR cell related measurement should be supported

9	RAN2 to send an Ls to RAN4 to ask whether it is feasible to use autonomous gap

10	For ANR CGI reporting, only one NR neighbor cell configuration is support at a time

11:

a.	Introduce a UE capability bit in NR for Intra-RAT ANR (including inter and Intra frequency)

b.	Introduce a UE capability bit in NR for Inter-RAT ANR towards LTE cell.

c.	Introduce a UE capability bit in LTE for Inter-RAT ANR towards NR cell.

FFS Whether 2 separate capability bits are needed for LTE with and without EN-DC configured.

12: For ANR support, RAN2 sees no need to differentiate between FR1 and FR2. RAN2 to send an Ls to RAN4 to ask for opinion

FFS where cellsForWhichToReportCGI is added (measID or reportConfig)

In this contribution, we discuss the need to support autonomous gaps to retrieve the neighbor cell CGI. That relates to an ASN.1 review issue E323.

# Discussion

Much of the operations costs associated to network management is associated to manual labor, in particular when deploying and rolling out a new network. One key part remaining is to ensure support for the UE retrieval of the neighbor cell CGI. In LTE, autonomous gaps were introduced as an efficient means for CGI retrieval, where the UE is allowed to identify which resources it needs to monitor for CGI retrieval and skip serving cell UL and DL allocations during those resources to prioritize CGI retrieval.

- Observation 1	In LTE, autonomous gaps is an efficient means for ensuring gaps for the UE when retrieving neighbor cell CGI

Furthermore, autonomous gaps can be used for retrieving CGI from an LTE neighbor cell as well as an NR intra- or inter-frequency neighbor cell.

- Observation 2	Autonomous gaps can be used for retrieving CGI from an LTE neighbor cell as well as an NR intra- or inter-frequency neighbor cell.

Therefore, we propose to support autonomous gaps to support CGI retrieval from neighbor cell.

- Proposal 1	Support autonomous gaps for LTE and NR intra-/inter-frequency neighbor cell CGI retrieval.

- Proposal 2	Send an LS based on the draft LS [1] to RAN4 about feasibility of and requirements for autonomous gaps for LTE and NR intra-/inter-frequency neighbor cell CGI retrieval

# Conclusion

- In this contribution, we discuss autonomous gaps for neighbor cell CGI retrieval. In particular, we have the following observations and proposals:

- Observation 1	In LTE, autonomous gaps is an efficient means for ensuring gaps for the UE when retrieving neighbor cell CGI

- Observation 2	Autonomous gaps can be used for retrieving CGI from an LTE neighbor cell as well as an NR intra- or inter-frequency neighbor cell.

- Proposal 1	Support autonomous gaps for LTE and NR intra-/inter-frequency neighbor cell CGI retrieval.

- Proposal 2	Send an LS to RAN4 about feasibility of and requirements for autonomous gaps for LTE and NR intra-/inter-frequency neighbor cell CGI retrieval

# References

[1]	R2-1809688 Draft LS to RAN4 on the need for autonomous gaps to support ANR