3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1809916

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.1.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN area update procedure text

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss how to capture RAN are update procedure text.

# 2	RAN area update procedure text

When UE receives SIB1 it may need to initiate RAN area update procedure if the new serving cell is not anymore belonging to the ran-NotificationAreaInfo as signaled in the release message.  This procedural text is missing from 38.331. Thre are few alternatives how to do this:

- Capture in very detailed manner UE behavior:

  - If cellist was signaled ran-NotificationAreaInfo

    - if SIB1 signalled PLMN-Identity+CellIdentity does not belong to the ran-NotificationAreaInfo start performing procedures as defined in 38.331 5.3.13.2 (initiation of resume request procedure)

  - if ranAreaConfigList was signaled ran-NotificationAreaInfo

    - if PLMNIdentity+TAC and RANNotificationAreaCode, if signaled as part of ran-NotificationAreaInfo are broadcasted by the serving cell, then start performing procedures as defined in 38.331 5.3.13.2 (initiation of resume request procedure)

- Just capture that UE will start resume request procedure if current cell does not belong to ran-NotificationArea

In our view there is no need to capture all the details of comparision of cellIdentity/Ran area codes as the UE behaviour seem to be self contained with ASN.1. Thus we propose to capture following text for procedural text of RAN area update procedure:

1>	if the UE is in RRC_INACTIVE state:

2>	If the serving cell does not belong to the ran-NotificationAreaInfo as signalled in the last RRCRelease message:

3> intiate RRC resume procedure as defined in sub-clause 5.3.13.2 with cause value rna-Update;

# 5	Conclusion

We propose to capture procedural text for starting ran area update procedure in simple manner as described in the Annex

# Annex TP

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN:

2>	in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding cellAccessRelatedInfo containing the selected PLMN;

1>	forward the cellIdentity to upper layers;

1>	forward the trackingAreaCode to upper layers;

1>	forward the ims-EmergencySupport to upper layers, if present;

1>	forward the eCallOverIMS-Support to upper layers, if present;

1>	apply the configuration included in the servingCellConfigCommon;

1>	if the UE has a stored valid version of a required SIB in accordance with sub-clause 5.2.2.2.1:

2>	use that stored version of the SIB;

1>	if the UE has not stored the valid version of one or several required SIB(s), in accordance with sub-clause 5.2.2.2.1:

2>	for the SI message(s) containing the required SIB(s) according to the the si-SchedulingInfo in the SIB1 :

3> if indicated as currently not broadcast;

4> trigger a request to acquire the SI message(s) (if needed) as defined in sub-clause 5.2.2.3.3.

3>	else;

4> acquire the SI message(s) (if needed) as defined in sub-clause 5.2.2.3.2.

1>	if the UE is in RRC_INACTIVE state:

2>	If the serving cell does not belong to the ran-NotificationAreaInfo as signalled in the last RRCRelease message:

3> intiate RRC resume procedure as defined in sub-clause 5.3.13.2 with cause value rna-Update;

Editor’s Note: To be further updated when content of the SIB1 has been completed.