3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810085

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.1.4.3

Source:	Ericsson

Title:	UL carrier selection in PDCCH order

Document for:	Discussion, Decision

# Introduction

At the previous RAN2#102 meeting, the question of whether a PDCCH order to initiate CFRA should leave the UE with the freedom to select which UL carrier to use. We further discuss this topic in this contribution.

# Discussion

In the previous RAN2#102 meeting two contributions, [1] and [2], discussed if the PDCCH order should indicate UL carrier. In the previous RAN1#93 it was agreed, see [3], that the PDCCH order to initiate RA should contain a 1 bit SUL indicator to distinguish if the preamble should be transmitted on the SUL or NUL. The same logic is used when CFRA resources are configured using RRC in the RACH-ConfigfDedicated IE in ReconfigurationWithSync for HO.

Both RRC configuration of CFRA resources and PDCCH order to initiate CFRA indicate if the SUL or NUL is used.

We believe that when the gNB sends a PDCCH order to a UE to initiate CFRA it will have sufficient knowledge of the channel conditions to decide which UL carrier should be used. In any event, if the gNB has outdated information or is unsure if the channel conditions on NUL is sufficient, it can always choose the SUL carrier since it can be assumed to have better channel conditions.  In addition to this, the gNB has the knowledge to determine how to distribute load on the carriers.

To conclude, we propose:

The PDCCH order for Random Access agreed by RAN1 is sufficient and requires no updates to 38.321.

# Conclusion

In section 2 we made the following observations:

Observation 1	Both RRC configuration of CFRA resources and PDCCH order to initiate CFRA indicate if the SUL or NUL is used.

Based on the discussion in section 2 we propose the following:

Proposal 1	The PDCCH order for Random Access agreed by RAN1 is sufficient and requires no updates to 38.321.

# References

R2-1806999, CFRA resources allocation in the cell with UL and SUL, CATT, 3GPP TSG-RAN WG2#102, 21th – 25th May 2018

R2-1808554, PDCCH order on both SUL and NUL, LG Electronics Inc., 3GPP TSG-RAN WG2 Meeting #102, 21th – 25th May 2018

RAN1 Chairman’s Notes, TSG RAN WG1 Meeting #93, May 21st – 25th, 2018