3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810144

# Montreal, Canada, 2nd - 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSSAI format (N011) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR is based on TS38.331 baseline that is being used in ASN.1 process (SA ASN.1 review 5.3 Connection Control). SA2 agreed Network Slicing support as mandatory functionality. It requires the UE to provide NSSAI in RRCSetupComplete, based on provision the information by NAS). The current message allows conveying the NSSAI in a form of BIT STRING, requiring the entire length needs to be conveyed always. This is strenghten requirement, as the actual content of the paramater remains flexible in a way that not required to fullfil 32 bits always: TS38.300 defines that NSSAI is the list of S-NSSAIs, and the UE should have the ability to signal each S-NSSAI with mandatory SST field and optionally included SD field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding finer contents of NSSAI in RRCSetupComplete (Slice assistance information coding in ASN.1 ) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE provision of slice assistance information remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Modified Subclause

### 6.3.4	Other information elements

#### –	S-NSSAI

The IE S-NSSAI (Single Network Slice Selection Assistance Information) identifies a Network Slice end to end and comprises a slice/service type and a slice differentiator, see TS 23.003 [20].

S-NSSAI information element

-- ASN1START

-- TAG-S-NSSAI-START

S-NSSAI ::=						 SEQUENCE {

sst							BIT STRING (SIZE (8)),

sd							BIT STRING (SIZE (16))		OPTIONAL

}

-- TAG-S-NSSAI-START

-- ASN1STOP

S-NSSAI field descriptions

Sst Slice/Service Type, see TS 23.003 [20].

Sd Slice Differentiator, see TS 23.003 [20].

End of Modified Subclause