3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810175

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	Clarification of RLC poll handling

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#102, it was agreed the following:

R2 assumes that UE implementation can set poll bit for the last PDU correctly, and the cases when this would not happen would be very rare.

However, there are cases which the UE will simply not be able to handle by UE implementation without breaking the specifications so a way forward needs to be agreed.

# 2	Discussion

When the UE is configured with data duplication, there may be situations in which the last RLC SDU in the buffer is discarded and all the previous PDU are under transmission.

In data duplication, when the PDCP entity receives a successful delivery indication from one of the RLC entities, PDCP indicates to the second RLC entity to discard the duplicated PDCP PDU.

Here may thus be situations in which the last RLC PDU waiting to be transmitted in one of the RLC entities is discarded due to an indication from higher layers. In this case this may lead to that the poll bit cannot be set. Similar situation may happen when duplication is deactivated. All duplicated RLC PDUs must be discarded and no additional PDUs must be transmitted. In such case, if the UE had not set the poll bit in the last transmitted PDUs, a poll bit will not be transmitted.

These situations will be common when duplication is used. Other cases were also identified in the contributions submitted in RAN2#102. For example, in Dual Connectivity Split bearers, when the transmission on the RLC entities are changed (for uplink transmission this is triggered by RRC signaling), this results in that no more data would be available for the old RLC entity. Since the RLC does not know when this decision is taken, the poll might not be set. Similar situation may appear when the splitting threshold is configured and buffered data changes from more than the threshold to less than the threshold. Yet, this is not known to the RLC. Since no more data is available for the secondary associated RLC, the last RLC PDU might not have set the poll bit.

While RAN2 agreed to leave it up to the UE implementation; however, these situations cannot be easily resolved by the UE. RAN2 assumes that these cases would be very rare, however, PDCP duplication deactivation with discarding on RLC i.e. discarding the last PDU in the buffer, is not rare, on the contrary, given the dynamic usage of MAC CEs for duplication activation/deactivation, this should rather be considered a typical scenario. Therefore, it is preferable for the network to have a consistent specified UE behavior. For useful MAC CE usage for duplication control, when duplication is deactivated, the secondary RLC should finalize its retransmissions in order to be able to transfer new RLC PDUs right from the start when another MAC CE is received to activate duplication again. Therefore, an RLC status report should have been polled and retransmissions triggered and finalized. I.e. the RLC state should be concluded/finalized before duplication is re-initialized. Otherwise after duplication re-activation, new duplicates would suffer an unnecessary delay before they could be delivered.

The simplest approach would be to retransmit the last RLC PDU and include the poll bit. Even if the last RLC PDU had been received by the RLC entity, the retransmitted RLC PDU including the poll bit set will trigger the status report.

The last transmitted RLC PDU shall be re-transmitted with a poll bit in cases when no more data is available for transmission and the RLC buffer is empty.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The last transmitted RLC PDU shall be re-transmitted with a poll bit in cases when no more data is available for transmission and the RLC buffer is empty.