3GPP TSG-RAN WG2 NR Ad hoc 1807	R2-1810288

Montreal, Canada, 2-6 July 2018	Revision of R2-1808171

Agenda item:	11.3

Source: 	Huawei, HiSilicon

Title: 	Consideration and evaluation results for IMT-2020 for user plane latency

Document for:	Discussion and Decision

# Introduction

In this document, the evaluation method and evaluation results of NR user plane latency are provided. This document is updated based on R2-1808171. Further analysis on parameters assumptions for each step of DL and UL procedure are proposed. And the updated results of UP latency are given with PDSCH/PUSCH mapping Type A and Type B. The cases of NR FDD, TDD, and TDD+SUL are investigated.

# Discussion

## Metric definition and requirements

According to Report ITU-R M.2410 (see [1]), user plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the eMBB and URLLC usage scenarios.

The minimum requirements for user plane latency are

–	4 ms for eMBB

–	1 ms for URLLC

assuming unloaded conditions (i.e., a single user) for small IP packets (e.g., 0 byte payload + IP header), for both downlink and uplink.

Table 1 the requirements of user plane latency

| Technical performance requirement | Values |
| --- | --- |
| DL/UL user plane latency for eMBB (ms) | 4 |
| DL/UL user plane latency for URLLC (ms) | 1 |

## User plane latency evaluation

Based on the definition and the evaluation method provided in Report ITU-R M.2412 [2], the components and values of user plane latency for DL and UL are listed in Table 2 & 3.

Table 2 UL user plane procedure

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | UL data transfer | T1 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx |  |
| 1.1 | UE processing delay | tUE,tx The time interval between the data is arrived, and packet is generated; | N2/2 OFDM symbols (The value of N2 is in Table 5) |
| 1.2 | UL Frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for next available UL slot (If the transmission duration is longer than the remaining available duration in the slot, then it is assumed that the transmitter needs to wait for the next slot.) | TFA + Twait, TFA is the frame alignment time within the current UL slot. Twait is the waiting time for next available UL slot if the current slot is not UL slot |
| 1.3 | TTI for UL data packet transmission | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | BS processing delay | tBS,rx The time interval between the PUSCH is received and the data is decoded; | N1/2 OFDM symbols (The value of N1 is in Table 4 with “No additional PDSCH DM-RS configured”; It is assumed that BS processing delay is equal to UE processing delay as for PDSCH) |
| 2 | HARQ retransmission | THARQ = T2 + T1 T2 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | BS processing delay | tBS,tx The time interval between the data is decoded, and ACK/NACK packet is generated | N1/2 OFDM symbols |
| 2.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot Twait is the waiting time for next available DL slot if the current slot is not DL slot; (It is reasonable to assume that the packet arrives at the starting of any OFDM symbol in a radio frame with uniform probability) |
| 2.3 | TTI for ACK/NACK transmission | tDL_duration | 1 OFDM symbols for PDCCH transmitting ACK/NACK |
| 2.4 | UE processing delay | tUE,rx The time interval between the ACK is received and the ACK is decoded. | N2/2 OFDM symbols |
| 2.5 | Repeat UL data transfer from 1.1 to 1.4 | T1 |  |
|  | Total one way user plane latency for UL | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  |

Table 3 DL user plane procedure

| ID | Component | Notations | Value |
| --- | --- | --- | --- |
| 1 | DL data transfer | T2 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx |  |
| 1.1 | BS processing delay | tBS,tx The time interval between the data is arrived, and packet is generated. | N1/2 OFDM symbols |
| 1.2 | DL Frame alignment (transmission alignment) | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot. Twait is the waiting time for next available DL slot if the current slot is not DL slot. |
| 1.3 | TTI for DL data packet transmission | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | UE processing delay | tUE,rx The time interval between the PDSCH is received and the data is decoded; | N1/2 OFDM symbols |
| 2 | HARQ retransmission | THARQ = T1 + T2 T1 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | UE processing delay | tUE,tx The time interval between the data is decoded, and ACK/NACK packet is generated. | N1/2 OFDM symbols |
| 2.2 | UL frame alignment (transmission alignment) | tFA,UL It includes frame alignment time, and the waiting time for the next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot. Twait is the waiting time for next available UL slot if the current slot is not UL slot. |
| 2.3 | TTI for ACK/NACK transmission | tUL_duration | For short PUCCH: 1 OFDM symbols; For long PUCCH: 7 OFDM symbols. |
| 2.4 | BS processing delay | tBS,rx The time interval between the ACK is received and the ACK is decoded. | N1/2 OFDM symbols |
| 2.5 | Repeat DL data transfer from 1.1 to 1.4 | T2 |  |
| - | Total one way user plane latency for DL | TUP= T2 + n×THARQ where n is the number of re-transmissions (n≥0) |  |

The use of these components in a BS-to-UE data transmission procedure is further illustrated in Figure 1.

Figure 1 Illustration of UP latency components

It is noted that the values of the above components are related to frame structure and numerology, UE capability on processing, as well as PDSCH/PUSCH mapping type. These impact factors are further subject to duplexing schemes like FDD, TDD and TDD+SUL. Therefore, in the following we consider the user plane latency for in terms of the above aspects.

### UE and BS processing time

The UE processing time, tUE, has been agreed in RAN1#90bis, known as UE capability 1 (PDSCH/PUSCH processing capability 1). For different configurations, the value of tUE is expressed in terms of OFDM symbols, say K OFDM symbols, as listed in Table 4 & 5 (see section 5.3 & 6.3 in [3]). The values under “No additional PDSCH DM-RS configured” are used.

Table 4 PDSCH processing time for PDSCH processing capability 1

|  | Sub-carrier spacing | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- | --- |
|  |  | No additional PDSCH DM-RS configured | Additional PDSCH DM-RS configured |
| 0 | 15 kHz | 8 | 13 |
| 1 | 30 kHz | 10 | 13 |
| 2 | 60 kHz | 17 | 20 |
| 3 | 120 kHz | 20 | 24 |

Table 5 PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

For this evaluation, The values of tBS,tx, tUE,rx, tUE,tx and tBS,rx are shown in Table 2 & 3.

### User plane latency of FDD

For NR FDD, the user plane latency is calculated according to Table 2 & 3. It is noted that in NR DL, the start and length indicator SLIV can be used to indicate the starting symbol relative to the start of the slot, and the number of consecutive symbols counting from the symbol allocated for the PDSCH. Similar mechanism is also introduced for PUSCH. Therefore, it is reasonable to assume that there is no alignment delay as once the data arrived at the transmitter is processed, it can be transmitted immediately. Therefore the frame alignment can be assumed to be 0 for NR FDD, i.e., tFA,DL = tFA,UL = 0.

If the number of OFDM symbols per non-slot is M, then tdata_duration=M; the average one-way latency can be calculated as below as an approximation. Note that K and M maybe different for DL and UL, or for data channel and control channel.

where p is the error probability of the first HARQ retransmission. Note that in this paper, grant free based UL transmission is considered in order to derive the lower bound of user plane latency performance.

The DL and UL user plane latency results for NR FDD are presented in Table 6 and Table 7, respectively, by assuming that UE processing time is from front-loaded DMRS only configuration [3].

Table 6 DL User plane latency results for NR FDD (UE capability 1)

| Slot / non-slot duration | Error probability | SCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz | 30kHz | 60 kHz |
|  |  | FDD | FDD | FDD |
| M=2 (2OS non-slot) | p=0 | 0.71 | 0.43 | 0.34 |
|  | p=0.1 | 0.86 | 0.51 | 0.41 |
| M =4 (4OS non-slot) | p=0 | 0.86 | 0.50 | 0.38 |
|  | p=0.1 | 1.03 | 0.60 | 0.45 |
| M =7 (7OS non-slot) | p=0 | 1.07 | 0.61 | 0.43 |
|  | p=0.1 | 1.29 | 0.73 | 0.51 |
| M =14 (14OS slot) | p=0 | 1.57 | 0.86 | 0.55 |
|  | p=0.1 | 1.89 | 1.03 | 0.66 |

Table 7 UL User plane latency results for NR FDD (UE capability 1)

| Slot / non-slot duration | Error probability | SCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15 kHz | 30kHz | 60 kHz |
|  |  | FDD | FDD | FDD |
| M=2 (2OS non-slot) | p=0 | 0.71 | 0.43 | 0.34 |
|  | p=0.1 | 0.86 | 0.51 | 0.41 |
| M =4 (4OS non-slot) | p=0 | 0.86 | 0.50 | 0.38 |
|  | p=0.1 | 1.03 | 0.60 | 0.45 |
| M =7 (7OS non-slot) | p=0 | 1.07 | 0.61 | 0.43 |
|  | p=0.1 | 1.29 | 0.73 | 0.51 |
| M =14 (14OS slot) | p=0 | 1.57 | 0.86 | 0.55 |
|  | p=0.1 | 1.89 | 1.03 | 0.66 |

Observation 1: NR FDD can fulfil user plane latency requirement of IMT-2020 with UE capability 1 under SCS of 15 kHz, 30 kHz, and 60 kHz, and larger SCS can help reducing latency.

### User plane latency of TDD with and without SUL

For NR TDD and TDD+SUL, the user plane latency is calculated according to Table 2 & 3. In the following, we give the user plane latency results with frame structure DL-DL-DL-DL-UL (i.e. “DDDSU”). This is for the consideration of the asymmetric traffic pattern in practical systems. Similar assumption of UE capability to NR FDD is assumed, i.e., the UE processing time is from front-loaded DMRS only configuration [3].

Resource allocation in time domain is taken into account for both DL and UL, i.e., PDSCH/PUSCH mapping Type A/B for combinations of valid starting symbol index relative to the start of the slot and the number of consecutive symbols. That is, if the evaluation is for 14 OFDM symbol length slot, then slot based scheduling is used. And the data transmission will require 14 OFDM symbols. And if the evaluation is for K OFDM symbol length non-slot, then non-slot based scheduling is used. And the data transmission will require K OFDM symbols.

When re-transmission occurs, the round trip time of HARQ should also be included, which is derived according to the processing time and specific DL/UL configuration.

For frame alignment,

- In case of TDD, the frame alignment delay for both DL and UL, tFA,DL and tFA,UL, might be larger than 0 due to the DL/UL configuration (The number of OFDM symbols required in the data transmission duration is less than the available remaining OFDM symbols in the slot, so the time needed to wait for the next available DL/UL slot/non-slot).

- The latency can be reduced by using TDD+SUL. In this case, the frame alignment delay of UL, tFA,UL, is reduced to approximately 0, which can reduce the latency (especially for UL) significantly.

- For Type A, the transmission can start immediately if it is currently in OFDM symbol #0~#3 (for PDSCH) or #0 (for PUSCH) of the slot. In this case, frame alignment time is 0. Otherwise, the frame alignment time equals to the number of OFDM symbols between the current OFDM symbol and OFDM symbol#0 of the next slot (note that waiting time is accounted separately, and therefore it should be “the next slot”, NOT the next available DL/UL slot).

- For Type B, the transmission can start immediately if it is currently in OFDM symbol#0~#12 (for PDSCH) or #0~#13 (for PUSCH) of the slot. In this case, frame alignment time is 0. Otherwise, the frame alignment time equals to the number of OFDM symbols between the current OFDM symbol and OFDM symbol#0 of the next slot (note that waiting time is accounted separately, and therefore it should be “the next slot”, NOT the next available DL/UL slot).

The user plane latency results for NR TDD and TDD+SUL are presented. For PDSCH/PUSCH mapping Type A, the DL and UL evaluation results are presented in Table 8 and Table 9, respectively. It is noted that PDSCH/PUSCH mapping Type A is more likely to be used for eMBB user terminals.

Table 8 DL user plane latency results for NR TDD (DL:UL=4:1) and SUL (Type A and UE capability 1)

| Slot / non-slot duration | Error probability | SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  | 30kHz |  | 60 kHz |  |
|  |  | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) |
| M=2 (2OS non-slot) | p=0 | 1.26 | 1.26 | 0.70 | 0.70 | 0.49 | 0.49 |
|  | p=0.1 | 1.63 | 1.42 | 0.90 | 0.80 | 0.60 | 0.56 |
| M =4 (4OS non-slot) | p=0 | 1.33 | 1.33 | 0.74 | 0.74 | 0.51 | 0.51 |
|  | p=0.1 | 1.71 | 1.50 | 0.93 | 0.84 | 0.63 | 0.58 |
| M =7 (7OS non-slot) | p=0 | 1.54 | 1.54 | 0.87 | 0.87 | 0.64 | 0.64 |
|  | p=0.1 | 1.92 | 1.74 | 1.07 | 0.98 | 0.78 | 0.73 |
| M =14 (14OS slot) | p=0 | 2.64 | 2.64 | 1.39 | 1.39 | 0.82 | 0.82 |
|  | p=0.1 | 3.19 | 2.98 | 1.67 | 1.61 | 0.98 | 0.97 |

Table 9 UL user plane latency results for NR TDD (DL:UL=4:1) and SUL (Type A and UE capability 1)

| Slot / non-slot duration | Error probability | SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  | 30kHz |  | 60 kHz |  |
|  |  | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) |
| M=2 (2OS non-slot) | p=0 | 3.32 | 1.32 | 1.73 | 0.99 | 1.04 | 0.86 |
|  | p=0.1 | 3.75 | 1.51 | 1.95 | 1.13 | 1.17 | 0.98 |
| M =4 (4OS non-slot) | p=0 | 3.40 | 1.39 | 1.77 | 1.05 | 1.06 | 0.91 |
|  | p=0.1 | 3.82 | 1.59 | 1.99 | 1.20 | 1.19 | 1.02 |
| M =7 (7OS non-slot) | p=0 | 3.61 | 1.61 | 1.88 | 1.23 | 1.12 | 1.02 |
|  | p=0.1 | 4.03 | 1.84 | 2.10 | 1.39 | 1.25 | 1.14 |
| M =14 (14OS slot) | p=0 | 4.10 | 2.11 | 2.13 | 1.66 | 1.24 | 1.22 |
|  | p=0.1 | 4.63 | 2.45 | 2.39 | 1.89 | 1.39 | 1.38 |

For PDSCH/PUSCH mapping Type B, the DL and UL evaluation results are presented in Table 10 and Table 11, respectively. It is noted that PDSCH/PUSCH mapping Type B is more likely to be used for URLLC user terminals.

Table 10 DL user plane latency results for NR TDD (DL:UL=4:1) and SUL (Type B and UE capability 1)

| Slot / non-slot duration | Error probability | SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  | 30kHz |  | 60 kHz |  |
|  |  | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) |
| M=2 (2OS non-slot) | p=0 | 0.91 | 0.91 | 0.55 | 0.55 | 0.42 | 0.42 |
|  | p=0.1 | 1.25 | 1.04 | 0.73 | 0.63 | 0.53 | 0.49 |
| M =4 (4OS non-slot) | p=0 | 1.11 | 1.11 | 0.65 | 0.65 | 0.48 | 0.48 |
|  | p=0.1 | 1.47 | 1.26 | 0.84 | 0.74 | 0.59 | 0.54 |
| M =7 (7OS non-slot) | p=0 | 1.44 | 1.44 | 0.82 | 0.82 | 0.59 | 0.59 |
|  | p=0.1 | 1.82 | 1.62 | 1.02 | 0.93 | 0.72 | 0.67 |
| M =14 (14OS slot) | p=0 | 2.64 | 2.64 | 1.39 | 1.39 | 0.82 | 0.82 |
|  | p=0.1 | 3.19 | 2.98 | 1.67 | 1.61 | 0.97 | 0.97 |

Table 11 UL user plane latency results for NR TDD (DL:UL=4:1) and SUL (Type B and UE capability 1)

| Slot / non-slot duration | Error probability | SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 15 kHz |  | 30kHz |  | 60 kHz |  |
|  |  | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) | TDD | TDD+ SUL (15kHz SCS) |
| M=2 (2OS non-slot) | p=0 | 2.36 | 0.79 | 1.25 | 0.52 | 0.80 | 0.49 |
|  | p=0.1 | 2.70 | 0.93 | 1.43 | 0.61 | 0.91 | 0.57 |
| M =4 (4OS non-slot) | p=0 | 2.73 | 0.96 | 1.44 | 0.67 | 0.90 | 0.63 |
|  | p=0.1 | 3.09 | 1.12 | 1.63 | 0.78 | 1.01 | 0.72 |
| M =7 (7OS non-slot) | p=0 | 3.13 | 1.25 | 1.63 | 0.93 | 1.00 | 0.83 |
|  | p=0.1 | 3.51 | 1.44 | 1.83 | 1.06 | 1.12 | 0.94 |
| M =14 (14OS slot) | p=0 | 4.11 | 2.11 | 2.12 | 1.66 | 1.24 | 1.22 |
|  | p=0.1 | 4.64 | 2.45 | 2.39 | 1.89 | 1.40 | 1.38 |

Observation 2: NR TDD with DL-DL-DL-DL-UL pattern can fulfil both DL and UL user plane latency requirement of IMT-2020 with UE capability 1 and PDSCH/PUSCH mapping type A/B under SCS of 30kHz, 60kHz.

Observation 3: NR TDD+SUL with DL-DL-DL-DL-UL pattern can fulfil both DL and UL user plane latency requirement of IMT-2020 with UE capability 1 and PDSCH/PUSCH mapping type A/B under SCS of 15kHz, 30kHz, 60kHz.

Observation 4: With PDSCH/PUSCH mapping type B, both DL and UL user plane latency can be reduced compared with PDSCH/PUSCH mapping type A.

Observation 5: TDD with SUL can reduce DL and UL latency compared to TDD only operation.

Proposal: Capture the evaluation results in Section 3 into TR37.910.

# Conclusion

In this document, we discussed NR performance in terms of user plane latency. The following observations and proposal are made.

Observation 1: NR FDD can fulfil user plane latency requirement of IMT-2020 with UE capability 1 under SCS of 15 kHz, 30 kHz, and 60 kHz, and larger SCS can help reducing latency.

Observation 2: NR TDD with DL-DL-DL-DL-UL pattern can fulfil both DL and UL user plane latency requirement of IMT-2020 with UE capability 1 and PDSCH/PUSCH mapping type A/B under SCS of 30 kHz, 60 kHz.

Observation 3: NR TDD+SUL with DL-DL-DL-DL-UL pattern can fulfil both DL and UL user plane latency requirement of IMT-2020 with UE capability 1 and PDSCH/PUSCH mapping type A/B under SCS of 15 kHz, 30 kHz, 60 kHz.

Observation 4: With PDSCH/PUSCH mapping type B, both DL and UL user plane latency can be reduced compared with PDSCH/PUSCH mapping type A.

Observation 5: TDD with SUL can reduce DL and UL latency compared to TDD only operation.

Proposal: Capture the evaluation results in Section 3 into TR37.910.

# References

- Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

- Report ITU-R M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”, Nov. 2017.

- 3GPP TS 38.214 V15.0.0, “NR; Physical layer procedures for data (Release 15)”, December, 2017.