3GPP TSG-RAN2 Meeting NR AH 1807	R2-1810457

# Montreal, Canada, 2nd Jul 2018 – 6th July 2018

Agenda Item:	10.3.1.2

Source: 	ITL

Title: 	BWP inactivity Timer upon initiation of RA Procedure on SCell

Document for:	Discussion and decision

Introduction

In RAN2#102 meeting, we agreed that there is no linkage between UL BWP and DL BWP of SCell [1].

RAN2#102 agreements

For RA procedure initiated on SCell (e.g. SCell X), RAR is received on SpCell. So,

a)	If RA resources are not configured for the active UL BWP of SCell X, UE needs to switch the UL BWP to initial UL BWP of SCell X. There is no need to switch the DL BWP of SCell X.

b)	If RA resources are configured for the active UL BWP of SCell X, the switching of DL BWP of SCell X based on linkage between UL BWP and DL BWP of SCell X is not needed.

In this contribution, we further discuss the UE behavior of BWP InactivityTimer by considering this agreement.

Discussion

In the current TS 38.321, the BWP switching for RACH is specified as below:

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell: 1> if PRACH occasions are not configured for the active UL BWP: 2> switch the active UL BWP to BWP indicated by initialUplinkBWP; 2> if the Serving Cell is a SpCell: 3>switch the active DL BWP to BWP indicated by initialDownlinkBWP; 1> else: 2> if the Serving Cell is a SpCell: 3> if the active DL BWP does not have the same bwp-Id as the active UL BWP: 4> switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP; 1> perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell .

From the above description, when the RA procedure is initiated on SCell, the UE doesn’t switch the active DL BWP of SCell (i.e., there is no linkage between UL BWP and DL BWP of SCell). This is feasible because the RA procedure on SCell is CFRA, and RAR is received on the active DL BWP of SpCell.

And, the bwp-InactivityTimer for RACH is specified as below:

If the bwp-InactivityTimer is configured, the MAC entity shall for each activated Serving Cell: 1> if the default-DL-BWP is configured, and the active DL BWP is not the BWP indicated by the default-DL-BWP; or 1> if the default-DL-BWP is not configured, and the active DL BWP is not the initial BWP: 2> if Random Access procedure is initiated on this Serving Cell: 3> stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell. 3> if the Serving Cell is SCell (other than PSCell): 4> stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

In the above description, when the RA procedure is initiated on SCell, the UE stops the bwp-InactivityTimer on both SCell and SpCell. The intention of this UE behaviour is to keep the active DL BWP of SCell and SpCell.

However, by taking into account the first parts description, the UE does not need to keep the active DL BWP of the SCell. Since the UE expects to receive the RAR on the active DL BWP of SpCell, there is no problem even if the active DL BWP of SCell is switched.

For example, even if the UE switches the active DL BWP to the default BWP while the UE is performing the RA procedure, the UE can receive the RAR on active DL BWP of SpCell. After the RA procedure is successfully completed, the gNB can indicate the UE to switch the active DL BWP of SCell and transmit/receive packet using the active DL BWP of SCell.

Therefore, we don’t think that the bwp-InactivityTimer on SCell needs to be stopped upon initiation of RACH procedure on SCell.

Proposal. For RA procedure initiated on SCell, there is no need to stop the bwp-InactivityTimer of SCell.

For the above proposal, a corresponding CR has been provided in [2]

Conclusion

In this contribution, we provide our view on the bwp-InactivityTimer of SCell. Based on the discussion, the following proposal is drawn:

Proposal. For RA procedure initiated on SCell, there is no need to stop the bwp-InactivityTimer of SCell.

Reference

3GPP RAN2#102 Chairman’s note

R2-1810462, Correction on BWP inactivity timer upon initiation of RA Procedure on SCell, ITL