3GPP TSG-RAN2 NR AH#0218	                             	R2-1810613

Montreal, Canada, July 2-6, 2018

Agenda item:         10.4.1.3.1.1

Source:		Mediatek Inc.

Title:  	   RRC Message Size Considering CSI-RS Related Configurations

Document for:	Discussion

Introduction

In this contribution, we calculate the RRC message size containing CSI-RS relevant configurations for RACH-ConfigDedicated, MeasObjectNR and CSI-MeasConfig.

Discussion

- RACH-ConfigDedicated

The size of rach-ConfigDedicated is almost 7KBytes, based on current signaling structure and the maximum values.

The size of the IE is mainly determined by two dimensions, i.e. csirs-ResourceList and ra-OccasionList.

maxRA-CSIRS-Resources 					INTEGER ::= 96

maxRA-OccasionsPerCSIRS					INTEGER ::= 64			-- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1						INTEGER ::=	511			-- Maximum number of RA occasions in the system

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIG-DEDICATED-START

-- FFS_Standlone: resources for msg1-based on-demand SI request

RACH-ConfigDedicated ::=		SEQUENCE {

cfra							CFRA																	OPTIONAL,	-- Need N

ra-Prioritization				RA-Prioritization																OPTIONAL,	-- Need N

...

}

CFRA	 ::=	 				SEQUENCE {

occasions						SEQUENCE {

rach-ConfigGeneric				RACH-ConfigGeneric,

ssb-perRACH-Occasion			ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}	OPTIONAL	-- Cond SSB-CFRA

}																												OPTIONAL,	-- Need S

resources						CHOICE {

ssb								SEQUENCE {

ssb-ResourceList				SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex		INTEGER (0..15)

},

csirs							SEQUENCE {

csirs-ResourceList				SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS			RSRP-Range

}

},

...

}

CFRA-SSB-Resource ::= 			SEQUENCE {

ssb								SSB-Index,

ra-PreambleIndex				INTEGER (0..63),

...

}

CFRA-CSIRS-Resource ::= 		SEQUENCE {

csi-RS							CSI-RS-Index,

ra-OccasionList					SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex				INTEGER (0..63),

...

}

-- TAG-RACH-CONFIG-DEDICATED-STOP

-- ASN1STOP

- CSI-RS-ResourceConfigMobility

The size of CSI-RS-ResourceConfigMobility is almost 1KBytes, based on current signaling structure and the maximum values.

The size of the IE is mainly determined by two dimensions, i.e. csi-RS-CellList-Mobility and csi-rs-ResourceList-Mobility.

maxNrofCSI-RS-CellsRRM 					INTEGER ::= 96 		-- Maximum number of FFS

maxNrofCSI-RS-ResourcesRRM				INTEGER ::= 96		-- Maximum number of CSI-RS resources for an RRM measurement object

CSI-RS-ResourceConfigMobility information element

-- ASN1START

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-START

CSI-RS-ResourceConfigMobility ::= 	SEQUENCE {

subcarrierSpacing					SubcarrierSpacing,

csi-RS-CellList-Mobility 			SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM))	OF CSI-RS-CellMobility,

...

}

CSI-RS-CellMobility ::=				SEQUENCE {

cellId								PhysCellId,

csi-rs-MeasurementBW				SEQUENCE {

nrofPRBs							ENUMERATED { size24, size48, size96, size192, size264},

startPRB							INTEGER(0..2169)

},

density								ENUMERATED {d1,d3}												OPTIONAL,

csi-rs-ResourceList-Mobility 		SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM))	OF CSI-RS-Resource-Mobility

}

CSI-RS-Resource-Mobility ::=		SEQUENCE {

csi-RS-Index						CSI-RS-Index,

slotConfig							CHOICE {

ms4									INTEGER (0..31),

ms5									INTEGER (0..39),

ms10								INTEGER (0..79),

ms20								INTEGER (0..159),

ms40								INTEGER (0..319)

},

associatedSSB						SEQUENCE {

ssb-Index							SSB-Index,

isQuasiColocated					BOOLEAN

}																										OPTIONAL, -- Need R

frequencyDomainAllocation			CHOICE {

row1								BIT STRING (SIZE (4)),

row2								BIT STRING (SIZE (12))

},

firstOFDMSymbolInTimeDomain			INTEGER (0..13),

sequenceGenerationConfig			INTEGER (0..1023),

...

}

CSI-RS-Index ::= 					INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1)

-- TAG-CSI-RS-RESOURCECONFIGMOBILITY-STOP

-- ASN1STOP

- MeasObjectNR

The size of MeasObjectNR is almost 3KBytes. If at maximum 64 MOs are considered, the maximum size for MO configuration would be up to 190KBytes.

maxNrofObjectId							INTEGER ::= 64		-- Maximum number of measurement objects

- CSI-MeasConfig

CSI-MeasConfig size is almost 150KBytes per carrier, based on current signaling structure and the maximum values.

In this IE, some fields have only one dimension, e.g. nzp-CSI-RS-ResourceToAddModList;

Some fields have two dimensions:

- E.g. nzp-CSI-RS-ResourceSetToAddModList. The size is determined by maxNrofNZP-CSI-RS-ResourceSets and maxNrofNZP-CSI-RS-ResourcesPerSet.

- E.g. csi-ResourceConfigToAddModList. The size is determined by maxNrofCSI-ResourceConfigurations and maxNrofNZP-CSI-RS-ResourceSetsPerConfig.

maxNrofCSI-ReportConfigurations			INTEGER ::= 48 		-- Maximum number of report configurations

maxNrofCSI-ResourceConfigurations		INTEGER ::= 112		-- Maximum number of resource configurations

maxNrofNZP-CSI-RS-Resources				INTEGER ::= 192		-- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofAP-CSI-RS-ResourcesPerSet		INTEGER ::= 16

maxNrofCSI-IM-Resources					INTEGER ::= 32		-- Maximum number of CSI-IM resources. See CSI-IM-ResourceMax in 38.214.

maxNrofCSI-IM-ResourceSets 				INTEGER ::=	64		-- Maximum number of NZP CSI-IM resources per cell

maxNrofCSI-SSB-ResourceSets 			INTEGER ::=	64		-- Maximum number of CSI SSB resource sets per cell

maxNrOfCSI-AperiodicTriggers			INTEGER ::= 128		-- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger	INTEGER ::= 16		-- Maximum number of report configurations per trigger state for aperiodic reporting

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEAS-CONFIG-START

CSI-MeasConfig ::=					SEQUENCE {

nzp-CSI-RS-ResourceToAddModList		SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource					OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId				OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet			OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList	SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId			OPTIONAL, -- Need N

csi-IM-ResourceToAddModList			SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource							OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId						OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet					OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId					OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet					OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddReleaseList	SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId				OPTIONAL, -- Need N

csi-ResourceConfigToAddModList 		SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig			OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList 	SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId			OPTIONAL, -- Need N

csi-ReportConfigToAddModList		SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig				OPTIONAL, -- Need N

csi-ReportConfigToReleaseList		SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId				OPTIONAL, -- Need N

reportTriggerSize					INTEGER (0..6)																			OPTIONAL,

aperiodicTriggerStateList			SetupRelease { CSI-AperiodicTriggerStateList }											OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList			SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }					OPTIONAL, -- Need M

...

}

-- TAG-CSI-MEAS-CONFIG-STOP

-- ASN1STOP

Based on current signalling structure and the maximum values, the size of the IEs related to CSI-RS are quite huge, which is up to hundreds of KBytess. Furthermore, the size will be scaled up if configuration for different carriers are required. The huge RRC message size not only introduce large amount of signaling overheads over the air interface, but also higher requirement on both UE process and memory.  It’s quite challenging to UE implementation to process the RRC messages having the size of hundreds of KBytes.

Observation 1: The size of the IEs relevant to CSI-RS grow up to hundreds of KBytes in NR.

Observation 2: The huge RRC message size produces large amount of signaling overhead and is challenging to UE processing and memory.

Proposal 1: RAN2 shall restrict NR RRC message size.

In order to reduce the RRC message size, one straightforward way is to reduce the maximum value. The total size of the IE can be reduced, because the length for each dimension is reduced.  Since the maximum value of those parameters are determined by RAN1, RAN1 needs to reconsider the maximum values and provide suggestions.

Proposal 2: Reduce the size by reducing the maximum values of the parameters.

If the maximum value for certain parameters can’t be reduced, another method is to restrict the total number of entries if the configurations have more than one dimensions. For example, for MO configuration, although the maximum number of CSI-RS resources per cell and the maximum number of cells per MO are both 96, the total number of CSI-RS resource can be configured per MO is 96 instead of 96*96. The restriction should be captured in the field description.

Proposal 3: If P2 cannot be done, for configuration with more than one dimensions, reduce the size by restricting the total number of entries.

Conclusion

Based on the following observations:

Observation 1: The size of the IEs relevant to CSI-RS grows up to hundreds of KBs in NR.

Observation 2: The huge RRC message size produces large amount of signaling overhead and is challenging to UE processing and memory.

We propose:

Proposal 1: RAN2 shall restrict NR RRC message size.

Proposal 2: Reduce the size by reducing the maximum values of the parameters.

Proposal 3: If P2 cannot be done, for configuration with more than one dimensions, reduce the size by restricting the total number of entries.