3GPP TSG-RAN WG2 NR AH#4			    R2-1810676

Montreal, Canada, 2nd – 6th July, 2018

Agenda item:	   11.1.2 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	Consideration on QoS impacts

Document for:	Discussion and Decision

1.	Introduction

According to TR 38.874, IAB (Integrated access and backhaul) architectures consist of one or more IAB nodes, which support wireless access to UEs and wirelessly backhauls the access traffic, and one or more IAB donors which provide UE’s interface to core network and wireless backhauling functionality to IAB nodes.

Figure 1: Reference diagram for IAB architectures

These IAB nodes and IAB donors support QoS enforcement by the scheduler on DL and UL on the wireless backhaul and access links. In this document, we are discussing on what factors affect the QoS enforcement in the IAB architectures.

2.	Discussion

In 5G QoS model, a QoS flow is the finest granularity of QoS differentiation in a PDU session which refers association between a UE and a data network that provides PDU connectivity services (e.g. operator services, Internet access or 3rd party services). Each QoS flow of the PDU session may require different packet forwarding treatment depending on the service requirements. For such different packet forwarding treatment, 5GC provides NG-RAN with 5G QoS characteristics and 5G QoS parameters for the QoS flows. Based on these 5G QoS characteristics and 5G QoS parameters, the NG-RAN configures one or more DRBs with the different QoS treatments (e.g. scheduling weights, link layer protocol configuration, etc.) so that QoS requirements for the QoS flows can be satisfied.

Similarly, IAB donor and IAB nodes can decide different QoS treatment on the wireless backhaul and access links based on the following 5G QoS characteristics and 5G QoS parameters:

- 5G QoS Identifier (5QI);

- Allocation and Retention Priority (ARP);

- Guaranteed Flow Bit Rate (GFBR) for both uplink and downlink;

- Maximum Flow Bit Rate (MFBR) for both uplink and downlink;

- Maximum Packet Loss Rate for both uplink and downlink;

- Resource Type (GBR, delay critical GBR or Non-GBR);

- Priority level;

- Packet Delay Budget;

- Packet Error Rate;

- Averaging window;

- Maximum Data Burst Volume.

Proposal 1: 5G QoS characteristics and 5G QoS parameters defined for 5G QoS model are considered as the baseline for QoS enforcement in IAB architectures.

However, we think there may be additional factor(s) for the QoS enforcement because the QoS flows are transmitted though zero or more IAB nodes. In Figure 1, UE 3’s QoS flow is directly sent to IAB donor whereas UE 2’s QoS flow is sent to the IAB donor via two IAB nodes. If the QoS flow of the UE 2 has the same QoS requirements as that of the UE 3, each QoS flow of the UE 2 and 3 may need to be differently treated on the wireless backhaul and/or access links.

Proposal 2: The number of IAB nodes between UE and IAB donor is considered for QoS enforcement in IAB architectures.

Also, lossless transmission should also be supported in the IAB architectures. However, the current PDCP specifications cannot ensure the lossless transmission because the PDCP Data PDU, whose successful delivery has been confirmed by lower layers, would not be retransmitted even though successful delivery of the PDCP Data PDU has been confirmed by lower layers which does NOT mean the receiving PDCP entity has successfully received the PDCP Data PDU.

TS 38.323

for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

- perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

- perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

- submit the resulting PDCP Data PDU to lower layer.

Therefore, new mechanism should be required for lossless transmission. We think there is two options:

- Option #1: Transmitting PDCP retransmits all PDCP SDUs regardless of whether the successful delivery of the corresponding PDCP Data PDUs has been confirmed by lower layers;

- Option #2: Transmitting PDCP retransmits all PDCP SDUs for which discardTimer is running.

Proposal 3: Discuss the following options for lossless transmission in the IAB architectures:

- Option #1: Transmitting PDCP retransmits all PDCP SDUs regardless of whether the successful delivery of the corresponding PDCP Data PDUs has been confirmed by lower layers;

- Option #2: Transmitting PDCP retransmits all PDCP SDUs for which discardTimer is running.

3.	Proposal

In this document, we present our view on QoS impacts. We have following proposals:

Proposal 1: 5G QoS characteristics and 5G QoS parameters defined for 5G QoS model are considered as the baseline for QoS enforcement in IAB architectures.

Proposal 2: The number of IAB nodes between UE and IAB donor is considered for QoS enforcement in IAB architectures.

Proposal 3: Discuss the following options for lossless transmission in the IAB architectures:

- Option #1: Transmitting PDCP retransmits all PDCP SDUs regardless of whether the successful delivery of the corresponding PDCP Data PDUs has been confirmed by lower layers;

- Option #2: Transmitting PDCP retransmits all PDCP SDUs for which discardTimer is running.